3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220695

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2546 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC Rel-16 test cases 8.1.6.1.3.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At step 12 of these test cases, in the UEInformationResponse under nrFailedPCellId-r16 the UE can set “Either pci-arfcn-r16 or cellGlobalId-r16 corresponding to NR Cell 1”, according e.g. 38.523-1 table 8.1.6.1.3.1.3.2-3. If the UE includes pci-arfcn-r16 then the current TTCN implementation will give a run-time error as it tries to check “v_ReceivedReport.nr_RLF_Report_r16.failedPCellId_r16.nrFailedPCellId_r16.cellGlobalId_r16. cellIdentity_r16”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Expand checking to confirm if pci-arfcn-r16 or cellGlobalId-r16 is selected before checking before checking respective cell identities |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.3.x.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_6_1_3_Common |
| --- | --- |
| Reason for change | At step 12 of these test cases, in the UEInformationResponse under nrFailedPCellId-r16 the UE can set “Either pci-arfcn-r16 or cellGlobalId-r16 corresponding to NR Cell 1”, according e.g. 38.523-1 table 8.1.6.1.3.1.3.2-3. If the UE includes pci-arfcn-r16 then the current TTCN implementation will give a run-time error as it tries to check “v_ReceivedReport.nr_RLF_Report_r16.failedPCellId_r16.nrFailedPCellId_r16.cellGlobalId_r16. cellIdentity_r16”. |
| Summary of change | Expand checking to confirm if pci-arfcn-r16 or cellGlobalId-r16 is selected before checking before checking respective cell identities |
| TTCN module | RRC_MDT_IntraNR_RLF_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_6_1_3_Common(NR_CellId_Type p_NeighbourCell, boolean p_CheckLocationInfo := false) runs on NR5GC_PTC {//@sic R5s211638, R5-217921 sic@ ………………………….. //@siclog "Step 12" siclog@ //Check: Does the UE transmit a UEInformationResponse message on NR Cell 2? v_ARFCN_Neighbour := f_NR_CellInfo_GetFrequencySSB(p_NeighbourCell); v_CellIdentity_Serving := f_NR_CellInfo_GetCellIdentity(nr_Cell1); v_CellIdentity_Neighbour := f_NR_CellInfo_GetCellIdentity(p_NeighbourCell); v_UEInformationResponse := cr_UEInformationResponse_r16_RLF(v_PhysCellIdNeighbour, v_ARFCN_Neighbour, v_CellIdentity_Neighbour, f_NR_CellInfo_GetRNTI(nr_Cell1)); // @sic R5s211638 sic@ if(p_CheckLocationInfo) { v_UEInformationResponse.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16.nr_RLF_Report_r16.locationInfo_r16 := ?; //@sic R5-217923 sic@ } SRB.receive(car_NR_SRB_RrcPdu_IND(p_NeighbourCell, tsc_NR_RbId_SRB1, v_UEInformationResponse)) -> value v_ReceivedMsg; // @sic R5s211638 sic@ v_ReceivedReport := v_ReceivedMsg.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16; if( v_ReceivedReport.nr_RLF_Report_r16.failedPCellId_r16.nrFailedPCellId_r16.cellGlobalId_r16.cellIdentity_r16 == v_CellIdentity_Serving or //@sic R5-217921 sic@ v_ReceivedReport.nr_RLF_Report_r16.failedPCellId_r16.nrFailedPCellId_r16.pci_arfcn_r16.physCellId_r16 == v_PhysCellIdServing) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Test " & testcasename() & " Step 12"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test " & testcasename() & " Step 12: wrong failed PCell ID "); } //@siclog "Step 13" siclog@ //Void @sic R5-216289 sic@ f_NR_TestBody_Set(false); // Finish f_NR_Postamble(p_NeighbourCell, STATE_CONNECTED_3A); // @sic R5-216289 sic@ } // end of f_TC_8_1_6_1_3_Common

After Change

function f_TC_8_1_6_1_3_Common(NR_CellId_Type p_NeighbourCell, boolean p_CheckLocationInfo := false) runs on NR5GC_PTC {//@sic R5s211638, R5-217921 sic@ ……………………… //@siclog "Step 12" siclog@ //Check: Does the UE transmit a UEInformationResponse message on NR Cell 2? v_ARFCN_Neighbour := f_NR_CellInfo_GetFrequencySSB(p_NeighbourCell); v_CellIdentity_Serving := f_NR_CellInfo_GetCellIdentity(nr_Cell1); v_CellIdentity_Neighbour := f_NR_CellInfo_GetCellIdentity(p_NeighbourCell); v_UEInformationResponse := cr_UEInformationResponse_r16_RLF(v_PhysCellIdNeighbour, v_ARFCN_Neighbour, v_CellIdentity_Neighbour, f_NR_CellInfo_GetRNTI(nr_Cell1)); // @sic R5s211638 sic@ if(p_CheckLocationInfo) { v_UEInformationResponse.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16.nr_RLF_Report_r16.locationInfo_r16 := ?; //@sic R5-217923 sic@ } SRB.receive(car_NR_SRB_RrcPdu_IND(p_NeighbourCell, tsc_NR_RbId_SRB1, v_UEInformationResponse)) -> value v_ReceivedMsg; // @sic R5s211638 sic@ v_ReceivedReport := v_ReceivedMsg.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16; if(ischosen(v_ReceivedReport.nr_RLF_Report_r16.failedPCellId_r16.nrFailedPCellId_r16.cellGlobalId_r16)) { if (v_ReceivedReport.nr_RLF_Report_r16.failedPCellId_r16.nrFailedPCellId_r16.cellGlobalId_r16.cellIdentity_r16 == v_CellIdentity_Serving) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Test " & testcasename() & " Step 12"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test " & testcasename() & " Step 12: wrong failed PCell ID for CGI"); } } else if(ischosen(v_ReceivedReport.nr_RLF_Report_r16.failedPCellId_r16.nrFailedPCellId_r16.pci_arfcn_r16)) { if (v_ReceivedReport.nr_RLF_Report_r16.failedPCellId_r16.nrFailedPCellId_r16.pci_arfcn_r16.physCellId_r16 == v_PhysCellIdServing) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Test " & testcasename() & " Step 12"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test " & testcasename() & " Step 12: wrong failed PCell ID for PCI"); } } //@siclog "Step 13" siclog@ //Void @sic R5-216289 sic@ f_NR_TestBody_Set(false); // Finish f_NR_Postamble(p_NeighbourCell, STATE_CONNECTED_3A); // @sic R5-216289 sic@ } // end of f_TC_8_1_6_1_3_Common