3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220730

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2564 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for ENDC testcase 8.2.2.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to R5w220109, in the RRC test cases where NR PDCP duplication is configured, it is not clear how SS NR PDCP entity is aware that the NR PDCP duplication is configured and whether (or not) SS NR PDCP entity shall route the data carried by duplicated PDCP packet(s) to the TTCN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the PDCP configuration entity in the SS to disable discarding duplicated PDUs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.2.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk12 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Corrections required

## 1.1 Change 1

| Function name | cs_NR_PDCP_RbConfig_ParamsSRB |
| --- | --- |
| Reason for change | Following R5w220109, an additional field is required to configure PDCP duplication discarding |
| Summary of change | Add a parameter to cs_NR_PDCP_RbConfig_ParamsSRB to allow configuration of PDCP duplication discarding in the SS |
| TTCN module | NR_PDCP_Templates |

Before change:

template (value) NR_PDCP_RbConfig_Type cs_NR_PDCP_RbConfig_ParamsSRB :=

{ /* @status    APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */

Params := {

Rb := {

Srb := true

},

DuplicateDiscarding := omit  //@sic R5w220109 sic@

}

};

After change:

template (value) NR_PDCP_RbConfig_Type cs_NR_PDCP_RbConfig_ParamsSRB (template (omit) boolean p_ConfigureDuplicateDiscarding := omit) :=

{ /* @status    APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */

Params := {

Rb := {

Srb := true

},

DuplicateDiscarding := p_ConfigureDuplicateDiscarding  //@sic R5w220109 sic@

}

};

## 2.2 Change 2

| Function name | cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA |
| --- | --- |
| Reason for change | As per Change 1, an additional parameter is required to configure duplicate discarding in the PDCP entity |
| Summary of change | Add a parameter to cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA to allow configuration of the duplicate discarding function of the receiving PDCP entity |
| TTCN module | RRCReconfig_ENDC_EUTRA |

Before change:

template (value) RadioBearer_Type cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(SRB_Identity_Type p_SRB_Id,

template (value) RlcBearerRouting_Type p_RlcBearerRouting,

template(omit) RLC_Configuration_Type p_RLC_Configuration := omit,

template(omit) MAC_Configuration_Type p_MAC_Configuration := omit,

template (omit) LogicalChannelId_Type p_LogicalChannelId := omit,

template (omit) boolean p_ConfigureDuplicateDiscarding := omit) :=

//@sic R5s210128 sic@

/* @status    APPROVED (ENDC) */

cs_RadioBearer_Reconfig(cs_RadioBearerId_SRB(p_SRB_Id),

cs_PDCP_Configuration_NR_Terminating(cs_NR_PDCP_RbConfig_ParamsSRB, p_RlcBearerRouting),

p_RLC_Configuration,

p_MAC_Configuration,

p_LogicalChannelId);

After change:

template (value) RadioBearer_Type cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(SRB_Identity_Type p_SRB_Id,

template (value) RlcBearerRouting_Type p_RlcBearerRouting,

template(omit) RLC_Configuration_Type p_RLC_Configuration := omit,

template(omit) MAC_Configuration_Type p_MAC_Configuration := omit,

template (omit) LogicalChannelId_Type p_LogicalChannelId := omit,

template (omit) boolean p_ConfigureDuplicateDiscarding := omit) :=

//@sic R5s210128 sic@

/* @status    APPROVED (ENDC) */

cs_RadioBearer_Reconfig(cs_RadioBearerId_SRB(p_SRB_Id),

cs_PDCP_Configuration_NR_Terminating(cs_NR_PDCP_RbConfig_ParamsSRB(p_ConfigureDuplicateDiscarding), p_RlcBearerRouting),

p_RLC_Configuration,

p_MAC_Configuration,

p_LogicalChannelId);

## 2.3 Change 3

| Function name | cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA |
| --- | --- |
| Reason for change | The configuration of the SS is not complete until duplicate discarding function of the receiving PDCP entity is disabled |
| Summary of change | Use the parameter added in Change 2 to disable duplicate discarding on the receiving PDCP entity |
| TTCN module | RRCReconfig_ENDC_EUTRA |

Before change:

function f_EUTRA38_ReconfigureSRBs_NR_PDCPSplit(EUTRA_CellId_Type   p_CellId,

boolean             p_Receive_RBConfig2 := false,

integer p_SK_Counter := 0,

boolean             p_DuplicatePDU := true

) runs on EUTRA_5GS_PTC

{// @sic R5s201348 R5s210128 R5s210647 sic@

var EUTRA38_SecurityParamsExtd_Type v_Security_Params := f_EUTRA38_Security_Get();

var template (value) octetstring v_NR_RBConfig1;

var template (omit) octetstring v_NR_RBConfig2 := omit;

var octetstring v_OctetData;

var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs;

var integer v_MCG_DRBId := f_EUTRA38_ENDC_GetDRBIdOfMCG();

var template (value) RadioBearerList_Type v_ConfigDrbSS := {cs_OneDRB_ConfigAM(v_MCG_DRBId)}; //DRB on MCG - unchanged through the TC

var template (value) RadioBearerList_Type v_SrbDrbList;

var template (omit) octetstring v_NR_SCGConfig := omit;

var template (value) SecurityActTimeList_Type v_SecurityActTimeList;

var DRB_IdentityList_AllAndAMonly_Type v_DRB_IdentityList:= f_EUTRA_DRB_IdentityList(v_ConfigDrbSS);

var SubFrameTiming_Type v_Timing;

if (p_SK_Counter != 0)

{

v_Security_Params.SK_Counter := p_SK_Counter;

}

v_OctetData := f_EUTRA_NR_WaitForCoOrd_Data(NR, "NR Ciphering Alg");

f_EUTRA_AS_CipheringAlgorithm_Set(f_EUTRA_ConvertCiphAlg2Bitstring(substr(oct2bit(v_OctetData), 5, 3)));

v_OctetData := f_EUTRA_NR_WaitForCoOrd_Data(NR, "NR Integrity Alg");

f_EUTRA_AS_IntegrityAlgorithm_Set(f_EUTRA_ConvertIntAlg2Bitstring(substr(oct2bit(v_OctetData), 5, 3)));

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_NR_RBConfig1 := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

if (p_Receive_RBConfig2) {

v_NR_RBConfig2 := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

}

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig, v_NR_RBConfig1, v_Security_Params.SK_Counter,fl_NR_GetP_Max(), v_NR_RBConfig2);

f_EUTRA_SS_PdcpCount_GetAmDrbs(p_CellId, v_DRB_IdentityList.AMonly, f_EUTRA_SS_PdcpCount_GetAll(p_CellId));

v_SecurityActTimeList := f_EUTRA_RRC_CipherActTime_GetForHO(p_CellId, v_DRB_IdentityList.AMonly, f_EUTRA_SS_PdcpCount_GetAll(p_CellId));

if (p_DuplicatePDU){

v_SrbDrbList := {cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(tsc_SRB1, cs_RlcBearerRouting_NR(nr_Cell1), cs_RLC_Configuration_TestModeAckProhibit(Prohibit, cs_RLC_RbConfig_SRB1), cs_MAC_Configuration(tsc_SRB1_LogicalChannelPriority, infinity_), 1),

cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(tsc_SRB2, cs_RlcBearerRouting_NR(nr_Cell1), cs_RLC_Configuration_AM(cs_TX_AM_RLC_SRB2_Def, cs_RX_AM_RLC_SRB2_Def), cs_MAC_Configuration(tsc_SRB2_LogicalChannelPriority, infinity_), 2)};

} else {

v_SrbDrbList := {cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(tsc_SRB1, cs_RlcBearerRouting_NR(nr_Cell1), cs_RLC_Configuration_AM(cs_TX_AM_RLC_SRB1_Def, cs_RX_AM_RLC_SRB1_Def), cs_MAC_Configuration(tsc_SRB1_LogicalChannelPriority, infinity_), 1),

cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(tsc_SRB2, crs_RlcBearerRouting_None, cs_RLC_Configuration_AM(cs_TX_AM_RLC_SRB2_Def, cs_RX_AM_RLC_SRB2_Def), cs_MAC_Configuration(tsc_SRB2_LogicalChannelPriority, infinity_), 2)};

}

v_Timing :=  f_EUTRA_GetNextSendOccasion(p_CellId, tsc_EUTRA_DelayBeforeIntraCellHO);

f_EUTRA_508RRC_IntraLTE_HO_IntraCell_Step1_5(p_CellId,

cs_TimingInfo_EUTRA_NB(v_Timing),

v_ConfigDrbSS,

-,  //default value tsc_C_RNTI_Def2

-, //MeasConfig,

-,

v_SecurityActTimeList,

-,

v_RRCConnectionReconfiguration_v890_IEs,

v_SrbDrbList

);

if (p_DuplicatePDU)

{

// Check: Does the UE send a RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete message on split SRB1 duplicated over the SCG path?

interleave {

[]SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, crs_RlcBearerRouting_Eutra(eutra_Cell1))) {}

[]SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, cr_RlcBearerRouting_NR(nr_Cell1))) {}

}

} else {

// Check: Does the UE send a RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete message on split SRB1 over the MCG path?

SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, crs_RlcBearerRouting_Eutra(eutra_Cell1)));

}

//After steps in table 8.2.2.2.1.3.2-2 are completed the SS is configured to send RLC ACK on split SRB1 and the last message received in step 2 is acknowledged.

if (p_DuplicatePDU)

{

f_EUTRA_SS_CommonRadioBearerConfig(eutra_Cell1, {cs_RB_Reconfig_TestModeAckProhibit( tsc_RbId_SRB1, Continue)});

}

//Complete the HO procedure steps 7

// Step 7: configure RACH procedure back to default

f_EUTRA_SS_ConfigRachProcedure_Def(eutra_Cell1);

// enable TA transmissions

f_EUTRA_SS_CommonCellConfig(eutra_Cell1, cas_PUCCH_Synch_Config_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_UplinkTimeAlignment_Synch_Auto_Def));

}

After change:

function f_EUTRA38_ReconfigureSRBs_NR_PDCPSplit(EUTRA_CellId_Type   p_CellId,

boolean             p_Receive_RBConfig2 := false,

integer p_SK_Counter := 0,

boolean             p_DuplicatePDU := true

) runs on EUTRA_5GS_PTC

{// @sic R5s201348 R5s210128 R5s210647 sic@

var EUTRA38_SecurityParamsExtd_Type v_Security_Params := f_EUTRA38_Security_Get();

var template (value) octetstring v_NR_RBConfig1;

var template (omit) octetstring v_NR_RBConfig2 := omit;

var octetstring v_OctetData;

var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs;

var integer v_MCG_DRBId := f_EUTRA38_ENDC_GetDRBIdOfMCG();

var template (value) RadioBearerList_Type v_ConfigDrbSS := {cs_OneDRB_ConfigAM(v_MCG_DRBId)}; //DRB on MCG - unchanged through the TC

var template (value) RadioBearerList_Type v_SrbDrbList;

var template (omit) octetstring v_NR_SCGConfig := omit;

var template (value) SecurityActTimeList_Type v_SecurityActTimeList;

var DRB_IdentityList_AllAndAMonly_Type v_DRB_IdentityList:= f_EUTRA_DRB_IdentityList(v_ConfigDrbSS);

var SubFrameTiming_Type v_Timing;

if (p_SK_Counter != 0)

{

v_Security_Params.SK_Counter := p_SK_Counter;

}

v_OctetData := f_EUTRA_NR_WaitForCoOrd_Data(NR, "NR Ciphering Alg");

f_EUTRA_AS_CipheringAlgorithm_Set(f_EUTRA_ConvertCiphAlg2Bitstring(substr(oct2bit(v_OctetData), 5, 3)));

v_OctetData := f_EUTRA_NR_WaitForCoOrd_Data(NR, "NR Integrity Alg");

f_EUTRA_AS_IntegrityAlgorithm_Set(f_EUTRA_ConvertIntAlg2Bitstring(substr(oct2bit(v_OctetData), 5, 3)));

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_NR_RBConfig1 := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

if (p_Receive_RBConfig2) {

v_NR_RBConfig2 := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

}

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig, v_NR_RBConfig1, v_Security_Params.SK_Counter,fl_NR_GetP_Max(), v_NR_RBConfig2);

f_EUTRA_SS_PdcpCount_GetAmDrbs(p_CellId, v_DRB_IdentityList.AMonly, f_EUTRA_SS_PdcpCount_GetAll(p_CellId));

v_SecurityActTimeList := f_EUTRA_RRC_CipherActTime_GetForHO(p_CellId, v_DRB_IdentityList.AMonly, f_EUTRA_SS_PdcpCount_GetAll(p_CellId));

if (p_DuplicatePDU){

v_SrbDrbList := {cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(tsc_SRB1, cs_RlcBearerRouting_NR(nr_Cell1), cs_RLC_Configuration_TestModeAckProhibit(Prohibit, cs_RLC_RbConfig_SRB1), cs_MAC_Configuration(tsc_SRB1_LogicalChannelPriority, infinity_), 1, false),

cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(tsc_SRB2, cs_RlcBearerRouting_NR(nr_Cell1), cs_RLC_Configuration_AM(cs_TX_AM_RLC_SRB2_Def, cs_RX_AM_RLC_SRB2_Def), cs_MAC_Configuration(tsc_SRB2_LogicalChannelPriority, infinity_), 2, false)};

} else {

v_SrbDrbList := {cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(tsc_SRB1, cs_RlcBearerRouting_NR(nr_Cell1), cs_RLC_Configuration_AM(cs_TX_AM_RLC_SRB1_Def, cs_RX_AM_RLC_SRB1_Def), cs_MAC_Configuration(tsc_SRB1_LogicalChannelPriority, infinity_), 1),

cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(tsc_SRB2, crs_RlcBearerRouting_None, cs_RLC_Configuration_AM(cs_TX_AM_RLC_SRB2_Def, cs_RX_AM_RLC_SRB2_Def), cs_MAC_Configuration(tsc_SRB2_LogicalChannelPriority, infinity_), 2)};

}

v_Timing :=  f_EUTRA_GetNextSendOccasion(p_CellId, tsc_EUTRA_DelayBeforeIntraCellHO);

f_EUTRA_508RRC_IntraLTE_HO_IntraCell_Step1_5(p_CellId,

cs_TimingInfo_EUTRA_NB(v_Timing),

v_ConfigDrbSS,

-,  //default value tsc_C_RNTI_Def2

-, //MeasConfig,

-,

v_SecurityActTimeList,

-,

v_RRCConnectionReconfiguration_v890_IEs,

v_SrbDrbList

);

if (p_DuplicatePDU)

{

// Check: Does the UE send a RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete message on split SRB1 duplicated over the SCG path?

interleave {

[]SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, crs_RlcBearerRouting_Eutra(eutra_Cell1))) {}

[]SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, cr_RlcBearerRouting_NR(nr_Cell1))) {}

}

} else {

// Check: Does the UE send a RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete message on split SRB1 over the MCG path?

SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, crs_RlcBearerRouting_Eutra(eutra_Cell1)));

}

//After steps in table 8.2.2.2.1.3.2-2 are completed the SS is configured to send RLC ACK on split SRB1 and the last message received in step 2 is acknowledged.

if (p_DuplicatePDU)

{

f_EUTRA_SS_CommonRadioBearerConfig(eutra_Cell1, {cs_RB_Reconfig_TestModeAckProhibit( tsc_RbId_SRB1, Continue)});

}

//Complete the HO procedure steps 7

// Step 7: configure RACH procedure back to default

f_EUTRA_SS_ConfigRachProcedure_Def(eutra_Cell1);

// enable TA transmissions

f_EUTRA_SS_CommonCellConfig(eutra_Cell1, cas_PUCCH_Synch_Config_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_UplinkTimeAlignment_Synch_Auto_Def));

}