3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220736

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2570 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC idle mode test case 6.4.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For a UE operating with IMS enabled, when switched on at step 6C the current TTCN implementation may not correctly handle the IMS registraion signalling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use function f_NR5GC_SwitchOnAndResetIMS() instead of f_UT_SwitchOnUE() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_6_4_2_1_NR5GC_TestBody() |
| --- | --- |
| Reason for change | For a UE operating with IMS enabled, when switched on at step 6C the current TTCN implementation may not correctly handle the IMS registraion signalling. |
| Summary of change | Use function f_NR5GC_SwitchOnAndResetIMS() instead of f_UT_SwitchOnUE() |
| TTCN module | Inactive_CellReSelection_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_6_4_2_1_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5s200377 R5-204415 R5-205600 R5-212084 R5s210581 sic@ //FR2 Power levels are FFS var integer v_T1_Cell1_RS_EPRE_FR1 := -115; var integer v_T2_Cell1_RS_EPRE_FR1 := tsc_NR_ServingCellSSS_EPRE_FR1; var integer v_T2_Cell2_RS_EPRE_FR1 := -78; // @sic R5-198879 sic@ var integer v_T0_Cell1_RS_EPRE_FR1 := tsc_NR_ServingCellSSS_EPRE_FR1; var integer v_T0_Cell2_RS_EPRE_FR1 := tsc_NR_NonSuitableOffCellSSS_EPRE; ………………….. // @siclog "Step 6C" siclog@ // The UE is switched on. f_UT_SwitchOnUE(UT, false); f_NR5GC_MobileInfo_Init();// @sic R5s210092 sic@ // @siclog "Step 7" siclog@ // Generic procedure specified in TS 38.508-1 Table 4.5.3.2-1 are performed, in which the RRCRelease message includes the suspendConfig with NR_RRC_INACTIVE condition thereby moving the UE to RRC_INACTIVE. f_NR5GC_RRC_IdleState1N_Def(nr_Cell1, -, noRrcConnectionRelease, Initial_NoSecurity); // @sic R5s210092 sic@ f_NR_RRC_InactiveDef(nr_Cell1, -, false); …………………. }

After Change:

function f_TC_6_4_2_1_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5s200377 R5-204415 R5-205600 R5-212084 R5s210581 sic@ //FR2 Power levels are FFS var integer v_T1_Cell1_RS_EPRE_FR1 := -115; var integer v_T2_Cell1_RS_EPRE_FR1 := tsc_NR_ServingCellSSS_EPRE_FR1; var integer v_T2_Cell2_RS_EPRE_FR1 := -78; // @sic R5-198879 sic@ var integer v_T0_Cell1_RS_EPRE_FR1 := tsc_NR_ServingCellSSS_EPRE_FR1; var integer v_T0_Cell2_RS_EPRE_FR1 := tsc_NR_NonSuitableOffCellSSS_EPRE; ………………….. // @siclog "Step 6C" siclog@ // The UE is switched on. f_NR5GC_SwitchOnAndResetIMS(); f_NR5GC_MobileInfo_Init();// @sic R5s210092 sic@ // @siclog "Step 7" siclog@ // Generic procedure specified in TS 38.508-1 Table 4.5.3.2-1 are performed, in which the RRCRelease message includes the suspendConfig with NR_RRC_INACTIVE condition thereby moving the UE to RRC_INACTIVE. f_NR5GC_RRC_IdleState1N_Def(nr_Cell1, -, noRrcConnectionRelease, Initial_NoSecurity); // @sic R5s210092 sic@ f_NR_RRC_InactiveDef(nr_Cell1, -, false); ……………………….. }