3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220784

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2600 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_ReconfigureSS_DRBs_NRDC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In function f_ReconfigureSS_DRBs_NRDC, for split bear cases we configure MCG bearer drb id 3(proxy) and SCG bearer(drb id 3 split) and depending on the keys being used we configure AS_SecurityConfig. currently for the case MCG_SCG_SecondaryKeys keys are provided on MCG (master) and SCG (Secondary). Its not clear, how TTCN informs the SS which keys shall be used for the split bearer It is proposed to only provide the keys to the SCG (Secondary) as the PDCP (non-proxy) entity is configured here (non-proxy) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Commented out ( to be removed) the code where the function f_NR_SS_RRC_EnableIntProt_CiphULandDL is called for MCG cell id Security configuration are provided via p_SCG_CellId |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Which security configuration to be used by SS will not be clear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.5.2.NR5GC, 8.2.2.9.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required f_ReconfigureSS_DRBs_NRDC	3

2.1	Correction to f_ReconfigureSS_DRBs_NRDC	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required f_ReconfigureSS_DRBs_NRDC

## Correction to f_ReconfigureSS_DRBs_NRDC

| Function name | f_ReconfigureSS_DRBs_NRDC |
| --- | --- |
| Reason for change | In function f_ReconfigureSS_DRBs_NRDC, for split bear cases we configure MCG bearer drb id 3(proxy) and SCG bearer(drb id 3 split) and depending on the keys being used we configure AS_SecurityConfig. currently for the case MCG_SCG_SecondaryKeys keys are provided on MCG (master) and SCG (Secondary). Its not clear, how TTCN informs the SS which keys shall be used for the split bearer It is proposed to only provide the keys to the SCG (Secondary) as the PDCP (non-proxy) entity is configured here (non-proxy) |
| Summary of change | Commented out ( to be removed) the code where the function f_NR_SS_RRC_EnableIntProt_CiphULandDL is called for MCG cell id Security configuration are providrd via p_SCG_CellId |
| TTCN module | NR5GC_RRCSteps_NRDC |
| MCC160 Comment |  |

Before Change:

function f_ReconfigureSS_DRBs_NRDC( NR_CellId_Type p_MCG_CellId, NR_CellId_Type p_SCG_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) NR_RadioBearerList_Type p_SS_ConfigMCG, template (omit) NR_RadioBearerList_Type p_SS_ConfigSCG, NRDC_SecurityActivation_Type p_NRDC_SecurityActivation, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value4, boolean p_ActivateSecurity := true ) runs on NR_BASE_PTC { //@sic R5s220185 : added p_ActivateSecurity sic@ var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); //Reconfigure DRB in SS - existing DRBs are not reconfigured during release if (isvalue(p_SS_ConfigMCG)) { f_NR_SS_CommonRadioBearerConfig(p_MCG_CellId, p_SS_ConfigMCG, p_TimingInfo); } //Reconfigure SS SCG if (isvalue(p_SS_ConfigSCG)) { f_NR_SS_CommonRadioBearerConfig(p_SCG_CellId, p_SS_ConfigSCG, p_TimingInfo); f_NR_CellInfo_SetRNTI(p_SCG_CellId, p_RNTI_Value); f_NR_SS_C_RNTI_Config(p_SCG_CellId, p_RNTI_Value); f_NR_ULGrantConfiguration_Start(p_SCG_CellId); } //Configure PDCP security on MCG and SCG if (p_ActivateSecurity) { select (p_NRDC_SecurityActivation) { case (MCG) { v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_MCG_CellId, false); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_MCG_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); } case (MCG_SCG_MasterKeys) { v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_MCG_CellId, false); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_MCG_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_SCG_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); } case (MCG_SCG_SecondaryKeys) { v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_MCG_CellId, false); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_MCG_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); v_SecurityParams.AS_Ciphering_S.ActTimeList := f_NR_RRC_CipherActTime_SCG(p_SCG_CellId, p_SS_ConfigSCG); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_SCG_CellId, v_SecurityParams.AS_Integrity_S, v_SecurityParams.AS_Ciphering_S); } case(SCG) { v_SecurityParams.AS_Ciphering_S.ActTimeList := f_NR_RRC_CipherActTime_SCG(p_SCG_CellId, p_SS_ConfigSCG); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_SCG_CellId, v_SecurityParams.AS_Integrity_S, v_SecurityParams.AS_Ciphering_S); } } } }

After Change:

function f_ReconfigureSS_DRBs_NRDC( NR_CellId_Type p_MCG_CellId, NR_CellId_Type p_SCG_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) NR_RadioBearerList_Type p_SS_ConfigMCG, template (omit) NR_RadioBearerList_Type p_SS_ConfigSCG, NRDC_SecurityActivation_Type p_NRDC_SecurityActivation, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value4, boolean p_ActivateSecurity := true ) runs on NR_BASE_PTC { //@sic R5s220185 : added p_ActivateSecurity sic@ var NR_SecurityParams_Type v_SecurityParams := f_NR_Security_Get(); //Reconfigure DRB in SS - existing DRBs are not reconfigured during release if (isvalue(p_SS_ConfigMCG)) { f_NR_SS_CommonRadioBearerConfig(p_MCG_CellId, p_SS_ConfigMCG, p_TimingInfo); } //Reconfigure SS SCG if (isvalue(p_SS_ConfigSCG)) { f_NR_SS_CommonRadioBearerConfig(p_SCG_CellId, p_SS_ConfigSCG, p_TimingInfo); f_NR_CellInfo_SetRNTI(p_SCG_CellId, p_RNTI_Value); f_NR_SS_C_RNTI_Config(p_SCG_CellId, p_RNTI_Value); f_NR_ULGrantConfiguration_Start(p_SCG_CellId); } //Configure PDCP security on MCG and SCG if (p_ActivateSecurity) { select (p_NRDC_SecurityActivation) { case (MCG) { v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_MCG_CellId, false); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_MCG_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); } case (MCG_SCG_MasterKeys) { v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_MCG_CellId, false); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_MCG_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_SCG_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); } case (MCG_SCG_SecondaryKeys) { /*v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_MCG_CellId, false); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_MCG_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering);*/ //commented out MCG ciphering activation v_SecurityParams.AS_Ciphering_S.ActTimeList := f_NR_RRC_CipherActTime_SCG(p_SCG_CellId, p_SS_ConfigSCG); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_SCG_CellId, v_SecurityParams.AS_Integrity_S, v_SecurityParams.AS_Ciphering_S); } case(SCG) { v_SecurityParams.AS_Ciphering_S.ActTimeList := f_NR_RRC_CipherActTime_SCG(p_SCG_CellId, p_SS_ConfigSCG); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_SCG_CellId, v_SecurityParams.AS_Integrity_S, v_SecurityParams.AS_Ciphering_S); } } } }