3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220812

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2622 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MDT testcases 8.1.6.1.2.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The UE shall omit the nonCriticalExtension (RRCSetupComplete_v1610_IEs) when none of the IEs are set, therefore it is incorrect to expect nonCriticalExtension to be present with all its IEs set as omit |  |  |  |  |  |  |  |  |
| Summary of change: |  | When calling f_NR_RRC_ConnEst_DefWithNas allowed any or omit for nonCriticalExtension |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.4.NR5GC, 8.1.6.1.2.5.NR5GC, 8.1.6.1.2.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required for MDT test cases 8.1.6.1.2.x	2

2.1	Correction to f_TC_8_1_6_1_2_4_TestBody	2

2.2	Correction to f_TC_8_1_6_1_2_5_TestBody	4

2.3	Correction to f_TC_8_1_6_1_2_6_TestBody	5

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required for MDT test cases 8.1.6.1.2.x

## Correction to f_TC_8_1_6_1_2_4_TestBody

| Function name | f_TC_8_1_6_1_2_4_TestBody |
| --- | --- |
| Reason for change | The UE shall omit the nonCriticalExtension (RRCSetupComplete_v1610_IEs) when none of the IEs are set, therefore it is incorrect to expect nonCriticalExtension to be present with all its IEs set as omit |
| Summary of change | When calling f_NR_RRC_ConnEst_DefWithNas allowed any or omit for nonCriticalExtension |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_6_1_2_4_TestBody() runs on NR5GC_PTC { // @sic R5-216300, R5-220331, R5-222118, R5-223496 sic@ //@siclog "Steps 5" siclog@ //The UE transmits an RRCSetupRequest message on NR Cell 11 to initiate a tracking area update procedure //@siclog "Steps 6" siclog@ //SS transmits an RRCSetup message //@siclog "Steps 7" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell11,tsc_NR_RRC_TI_Def,?,-,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(omit, omit, omit))); // @sic R5s220072 sic@ if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); … … … //@siclog "Steps 5" siclog@ //The UE transmits an RRCSetupRequest message on NR Cell 11 to initiate a tracking area update procedure //@siclog "Steps 6" siclog@ //SS transmits an RRCSetup message //@siclog "Steps 7" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell11,tsc_NR_RRC_TI_Def,?,-,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(omit, omit, omit))); // @sic R5s220072 sic@ if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }

After Change:

function f_TC_8_1_6_1_2_4_TestBody() runs on NR5GC_PTC { // @sic R5-216300, R5-220331, R5-222118, R5-223496 sic@ … … … //@siclog "Steps 5" siclog@ //The UE transmits an RRCSetupRequest message on NR Cell 11 to initiate a tracking area update procedure //@siclog "Steps 6" siclog@ //SS transmits an RRCSetup message //@siclog "Steps 7" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell11,tsc_NR_RRC_TI_Def,?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }

## Correction to f_TC_8_1_6_1_2_5_TestBody

| Function name | f_TC_8_1_6_1_2_5_TestBody |
| --- | --- |
| Reason for change | The UE shall omit the nonCriticalExtension (RRCSetupComplete_v1610_IEs) when none of the IEs are set, therefore it is incorrect to expect nonCriticalExtension to be present with all its IEs set as omit |
| Summary of change | When calling f_NR_RRC_ConnEst_DefWithNas allowed any or omit for nonCriticalExtension |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_6_1_2_5_TestBody() runs on NR5GC_PTC { … … … //@siclog "Steps 3" siclog@ //Wait 30s to allow UE to activate logging f_Delay(30.0); //@siclog "Steps 4-6" siclog@ //Steps 1 to 3 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure //@siclog "Steps 7" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? f_NR_Paging_Def(nr_Cell1, RRC_IDLE); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell1,tsc_NR_RRC_TI_Def,?,-,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(omit, omit, omit))); // @sic R5s220073, R5-223496 sic@

After Change:

function f_TC_8_1_6_1_2_5_TestBody() runs on NR5GC_PTC { … … … //@siclog "Steps 3" siclog@ //Wait 30s to allow UE to activate logging f_Delay(30.0); //@siclog "Steps 4-6" siclog@ //Steps 1 to 3 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure //@siclog "Steps 7" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? f_NR_Paging_Def(nr_Cell1, RRC_IDLE); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell1,tsc_NR_RRC_TI_Def,?); // @sic R5s220073, R5-223496 sic@

## Correction to f_TC_8_1_6_1_2_6_TestBody

| Function name | f_TC_8_1_6_1_2_6_TestBody |
| --- | --- |
| Reason for change | The UE shall omit the nonCriticalExtension (RRCSetupComplete_v1610_IEs) when none of the IEs are set, therefore it is incorrect to expect nonCriticalExtension to be present with all its IEs set as omit |
| Summary of change | When calling f_NR_RRC_ConnEst_DefWithNas allowed any or omit for nonCriticalExtension |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_6_1_2_6_TestBody() runs on NR5GC_PTC { … … … //@siclog "Steps 3" siclog@ //Wait 30s to allow UE to activate logging f_Delay(30.0); //@siclog "Steps 4-6" siclog@ //Steps 1 to 3 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed //@siclog "Steps 7" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? f_NR_Paging_Def(nr_Cell1, RRC_IDLE); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell1,tsc_NR_RRC_TI_Def,?,-,-,-,-,cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(omit, omit, omit))); // @sic R5s220094, R5-223496 sic@

After Change:

function f_TC_8_1_6_1_2_6_TestBody() runs on NR5GC_PTC { … … … //@siclog "Steps 3" siclog@ //Wait 30s to allow UE to activate logging f_Delay(30.0); //@siclog "Steps 4-6" siclog@ //Steps 1 to 3 of the generic procedure in TS 38.508-1 Table 4.5.4.2-3 are executed //@siclog "Steps 7" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? f_NR_Paging_Def(nr_Cell1, RRC_IDLE); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell1,tsc_NR_RRC_TI_Def,?); // @sic R5s220094, R5-223496 sic@