3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220814

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2624 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC test case 7.1.1.3.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The two variables v_RRC_MsgRcvd and v_PHR_Rcvd need to be re-initialised back to FALSE so that Step 10 (Power Headroom MAC Control Element) and Step 11 (RRCReconfigurationComplete message) can be handled correctly in the alt statement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Variables v_RRC_MsgRcvd and v_PHR_Rcvd are re-initialised back to FALSE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Step 10 and Step 11 will not be properly handled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.7.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required for test case 7.1.1.3.7	2

2.1	Correction to f_TC_7_1_1_3_7_ENDC	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required for test case 7.1.1.3.7

## Correction to f_TC_7_1_1_3_7_ENDC

| Function name | f_TC_7_1_1_3_7_ENDC |
| --- | --- |
| Reason for change | The two variables v_RRC_MsgRcvd and v_PHR_Rcvd need to be re-initialised back to FALSE so that Step 10 (Power Headroom MAC Control Element) and Step 11 (RRCReconfigurationComplete message) can be handled correctly in the alt statement. |
| Summary of change | Variables v_RRC_MsgRcvd and v_PHR_Rcvd are re-initialised back to FALSE |
| TTCN module | MAC_ENDC_NR |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_3_7_ENDC_NR() runs on ENDC_NR_PTC { //UE power headroom reporting / Periodic reporting / DL pathloss change reporting <<SKIPPED CODE>> //@siclog "Steps 9& 11" siclog@ v_MAC_CellGroupConfig.phr_Config := cs_38508_PHR_Config_Setup(cs_PHR_Config_7137(-,-,dB3)); f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG, omit, omit, v_MAC_CellGroupConfig); // PHR Prohibit Timer v_TimerMax := f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, 1.0); v_TimerMin := f_NR_SetTimerToleranceMin (nr_Cell1, l2Timer, 1.0); t_Watchdog.start; f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig,-,-,false); // Now send octetstrings to EUTRA //@siclog "Steps 10" siclog@ alt { [not v_RRC_MsgRcvd] EUTRA.receive(cmr_EUTRA_NR_OctetData( "Complete")) -> value v_ReceivedMsg { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); v_OctetData := v_ReceivedMsg.OctetData.Data; v_EncodedComplete := oct2bit(v_OctetData); v_Result := decvalue(v_EncodedComplete, v_Complete); if (v_Result != 0 or not match (v_Complete, cr_38508_RRCReconfigurationCompleteIE)) { FatalError(__FILE__, __LINE__, "Decoding error Step 11"); v_RRC_MsgRcvd := true; if(not v_PHR_Rcvd){ repeat;} } } [not v_PHR_Rcvd]SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(nr_Cell1, cr_NR_MAC_CE_MultiEntryPHR(?)))->value v_NR_SYSTEM_IND { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); v_TimingInfo1 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; v_PHR_Rcvd := true; if(not v_RRC_MsgRcvd){ repeat;} } [] t_Watchdog.timeout //@sic R5s220484 sic@ { if(not v_PHR_Rcvd) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3: TP1 failed, UE didn't send PHR after phr-PeriodicTimer is configured in UE "); } } }; <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_3_7_ENDC_NR() runs on ENDC_NR_PTC { //UE power headroom reporting / Periodic reporting / DL pathloss change reporting <<SKIPPED CODE>> //@siclog "Steps 9& 11" siclog@ v_MAC_CellGroupConfig.phr_Config := cs_38508_PHR_Config_Setup(cs_PHR_Config_7137(-,-,dB3)); f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG, omit, omit, v_MAC_CellGroupConfig); // PHR Prohibit Timer v_TimerMax := f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, 1.0); v_TimerMin := f_NR_SetTimerToleranceMin (nr_Cell1, l2Timer, 1.0); //WA# v_RRC_MsgRcvd := false; v_PHR_Rcvd := false; t_Watchdog.start; f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig,-,-,false); // Now send octetstrings to EUTRA //@siclog "Steps 10" siclog@ alt { [not v_RRC_MsgRcvd] EUTRA.receive(cmr_EUTRA_NR_OctetData( "Complete")) -> value v_ReceivedMsg { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); v_OctetData := v_ReceivedMsg.OctetData.Data; v_EncodedComplete := oct2bit(v_OctetData); v_Result := decvalue(v_EncodedComplete, v_Complete); if (v_Result != 0 or not match (v_Complete, cr_38508_RRCReconfigurationCompleteIE)) { FatalError(__FILE__, __LINE__, "Decoding error Step 11"); v_RRC_MsgRcvd := true; if(not v_PHR_Rcvd){ repeat;} } } [not v_PHR_Rcvd]SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(nr_Cell1, cr_NR_MAC_CE_MultiEntryPHR(?)))->value v_NR_SYSTEM_IND { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); v_TimingInfo1 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; v_PHR_Rcvd := true; if(not v_RRC_MsgRcvd){ repeat;} } [] t_Watchdog.timeout //@sic R5s220484 sic@ { if(not v_PHR_Rcvd) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3: TP1 failed, UE didn't send PHR after phr-PeriodicTimer is configured in UE "); } } }; <<SKIPPED CODE>>