3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220817

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2627 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC Test case 11.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The f_IMS_MOCallSetup_NR5GC_A_4_X_Step1 function already hadles reception of the INVITE so when calling also f_IMS_MOCallSetup_ReceiveINVITE_Common it leads to the test waiting for another INVITE to be received. - Since this test is not an early contention test it only needs to start UL grant instead of calling the entire f_NR_RRCSetup_Def function in a preparation for the new conneciton. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - fl_IMS_NR5GC_MOSpeechCall_Step1 is no more calling the f_IMS_MOCallSetup_ReceiveINVITE_Common twice. - Replaced f_NR_RRCSetup_Def with f_NR_ULGrantConfiguration_Start before the step 18 starts. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.1.NR5GC, 11.3.1.NR5GC, 11.4.1.NR5GC, 11.4.7.NR5GC, 11.4.8.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

2.2	Change 2	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_IMS_NR5GC_MOSpeechCall_Step1 |
| --- | --- |
| Reason for change | The f_IMS_MOCallSetup_NR5GC_A_4_X_Step1 function already hadles reception of the INVITE so when calling also f_IMS_MOCallSetup_ReceiveINVITE_Common it leads to the test waiting for another INVITE to be received. |
| Summary of change | fl_IMS_NR5GC_MOSpeechCall_Step1 is no more calling the f_IMS_MOCallSetup_ReceiveINVITE_Common twice. |
| TTCN module | IMS_NR5GC_Functions.ttcn |
| MCC160 Comment |  |

Before Change

function fl_IMS_NR5GC_MOSpeechCall_Step1(template SipUrl p_OtherCalleeUri := omit) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { if (pc_Preconditions) { // @sic R5-222077: Additional checks for A.4.2 (no 'precondition' option tag in Supported header field, no precondition attributes) sic@ f_IMS_MOCallSetup_NR5GC_A_4_1_Step1(p_OtherCalleeUri); } else { f_IMS_MOCallSetup_NR5GC_A_4_2_Step1(p_OtherCalleeUri); } return f_IMS_MOCallSetup_ReceiveINVITE_Common(p_OtherCalleeUri, -, tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, pc_Preconditions); // @sic R5s10603 sic@ }

After Change

function fl_IMS_NR5GC_MOSpeechCall_Step1(template SipUrl p_OtherCalleeUri := omit) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { if (pc_Preconditions) { // @sic R5-222077: Additional checks for A.4.2 (no 'precondition' option tag in Supported header field, no precondition attributes) sic@ return f_IMS_MOCallSetup_ReceiveINVITE_Common(p_OtherCalleeUri, -, tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, true); } else { return f_IMS_MOCallSetup_NR5GC_A_4_2_Step1(p_OtherCalleeUri); } }

## Change 2

| Function name | fl_TC_11_3_1_TestBody |
| --- | --- |
| Reason for change | Since this test is not an early contention test it only needs to start UL grant instead of calling the entire f_NR_RRCSetup_Def function. |
| Summary of change | Replaced f_NR_RRCSetup_Def with f_NR_ULGrantConfiguration_Start before the step 18 starts. |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_11_3_1_TestBody() runs on NR5GC_PTC { … //@siclog "Steps 18-28" siclog@ //Step 3 to step 13 from test procedure for IMS MO speech call establishment as described in TS 38.508-1 [4] Table 4.9.15.2.2-1 take place, // v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(nr_Cell1, '0001'B, mo_VoiceCall); //v_GSM_MobilityInfo := f_NR5GC_SpeechCall_Steps2_8(nr_Cell1, mo_VoiceCall, '0001'B); // @sic R5s210037 sic@ // send RRC Connection Setup on SRB0 (CCCH) and switch on UL grant assignments: f_NR_RRCSetup_Def (nr_Cell1); // UE shall establish SRB1 acc. to 38.331 cl. 5.3.3.1 // UE enters RRC_CONNECTED state // receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected ); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)), '0001'B)) { // @sic R5s200654 subject to prose CR approval sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); v_GSM_MobilityInfo:= f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); // done on IMS PTC // Wait until need to add voice QoS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_NR5GC_ModifyPDUSession_AddVoice(nr_Cell1, valueof(v_GSM_MobilityInfo)); if (not pc_Preconditions) { // @sic R5s220579 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Voice Call Established"); // Call now established f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //@siclog "Steps 29-31" siclog@

After Change

function fl_TC_11_3_1_TestBody() runs on NR5GC_PTC { … //@siclog "Steps 18-28" siclog@ //Step 3 to step 13 from test procedure for IMS MO speech call establishment as described in TS 38.508-1 [4] Table 4.9.15.2.2-1 take place, // v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(nr_Cell1, '0001'B, mo_VoiceCall); //v_GSM_MobilityInfo := f_NR5GC_SpeechCall_Steps2_8(nr_Cell1, mo_VoiceCall, '0001'B); // @sic R5s210037 sic@ // send RRC Connection Setup on SRB0 (CCCH) and switch on UL grant assignments: f_NR_ULGrantConfiguration_Start(nr_Cell1); // UE shall establish SRB1 acc. to 38.331 cl. 5.3.3.1 // UE enters RRC_CONNECTED state // receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected ); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)), '0001'B)) { // @sic R5s200654 subject to prose CR approval sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); v_GSM_MobilityInfo:= f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); // done on IMS PTC // Wait until need to add voice QoS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_NR5GC_ModifyPDUSession_AddVoice(nr_Cell1, valueof(v_GSM_MobilityInfo)); if (not pc_Preconditions) { // @sic R5s220579 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Voice Call Established"); // Call now established f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //@siclog "Steps 29-31" siclog@