3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220844

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2647 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 6.5.2.6 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR5GC test case 6.5.2.6 to the IWD_22wk23 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 6.5.2.6 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	4

2.1	f_TC_6_5_2_6_NR5GC	4

2.2	cs_CAGInfoList_oneEntry_SamePLMN_twoIDs	7

2.3	cs_CAGInfo_SamePLMN_twoIDs	8

2.4	cs_CAGInfoList_oneEntry_SamePLMN_twoIDs	9

2.5	fl_TC_6_5_2_6_TestBody	10

2.6	cs_CAGInfoList_oneEntry	16

3	Branches executed	17

4	Execution Log Files	17

4.1	MediaTek MT6983 UE	17

5	References	17

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 6.5.2.6 which is part of the NR5GC test suite in iWD_TTCN3-B2020-09_D22wk23.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	6.5.2.6.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D22wk23

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	MediaTek MT6983

Verification Status:	PASS

# Corrections required

## f_TC_6_5_2_6_NR5GC

| Function name | f_TC_6_5_2_6_NR5GC |
| --- | --- |
| Reason for change | In the current TTCN, both NR Cell 3 and NR Cell 6 belong to PLMN2 and also have same TAC. Because of this, the UE will not perporm Registration for mobility at Step 24. This needs to be addressed. For preamble, the function fl_NR5GC_RRC_IdleSteps4_20_withCAGInfoList(nr_Cell1, v_ReceivedMsg, Initial_NoSecurity, -, cs_CAGInfoList_twoEntries(v_CAGInfo_ID1,v_CAGInfo_ID2)); is called. This does not include the PLMN2 in EPLMN. This will result in UEs waiting for maximum 300s before camping on NR Cell 3 or NR Cell 6 during the testcase. In order to minimize execution time it is proposed to add PLMN2 in EPLMN during Registration Accept in preamble. Also, the current TTCN includes 2 entries for same PLMN each for a CAG ID. This is not required. According to 24.501 cl 9.11.3.18A, just one entry for PLMN is needed. This also needs to be corrected. A Prose CR will be raised for this. |
| Summary of change | Assigned TAC=2 for NR Cell 6. Included the PLMN2 in EPLMN list for Registration Accept, and also created new templates cs_CAGInfoList_oneEntry_SamePLMN_twoIDs and cs_CAGInfo_SamePLMN_twoIDs for using same PLMN for two CAG IDs. Please see below. |
| TTCN module | CAG_Selection_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_6_5_2_6_NR5GC() runs on NR5GC_PTC { //CAG / cell reservation var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (value) CAG_Info v_CAGInfo_ID1; var template (value) CAG_Info v_CAGInfo_ID2; f_NR5GC_Init(NR_4); f_NR_CellInfo_InitMaxReferencePower(nr_Cell6, -78, -78); //FR2 FFS f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell3);// prose CR is needed f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell6, 2);// prose CR is needed f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(nr_Cell6, true_); f_NR_CellInfo_SetPLMN(nr_Cell3, tsc_NR_HPLMN_002_11);//PLMN2 f_NR_CellInfo_SetPLMN(nr_Cell6, tsc_NR_HPLMN_002_11);//PLMN2 //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); //CAG Cell f_NR_CellConfig_Def(nr_Cell6); //CAG Cell //There is no CAG information list provisioned in the UE. //The UE is in Automatic PLMN selection mode. f_UT_AutomaticPLMN_Select (UT); //preamble v_ReceivedMsg := f_NR5GC_Preamble_Steps1_4 (nr_Cell1); //The UE is registered on NR Cell 1 using the procedure described in TS 38.508-1 [4] clause 4.5.2.2 except that the REGISTRATION ACCEPT message includes CAG information list with CAG-ID 1 and CAG-ID 2. v_CAGInfo_ID1 :=cs_CAGInfo_oneID(f_NR_Asn2Nas_PlmnId (f_NR_CellInfo_GetPLMN (nr_Cell3)),'0'B, int2oct(1,4)); v_CAGInfo_ID2 :=cs_CAGInfo_oneID(f_NR_Asn2Nas_PlmnId (f_NR_CellInfo_GetPLMN (nr_Cell6)),'0'B, int2oct(2,4)); fl_NR5GC_RRC_IdleSteps4_20_withCAGInfoList(nr_Cell1, v_ReceivedMsg, Initial_NoSecurity, -, cs_CAGInfoList_twoEntries(v_CAGInfo_ID1,v_CAGInfo_ID2)); f_NR_UE_DeRegisterOnSwitchOff (nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_6_5_2_6_TestBody(); f_NR_TestBody_Set(false); //And now remove the cell(s) f_NR_ReleaseAllCells(); }

After Change

function f_TC_6_5_2_6_NR5GC() runs on NR5GC_PTC { //CAG / cell reservation var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (value) CAG_Info v_CAGInfo_ID1; var template (value) CAG_Info v_CAGInfo_ID2; f_NR5GC_Init(NR_4); f_NR_CellInfo_InitMaxReferencePower(nr_Cell6, -78, -78); //FR2 FFS f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell3);// prose CR is needed f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell6, 2);// prose CR is needed f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(nr_Cell6, true_); f_NR_CellInfo_SetPLMN(nr_Cell3, tsc_NR_HPLMN_002_11);//PLMN2 f_NR_CellInfo_SetPLMN(nr_Cell6, tsc_NR_HPLMN_002_11);//PLMN2 f_NR_CellInfo_SetTAC(nr_Cell6, int2bit(2,24)); //WA#6_5_2_6 //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); //CAG Cell f_NR_CellConfig_Def(nr_Cell6); //CAG Cell //There is no CAG information list provisioned in the UE. //The UE is in Automatic PLMN selection mode. f_UT_AutomaticPLMN_Select (UT); //preamble v_ReceivedMsg := f_NR5GC_Preamble_Steps1_4 (nr_Cell1); //The UE is registered on NR Cell 1 using the procedure described in TS 38.508-1 [4] clause 4.5.2.2 except that the REGISTRATION ACCEPT message includes CAG information list with CAG-ID 1 and CAG-ID 2. v_CAGInfo_ID1 :=cs_CAGInfo_oneID(f_NR_Asn2Nas_PlmnId (f_NR_CellInfo_GetPLMN (nr_Cell3)),'0'B, int2oct(1,4)); v_CAGInfo_ID2 :=cs_CAGInfo_oneID(f_NR_Asn2Nas_PlmnId (f_NR_CellInfo_GetPLMN (nr_Cell6)),'0'B, int2oct(2,4)); fl_NR5GC_RRC_IdleSteps4_20_withCAGInfoList ( nr_Cell1, v_ReceivedMsg, Initial_NoSecurity, - , cs_CAGInfoList_oneEntry_SamePLMN_twoIDs (cs_CAGInfo_SamePLMN_twoIDs ( f_NR_Asn2Nas_PlmnId (f_NR_CellInfo_GetPLMN (nr_Cell3)),'0'B, int2oct(1,4), int2oct(2,4))), cs_PLMN_List_1PLMN (f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell3)))); f_NR_UE_DeRegisterOnSwitchOff (nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_6_5_2_6_TestBody(); f_NR_TestBody_Set(false); //And now remove the cell(s) //f_NR_ReleaseAllCells(); f_NR_Postamble(nr_Cell3, STATE_CONNECTED_3A, tsc_NR_RbId_SRB1); //WA#6_5_2_6 }

## cs_CAGInfoList_oneEntry_SamePLMN_twoIDs

| Function name | cs_CAGInfoList_oneEntry_SamePLMN_twoIDs |
| --- | --- |
| Reason for change | New template |
| Summary of change | Please see change 2.1 above |
| TTCN module | CAG_Selection_NR5GC.ttcn |
| MCC160 Comment |  |

template (value) CAGInfoList cs_CAGInfoList_oneEntry_SamePLMN_twoIDs(template(value) CAG_Info p_CAGInfo ) := { /* @status */ /* 24.501 cl. 9.11.3.18A */ //WA#6_5_2_4 iei := '75'O, iel := '000D'O, listOfCAGInfos := {p_CAGInfo} };

## cs_CAGInfo_SamePLMN_twoIDs

| Function name | cs_CAGInfo_SamePLMN_twoIDs |
| --- | --- |
| Reason for change | New template |
| Summary of change | Please see change 2.1 above |
| TTCN module | CAG_Selection_NR5GC.ttcn |
| MCC160 Comment |  |

template (value) CAG_Info cs_CAGInfo_SamePLMN_twoIDs (O3_Type p_Plmn, B1_Type p_CagOnlyFlag, O4_Type p_CAGId1, O4_Type p_CAGId2) := { /* @status*/ /* 24.501 cl. 9.11.3.18AA */ //WA#6_5_2_4 iel := '0C'O, plmn := p_Plmn, spareBits := tsc_Spare7, cagOnly := p_CagOnlyFlag, listOfCAGIds := {p_CAGId1,p_CAGId2} };

## cs_CAGInfoList_oneEntry_SamePLMN_twoIDs

| Function name | cs_CAGInfoList_oneEntry_SamePLMN_twoIDs |
| --- | --- |
| Reason for change | New template |
| Summary of change | Please see change 2.1 above |
| TTCN module | CAG_Selection_NR5GC.ttcn |
| MCC160 Comment |  |

template (value) CAGInfoList cs_CAGInfoList_oneEntry_SamePLMN_twoIDs(template(value) CAG_Info p_CAGInfo ) := { /* @status */ /* 24.501 cl. 9.11.3.18A */ //WA#6_5_2_4 iei := '75'O, iel := '000D'O, listOfCAGInfos := {p_CAGInfo} };

## fl_TC_6_5_2_6_TestBody

| Function name | fl_TC_6_5_2_6_TestBody |
| --- | --- |
| Reason for change | In current TTCN, at Step 1 after the call to function f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(nr_Cell1, true_);, the function f_NR_ModifySysinfo(nr_Cell1); needs to be called to broadcast the updated SIB1. This is missing. In the current TTCN for Steps 5-22a1 the function f_NR5GC_RRC_IdleState1N_Def(nr_Cell3, -, -, Initial_Secure); is called. This does not include the CAG ID of NR Cell 6. As a result, the UE may not look for NR Cell 6 once the UE camps on NR Cell 3. Hence, it is proposed to include CAG ID of NR Cell 6 in the Registration Accept message during Steps 5-22a1. A Prose CR will be raised for this. In the current TTCN at Step 24 the function f_NR_RRC_SetupRequest_Def is called to capture the rrcSetupREQ. However, this results in the preconfigured rrcSetup being sent immediately, and then the TTCN again sends the rrcSetup by callng function f_NR_RRCSetup_Def. This results in testcase failure, and needs to be corrected. Also, after the call to function f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(nr_Cell3, true_);, the function f_NR_ModifySysinfo(nr_Cell3); needs to be called to broadcast the updated SIB1. This is missing. The current TTCN calls function f_NR5GC_RegistrationReject. It is instead proposed to follow the successful Mobility Registration on NR Cell 3, as that will ensure that the UE has successfully moved to NR Cell 3 and it also will reduce signaling. A Prose CR will be raised for this. As a result of performing the Registration for Mobility (last point above), the postamble also needs to change to f_NR_Postamble(nr_Cell3, STATE_CONNECTED_3A, tsc_NR_RbId_SRB1); |
| Summary of change | Called function _NR_ModifySysinfo(nr_Cell1); after calling f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(nr_Cell1, true_);. Instead of calling f_NR5GC_RRC_IdleState1N_Def(nr_Cell3, -, -, Initial_Secure);, called following : v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell3, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); fl_NR5GC_RRC_IdleSteps4_20_withCAGInfoList(nr_Cell3, v_ReceivedMsg, Initial_Secure, -, cs_CAGInfoList_oneEntry(cs_CAGInfo_oneID(f_NR_Asn2Nas_PlmnId (f_NR_CellInfo_GetPLMN (nr_Cell6)),'0'B, int2oct(2,4)))); Re-wrote the TTCN for Steps 24-30, please see below. Called function _NR_ModifySysinfo(nr_Cell3); after calling f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(nr_Cell3, true_);. Instead of calling f_NR5GC_RegistrationReject(nr_Cell3, tsc_Cause_NR_NotAuthorizedCSG, Mobility);, called f_NR5GC_MobilityRegistration(nr_Cell3, -, -, -, -, noRrcConnectionRelease); Updated the Postamble (Please see change 2.1) |
| TTCN module | CAG_Selection_NR5GC.ttcn |
| MCC160 Comment |  |

Before:

function fl_TC_6_5_2_6_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; //@siclog "Step 1" siclog@ //The SIB1 for NR cell 1 is updated according to Table 6.5.2.6.3.3-2 f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(nr_Cell1, true_); //@siclog "Step 2" siclog@ //Power on the UE f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 3" siclog@ //Check: Does the UE transmit a RRCSetupRequest message on NR Cell 1 within 60s? f_NR_CheckNoRRCSetupReq(nr_Cell1, 60.0,-, "Step3: UE sent an RRCSetupRequest on NR Cell 1 within 60 s"); //@siclog "Step 4" siclog@ //The SS changes cell power levels according to row "T1" in Table 6.5.2.6.3.2-1 for FR1 and Table 6.5.2.6.3.2-2 for FR2. f_NR_SetCellPower (nr_Cell3,tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR1);//FR2 is FFS //@siclog "Steps 5-22a1" siclog@ //Steps 2-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 3. f_NR5GC_RRC_IdleState1N_Def(nr_Cell3, -, -, Initial_Secure); f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 5-22a1 pass"); //@siclog "Step 23" siclog@ //SS adjusts cell levels according to row T2 of table 6.5.2.6.3.2-1/2. f_NR_SetCellPower (nr_Cell1,tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_NR_SetCellPower (nr_Cell6,-78, -78);//FR2 is FFS //@siclog "Step 24" siclog@ //Check: Does the UE transmit a RRCSetupRequest message on NR Cell 6 f_NR_RRC_SetupRequest_Def(nr_Cell6); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 24 pass"); //@siclog "Steps 25-29" siclog@ //Steps 2-6a1 of Table 4.9.5.2.2-1 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 6. f_NR_RRCSetup_Def(nr_Cell6); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell6, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell6, v_GMM_MobilityInfo, -, -, noRrcConnectionRelease); //@siclog "Step 30" siclog@ //The SIB1 for NR cell 3 is updated according to Table 6.5.2.6.3.3-5 f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(nr_Cell3, true_); //@siclog "Step 31" siclog@ //SS adjusts cell levels according to row T3 of table 6.5.2.6.3.2-1/2. f_NR_SetCellPower (nr_Cell3,tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR1);//FR2 is FFS f_NR_SetCellPower (nr_Cell6,tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Steps 32-40" siclog@ //Steps 1-9 of Table 4.9.8.2.2-1 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 3. f_NR5GC_RegistrationReject(nr_Cell3, tsc_Cause_NR_NotAuthorizedCSG, Mobility); }//End of f_TC_6_5_2_6_TestBody

After:

function fl_TC_6_5_2_6_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; //@siclog "Step 1" siclog@ //The SIB1 for NR cell 1 is updated according to Table 6.5.2.6.3.3-2 f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(nr_Cell1, true_); f_NR_ModifySysinfo(nr_Cell1); //WA#6_5_2_6 //@siclog "Step 2" siclog@ //Power on the UE f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 3" siclog@ //Check: Does the UE transmit a RRCSetupRequest message on NR Cell 1 within 60s? f_NR_CheckNoRRCSetupReq(nr_Cell1, 60.0,-, "Step3: UE sent an RRCSetupRequest on NR Cell 1 within 60 s"); //@siclog "Step 4" siclog@ //The SS changes cell power levels according to row "T1" in Table 6.5.2.6.3.2-1 for FR1 and Table 6.5.2.6.3.2-2 for FR2. f_NR_SetCellPower (nr_Cell3,tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR1);//FR2 is FFS //@siclog "Steps 5-22a1" siclog@ //Steps 2-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 3. //WA#6_5_2_6 COMMENTED OUT f_NR5GC_RRC_IdleState1N_Def(nr_Cell3, -, -, Initial_Secure); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell3, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); //WA#6_5_2_6 fl_NR5GC_RRC_IdleSteps4_20_withCAGInfoList(nr_Cell3, v_ReceivedMsg, Initial_Secure, -, cs_CAGInfoList_oneEntry(cs_CAGInfo_oneID(f_NR_Asn2Nas_PlmnId (f_NR_CellInfo_GetPLMN (nr_Cell6)),'0'B, int2oct(2,4)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 5-22a1 pass"); //@siclog "Step 23" siclog@ //SS adjusts cell levels according to row T2 of table 6.5.2.6.3.2-1/2. f_NR_SetCellPower (nr_Cell1,tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_NR_SetCellPower (nr_Cell6,-78, -78);//FR2 is FFS //@siclog "Step 24" siclog@ //Check: Does the UE transmit a RRCSetupRequest message on NR Cell 6 //WA#6_5_2_6 COMMENTED OUT f_NR_RRC_SetupRequest_Def(nr_Cell6); //WA#6_5_2_6 COMMENTED OUT f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 24 pass"); //@siclog "Steps 25-29" siclog@ //Steps 2-6a1 of Table 4.9.5.2.2-1 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 6. //WA#6_5_2_6 COMMENTED OUT f_NR_RRCSetup_Def(nr_Cell6); /*WA#6_5_2_6 COMMENTED OUT v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell6, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def));*/ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell6, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); //WA#6_5_2_6 f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 24 pass"); //WA#6_5_2_6 if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell6, v_GMM_MobilityInfo); //WA#6_5_2_6 //WA#6_5_2_6 COMMENTED OUT f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell6, v_GMM_MobilityInfo, -, -, -, -, -, cs_PLMN_List_1PLMN (f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell3)))); //@siclog "Step 30" siclog@ //The SIB1 for NR cell 3 is updated according to Table 6.5.2.6.3.3-5 f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(nr_Cell3, true_); f_NR_ModifySysinfo(nr_Cell3); //WA#6_5_2_6 //@siclog "Step 31" siclog@ //SS adjusts cell levels according to row T3 of table 6.5.2.6.3.2-1/2. f_NR_SetCellPower (nr_Cell3,tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR1);//FR2 is FFS f_NR_SetCellPower (nr_Cell6,tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Steps 32-40" siclog@ //Steps 1-9 of Table 4.9.8.2.2-1 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 3. //WA#6_5_2_6 COMMENTED OUT f_NR5GC_RegistrationReject(nr_Cell3, tsc_Cause_NR_NotAuthorizedCSG, Mobility); f_NR5GC_MobilityRegistration(nr_Cell3, -, -, -, -, noRrcConnectionRelease); //WA#6_5_2_6 }//End of f_TC_6_5_2_6_TestBody

## cs_CAGInfoList_oneEntry

| Function name | cs_CAGInfoList_oneEntry |
| --- | --- |
| Reason for change | New template |
| Summary of change | Please see change 2.1 above |
| TTCN module | CAG_Selection_NR5GC.ttcn |
| MCC160 Comment |  |

Before:

template (value) CAGInfoList cs_CAGInfoList_oneEntry(template(value) CAG_Info p_CAGInfo ) := { /* @status */ /* 24.501 cl. 9.11.3.18A */ iei := '75'O, iel := '0011'O, listOfCAGInfos := {p_CAGInfo} };

After:

template (value) CAGInfoList cs_CAGInfoList_oneEntry(template(value) CAG_Info p_CAGInfo ) := { /* @status */ /* 24.501 cl. 9.11.3.18A */ iei := '75'O, iel := '0009'O, //WA#6_5_2_6 listOfCAGInfos := {p_CAGInfo} };

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band n78 using NEA1/NIA1  algorithms.

# Execution Log Files

## MediaTek MT6983 UE

The MediaTek MT6983 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_6_5_2_6_MTK.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s220845: Supporting information for addition of NR5GC TC 6.5.2.6 in FR1 |
| --- | --- |