3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220859

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2655 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_NR5GC_EmergencyCallLimitedService and fl_TC_11_4_4_NR5GC_TestBody |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Similar to R5s220821, in the current TTCN implementation of testcase 11.4.4, at steps 8A-9D the function f_NR5GC_EmergencyCallPDUSession is called which establishes a PDU Session for Emergency. After this the NR PTC sends a coordination message to IMS PTC so that procedure for IMS Emergency Voice call (A.6) is performed. Once this is done, the IMS PTC sends a coordination message to the NR PTC to add voice. After this, the IMS PTC waits for a coordination message from the NR PTC to release the call f_IMS_EmergencyCallAndReleaseWithoutRegistration f_IMS_IPCAN_WaitForTrigger(IPCAN); Once the NR PTC receives the coordination message from IMS PTC, the function f_NR5GC_ModifyPDUSession_AddVoice is called to add voice (f_NR5GC_EmergencyCallPDUSession f_NR5GC_ModifyPDUSession_AddVoice), and then, a coordination message is expected from IMS PTC: f_NR5GC_EmergencyCallPDUSession f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); At this stage, both, the NR PTC and IMS PTC are waiting for a coordination message and the testcase gets stuck. - Similar to 1), when f_NR5GC_EmergencyCallLimitedService invokes f_NR5GC_EmergencyCallPDUSession within it, the TC gets stuck as well waiting for coordination message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | After the NR PTC has added voice by calling function f_NR5GC_ModifyPDUSession_AddVoice, it should send a coordination message to IMS PTC, rather than wait for a coordination message |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the emergency test cases unfairly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.4.NR5GC, 11.4.2.NR5GC, 11.4.3.NR5GC, 11.4.9.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function fl_TC_11_4_4_NR5GC_TestBody() |
| --- | --- |
| Reason for change | Current sequence of functions leaves both, the NR PTC and IMS PTC waiting for a coordination message and the testcase gets stuck. |
| Summary of change | Instead of calling f_NR5GC_EmergencyCallPDUSession f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); Called: f_NR5GC_EmergencyCallPDUSession f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_11_4_4_NR5GC_TestBody() runs on NR5GC_PTC { <<SKIPPED CODE>> // @siclog "Step 8A - 9D" siclog@ //Check: Does the UE transmit an UL NAS TRANSPORT message with 'Request type' set to 'initial emergency request', and, a PDU SESSION ESTABLISHMENT REQUEST for establishing an emergency PDU? // Steps 9-13 from Generic Test Procedure for IMS Emergency call establishment with IMS Emergency registration as specified in TS 38.508-1 [4], subclause 4.9.11, including the parallel behaviour specified in table 4.9.11.2.2-2 are performed. //f_NR5GC_EmergencyCallWithRegistration(nr_Cell11); f_NR5GC_EmergencyCallPDUSession(nr_Cell11, tsc_NR_RbId_SRB1); //@sic R5s210515 sic@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); <<SKIPPED CODE>> //@siclog "Steps 16G-16L" siclog@ //Check: Does the UE perform steps 8-13 of Generic Test Procedure for IMS Emergency call establishment with IMS Emergency registration as specified in TS 38.508-1 [4], subclause 4.9.11? // Steps 8-13 f_NR5GC_EmergencyCallPDUSession(nr_Cell11, tsc_NR_RbId_SRB1); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Established"); f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 16G-16L "); //@siclog "Step 17 - 18" siclog@ //Make the UE release the emergency call. (NOTE 1) //The Generic test procedure for IMS MO Emergency call release as specified in TS 38.508-1 [4], subclause 4.9.12A takes place f_NR5GC_EmergencyCallRelease(nr_Cell11); f_NR_RRCRelease(nr_Cell11); }

After Change:

function fl_TC_11_4_4_NR5GC_TestBody() runs on NR5GC_PTC { <<SKIPPED CODE>> // @siclog "Step 8A - 9D" siclog@ //Check: Does the UE transmit an UL NAS TRANSPORT message with 'Request type' set to 'initial emergency request', and, a PDU SESSION ESTABLISHMENT REQUEST for establishing an emergency PDU? // Steps 9-13 from Generic Test Procedure for IMS Emergency call establishment with IMS Emergency registration as specified in TS 38.508-1 [4], subclause 4.9.11, including the parallel behaviour specified in table 4.9.11.2.2-2 are performed. //f_NR5GC_EmergencyCallWithRegistration(nr_Cell11); f_NR5GC_EmergencyCallPDUSession(nr_Cell11, tsc_NR_RbId_SRB1); //@sic R5s210515 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); <<SKIPPED CODE>> //@siclog "Steps 16G-16L" siclog@ //Check: Does the UE perform steps 8-13 of Generic Test Procedure for IMS Emergency call establishment with IMS Emergency registration as specified in TS 38.508-1 [4], subclause 4.9.11? // Steps 8-13 f_NR5GC_EmergencyCallPDUSession(nr_Cell11, tsc_NR_RbId_SRB1); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Established"); f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 16G-16L "); //@siclog "Step 17 - 18" siclog@ //Make the UE release the emergency call. (NOTE 1) //The Generic test procedure for IMS MO Emergency call release as specified in TS 38.508-1 [4], subclause 4.9.12A takes place f_NR5GC_EmergencyCallRelease(nr_Cell11); f_NR_RRCRelease(nr_Cell11); }

## Change 2

| Function name | f_NR5GC_EmergencyCallLimitedService() |
| --- | --- |
| Reason for change | Current sequence of functions leaves both, the NR PTC and IMS PTC waiting for a coordination message and the testcase gets stuck. |
| Summary of change | Instead of calling f_NR5GC_EmergencyCallPDUSession f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); Called: f_NR5GC_EmergencyCallPDUSession f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_EmergencyCallLimitedService(NR_CellId_Type p_NR_CellId, boolean p_IncludeTAIList := true) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams; var NR_SecurityParams_Type v_RRC_SecurityParams; var template NAS_SecurityReturnType v_NAS_SecurityReturnValue; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (p_NR_CellId); var template (omit) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); if (not p_IncludeTAIList) { // @sic R5s211510 sic@ v_TAIList := omit; } // Steps 1-3 RRC connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, emergency, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Emergency)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Registration Request"); } <<SKIPPED CODE>> // Steps 13-18 f_NR5GC_EmergencyCallPDUSession(p_NR_CellId, tsc_NR_RbId_SRB1); // @sic R5s210518 sic@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); // Emergency call now active f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Established"); }

After Change:

function f_NR5GC_EmergencyCallLimitedService(NR_CellId_Type p_NR_CellId, boolean p_IncludeTAIList := true) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams; var NR_SecurityParams_Type v_RRC_SecurityParams; var template NAS_SecurityReturnType v_NAS_SecurityReturnValue; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (p_NR_CellId); var template (omit) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); if (not p_IncludeTAIList) { // @sic R5s211510 sic@ v_TAIList := omit; } // Steps 1-3 RRC connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, emergency, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Emergency)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Registration Request"); } <<SKIPPED CODE>> // Steps 13-18 f_NR5GC_EmergencyCallPDUSession(p_NR_CellId, tsc_NR_RbId_SRB1); // @sic R5s210518 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); // Emergency call now active f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Established"); }