3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220905

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2555 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 6.2.3.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-05-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS38.523-1 Table 6.2.3.8.3.3-2,carrierFreqListNR-r15 in SystemInformationBlockType24 for EUTRA cell has just one entry, but in current TTCN implementation ,there are 3 entries.According to 3GPP TS36.133 cl4.2, this may lead to test case error because UE need more time to search for all the frequency and the 300s guard timer for this test case may not enough. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set sib24 according to TS38.523-1 Table 6.2.3.8.3.3-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.8.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1 Corrections required	2

1.1 Correction to f_TC_6_2_3_8_NR5GC_EUTRA	2

# Corrections required

## Correction to f_TC_6_2_3_8_NR5GC_EUTRA

| Function name | f_TC_6_2_3_8_NR5GC_EUTRA |
| --- | --- |
| Reason for change | According to TS38.523-1 Table 6.2.3.8.3.3-2,carrierFreqListNR-r15 in SystemInformationBlockType24 for EUTRA cell has just one entry, but in current TTCN implementation ,there are 3 entries.According to 3GPP TS36.133 cl4.2, this may lead to test case error because UE need more time to search for all the frequency and the 300s guard timer for this test case may not enough. |
| Summary of change | Set sib24 according to TS38.523-1 Table 6.2.3.8.3.3-2 |
| TTCN module | Idle_CellReSelection_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_6_2_3_8_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { // Inter-RAT cell reselection / From E-UTRA_Idle to NR RRC_IDLE according to RAT priority provided by dedicated signalling (RRConnRelease) var ReselectionThreshold v_ThreshServingLow_Cell1 := 3; var ReselectionThreshold v_S_NonIntraSearch_Cell1 := 3; f_EUTRA38_IRAT_Init(c31); // System information combination 31 as defined in TS 36.508-1 clause 4.4.3.1 is used in E-UTRA Cell f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); // sends Freq. Info to the other RAT // Table 6.2.3.8.3.3-1: SystemInformationBlockType3 of EUTRA Cell 1 f_EUTRA_CellInfo_SetSysInfo_ThreshServingLow (eutra_Cell1, v_ThreshServingLow_Cell1); f_EUTRA_CellInfo_SetSysInfo_S_NonIntraSearch(eutra_Cell1, v_S_NonIntraSearch_Cell1); // Table 6.2.3.8.3.3-2: SystemInformationBlockType24 of EUTRA Cell 1 f_EUTRA_CellInfo_SetSIB24_threshX_Q_r15(eutra_Cell1, 10, 20); f_EUTRA_CellInfo_SetSIB24_t_ReselectionNR_r15(eutra_Cell1, 7); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); //Preamble: finishes with EUTRA cell on (NR cell off) and co-ord msg sent to say UE has completed registration on EUTRA f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Preamble"); // Wait for UE to register on NR first f_EUTRA38_NR5GC_Preamble (eutra_Cell1, TESTMode_OFF, RRC_IDLE); // Go on to EUTRA RRC_Idle while NR is already off f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Preamble")); // inform NR side about registration on both sides f_EUTRA_TestBody_Set(true); fl_TC_6_2_3_8_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); // take down the cell now, UE finished postamble on NR side f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Postamble"); f_EUTRA_ReleaseAllCells(); }

After Change:

function f_TC_6_2_3_8_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { // Inter-RAT cell reselection / From E-UTRA_Idle to NR RRC_IDLE according to RAT priority provided by dedicated signalling (RRConnRelease) var template (value) EUTRA_CellInfo_Type v_CellInfo; var template (value) SystemInformationBlockType24_r15 v_SIB24; var template (omit) CarrierFreqListNR_r15 v_CarrierFreqListNR_r15 := omit; var ReselectionThreshold v_ThreshServingLow_Cell1 := 3; var ReselectionThreshold v_S_NonIntraSearch_Cell1 := 3; f_EUTRA38_IRAT_Init(c31); // System information combination 31 as defined in TS 36.508-1 clause 4.4.3.1 is used in E-UTRA Cell f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); // sends Freq. Info to the other RAT // Table 6.2.3.8.3.3-1: SystemInformationBlockType3 of EUTRA Cell 1 f_EUTRA_CellInfo_SetSysInfo_ThreshServingLow (eutra_Cell1, v_ThreshServingLow_Cell1); f_EUTRA_CellInfo_SetSysInfo_S_NonIntraSearch(eutra_Cell1, v_S_NonIntraSearch_Cell1); // Table 6.2.3.8.3.3-2: SystemInformationBlockType24 of EUTRA Cell 1 v_CellInfo := f_EUTRA_CellInfo_Get(eutra_Cell1); v_CarrierFreqListNR_r15[0] := v_CellInfo.Sysinfo.BCCH_Info.SIs[2].message_.c1.systemInformation.criticalExtensions.systemInformation_r8.sib_TypeAndInfo[0].sib24_v1530.carrierFreqListNR_r15[0]; v_SIB24 := v_CellInfo.Sysinfo.BCCH_Info.SIs[2].message_.c1.systemInformation.criticalExtensions.systemInformation_r8.sib_TypeAndInfo[0].sib24_v1530; v_SIB24.carrierFreqListNR_r15 := v_CarrierFreqListNR_r15 f_EUTRA_CellInfo_SetSIB24(eutra_Cell1,v_SIB24); f_EUTRA_CellInfo_SetSIB24_threshX_Q_r15(eutra_Cell1, 10, 20); f_EUTRA_CellInfo_SetSIB24_t_ReselectionNR_r15(eutra_Cell1, 7); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); //Preamble: finishes with EUTRA cell on (NR cell off) and co-ord msg sent to say UE has completed registration on EUTRA f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Preamble"); // Wait for UE to register on NR first f_EUTRA38_NR5GC_Preamble (eutra_Cell1, TESTMode_OFF, RRC_IDLE); // Go on to EUTRA RRC_Idle while NR is already off f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Preamble")); // inform NR side about registration on both sides f_EUTRA_TestBody_Set(true); fl_TC_6_2_3_8_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); // take down the cell now, UE finished postamble on NR side f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Postamble"); f_EUTRA_ReleaseAllCells(); }