3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220938

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2676 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC SDAP test case 7.1.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In this test case, UE is configured in test loop mode B with IP PDU delay set to 1 second and DL data PDUs are sent at steps 1, 3 and 7. As per TS 36.509 cl.5.4.4.3, IP PDU delay will be deactivated after the UEs PDCP SDU buffer becomes enpty, and so the configured IP PDU delay of 1 second will not be applied to the DL data PDUs sent at steps 3 and 7. TS 36.509, cl.5.4.4.3 NOTE 2: After the PDCP SDU buffer becomes empty the loopback will return any received PDCP SDU in uplink directly as specified in clause 5.4.4.2. In order to reactivate the loopback delay and PDCP SDU buffering the SS shall deactivate UE test loop B function first. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Before steps 3 and 7, Added the functions to open and then close the test loop so IP PDU delay is reactivated. Note: An associated prose CR on 38.523-1 will be submitted at RAN5#96-e |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_7_1_4_1_NR_TestBody |
| --- | --- |
| Reason for change | In this test case, UE is configured in test loop mode B with IP PDU delay set to 1 second and DL data PDUs are sent at steps 1, 3 and 7. As per TS 36.509 cl.5.4.4.3, IP PDU delay will be deactivated after the UEs PDCP SDU buffer becomes enpty, and so the configured IP PDU delay of 1 second will not be applied to the DL data PDUs sent at steps 3 and 7. TS 36.509, cl.5.4.4.3 NOTE 2: After the PDCP SDU buffer becomes empty the loopback will return any received PDCP SDU in uplink directly as specified in clause 5.4.4.2. In order to reactivate the loopback delay and PDCP SDU buffering the SS shall deactivate UE test loop B function first. |
| Summary of change | Before steps 3 and 7, Added the functions to open and then close the test loop so IP PDU delay is reactivated. Note: An associated prose CR on 36.523-1 will be submitted at RAN5#96-e |
| TTCN module | SDAP_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_7_1_4_1_NR_TestBody(integer p_FirstDRB, integer p_SecondDRB, GSM_MobilityInfo_Type p_GSM_MobilityInfo_Type) runs on NR_BASE_PTC { /**** Skiping code ***********/ // @siclog "Step 2" siclog@ // Check: Does the UE re-transmit SDAP Data PDU on DRB 2 with SDAP header as per the stored DRB mapping flow with QFI=1? v_ExpectedPdu := cr_SDAP_PDU_UL_WithHeader(cr_SDAP_UL_PduHeader_Data(tsc_QFI2),v_IPDataQFI2); //@sic R5-215714, R5s211487 sic@ DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_SecondDRB, -, {v_ExpectedPdu})); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @siclog "Step 3" siclog@ // The SS sends the SDAP Data PDU with SDAP header on DRB 2 and the following content to the UE: RDI=1, RQI=0, QFI=5. f_SDAP_SendWithHeader(nr_Cell1, p_SecondDRB, tsc_RDI1, tsc_RQI0, tsc_QFI5, v_IPDataQFI5); /**** Skiping code ***********/ // EXCEPTION: In parallel to the event described in step 6 the events specified in Tables 7.1.4.1.3.2-3 and 7.1.4.1.3.2-4 shall take place. // @siclog "Table 7.1.4.1.3.3-3, Step 1" siclog@ // @siclog "Table 7.1.4.1.3.3-4, Step 1" siclog@ // @siclog "Step 6" siclog@ // @sic R5s191119 sic@ v_EndMarker := false; // Reset variable for reusing do { alt { [not v_RrcMsg] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)) { // The UE transmits an RRCReconfigurationComplete message. v_RrcMsg := true; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } [not v_EndMarker] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_FirstDRB, -, {cr_SDAP_PDU_UL_EndMarker(tsc_QFI4)})) { // Check: Does the UE transmit End-Marker Control PDU on DRB 1 for QFI=4? v_EndMarker := true; f_NR_PreliminaryPass(__FILE__, __LINE__, "End-Marker received"); } [not v_NasMsg] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { // The UE Transmits PDU SESSION MODIFICATION COMPLETE if (match (v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg , cr_NG_PDU_SESSION_MODIFICATION_COMPLETE(?,?))) { v_NasMsg := true; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Unexpected NAS message received"); } } } } while (not(v_RrcMsg and v_EndMarker and v_NasMsg)); // @siclog "Step 7" siclog@ // The SS sends the SDAP Data PDU with SDAP header on DRB 2 and the following content to the UE: RDI=0. RQI=0, QFI=4. f_SDAP_SendWithHeader(nr_Cell1, p_SecondDRB, tsc_RDI0, tsc_RQI0, tsc_QFI4, v_IPDataQFI4); // @siclog "Step 8" siclog@ // Check: Does the UE re-transmit SDAP Data PDU on DRB 2 with SDAP header as per the stored DRB mapping Flow with QFI=4? v_ExpectedPdu := cr_SDAP_PDU_UL_WithHeader(cr_SDAP_UL_PduHeader_Data(tsc_QFI4),v_IPDataQFI4); DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_SecondDRB, -, {v_ExpectedPdu})); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); }

After Change:

function fl_TC_7_1_4_1_NR_TestBody(integer p_FirstDRB, integer p_SecondDRB, GSM_MobilityInfo_Type p_GSM_MobilityInfo_Type) runs on NR_BASE_PTC { /**** Skiping code ***********/ // @siclog "Step 2" siclog@ // Check: Does the UE re-transmit SDAP Data PDU on DRB 2 with SDAP header as per the stored DRB mapping flow with QFI=1? v_ExpectedPdu := cr_SDAP_PDU_UL_WithHeader(cr_SDAP_UL_PduHeader_Data(tsc_QFI2),v_IPDataQFI2); //@sic R5-215714, R5s211487 sic@ DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_SecondDRB, -, {v_ExpectedPdu})); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); f_NR5GC_OpenUE_TestLoopMode(nr_Cell1); f_NR5GC_CloseUE_TestLoopModeB (nr_Cell1, '01'O); // @siclog "Step 3" siclog@ // The SS sends the SDAP Data PDU with SDAP header on DRB 2 and the following content to the UE: RDI=1, RQI=0, QFI=5. f_SDAP_SendWithHeader(nr_Cell1, p_SecondDRB, tsc_RDI1, tsc_RQI0, tsc_QFI5, v_IPDataQFI5); /**** Skiping code ***********/ // EXCEPTION: In parallel to the event described in step 6 the events specified in Tables 7.1.4.1.3.2-3 and 7.1.4.1.3.2-4 shall take place. // @siclog "Table 7.1.4.1.3.3-3, Step 1" siclog@ // @siclog "Table 7.1.4.1.3.3-4, Step 1" siclog@ // @siclog "Step 6" siclog@ // @sic R5s191119 sic@ v_EndMarker := false; // Reset variable for reusing do { alt { [not v_RrcMsg] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)) { // The UE transmits an RRCReconfigurationComplete message. v_RrcMsg := true; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } [not v_EndMarker] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_FirstDRB, -, {cr_SDAP_PDU_UL_EndMarker(tsc_QFI4)})) { // Check: Does the UE transmit End-Marker Control PDU on DRB 1 for QFI=4? v_EndMarker := true; f_NR_PreliminaryPass(__FILE__, __LINE__, "End-Marker received"); } [not v_NasMsg] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { // The UE Transmits PDU SESSION MODIFICATION COMPLETE if (match (v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg , cr_NG_PDU_SESSION_MODIFICATION_COMPLETE(?,?))) { v_NasMsg := true; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Unexpected NAS message received"); } } } } while (not(v_RrcMsg and v_EndMarker and v_NasMsg)); f_NR5GC_OpenUE_TestLoopMode(nr_Cell1); f_NR5GC_CloseUE_TestLoopModeB (nr_Cell1, '01'O); // @siclog "Step 7" siclog@ // The SS sends the SDAP Data PDU with SDAP header on DRB 2 and the following content to the UE: RDI=0. RQI=0, QFI=4. f_SDAP_SendWithHeader(nr_Cell1, p_SecondDRB, tsc_RDI0, tsc_RQI0, tsc_QFI4, v_IPDataQFI4); // @siclog "Step 8" siclog@ // Check: Does the UE re-transmit SDAP Data PDU on DRB 2 with SDAP header as per the stored DRB mapping Flow with QFI=4? v_ExpectedPdu := cr_SDAP_PDU_UL_WithHeader(cr_SDAP_UL_PduHeader_Data(tsc_QFI4),v_IPDataQFI4); DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_SecondDRB, -, {v_ExpectedPdu})); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); }