3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220989

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2705 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC function f_Check_NG_PDUSessionReleaseReq |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If UE send PDU Session Modification Request instead of PDU Session Release Request, TC will fail with run-time error because when v_Result is set to false, function continue and not return v_Result immediately. Later it cause a run-time error during if (match (v_GSMMsg.pdu_Session_Release_Request.pduSessionId.sessionId, p_PDUSessionGlobalInfo[i].SessionId)) { |  |  |  |  |  |  |  |  |
| Summary of change: |  | When UE does not send PDU Session Release(but for example PDU Session Modification) function will set v_Result to false and return immediately to end function |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will end with run-time error |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Correction to f_Check_NG_PDUSessionReleaseReq	3

# Overview

This document lists all the changes needed to correct issues in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Correction to f_Check_NG_PDUSessionReleaseReq

| Function name | f_Check_NG_PDUSessionReleaseReq |
| --- | --- |
| Reason for change | If UE send PDU Session Modification Request instead of PDU Session Release Request, TC will fail with run-time error because when v_Result is set to false, function continue and not return v_Result immediately. Later it cause a run-time error during if (match (v_GSMMsg.pdu_Session_Release_Request.pduSessionId.sessionId, p_PDUSessionGlobalInfo[i].SessionId)) { |
| Summary of change | When UE does not send PDU Session Release(but for example PDU Session Modification) function will set v_Result to false and return immediately to end function. |
| TTCN module | NG_NASTemplateFunctions.ttcn |

Before Change:

function f_Check_NG_PDUSessionReleaseReq( out GSM_MobilityInfo_Type p_GSM_MobilityInfo,

PDUSessionInfoList_Type p_PDUSessionGlobalInfo,

NG_NAS_UL_Pdu_Type p_ReceivedMsgs,

template (present) O1_Type p_PDU_SessionId,

template (present) ProcedureTransactionIdentifier p_PTI := ?,

template GMM_GSM_Cause p_Cause := *,

template ExtdProtocolConfigOptions p_ExtdPCO := *,

template NG_PDU_SessionId p_OldPDU_SessionId  := *,

template NG_Request_Type p_RequestType  := *,

template S_NSSAI_Type p_S_NSSAI  := *,

template DNN p_DNN  := *,

template AdditionalInformation p_AdditionalInfo  := *,

template MA_PDUSessionInfo p_MA_PDUSessionInfo := *,      // @sic R5s201387 Baseline Moving sic@

template ReleaseAssistanceInd p_ReleaseAssistanceInd := *) return boolean

{

var boolean v_Result := false;

var boolean v_ExistingId := false;

var NG_NAS_UL_Message_Type v_GSMMsg := p_ReceivedMsgs.PiggybackedPduList[0].Msg;

var integer i;

// Check NAS Transport msg first

v_Result := f_Check_NG_ULNASTransport (p_ReceivedMsgs.Msg,

cr_PayloadContainerType(tsc_PayloadContainerESMMsg), // @sic R5w190208 sic@

p_PDU_SessionId,

p_OldPDU_SessionId,

p_RequestType,

p_S_NSSAI,

p_DNN,

p_AdditionalInfo,

p_MA_PDUSessionInfo,   // @sic R5s201387 Baseline Moving sic@

p_ReleaseAssistanceInd);

if (not v_Result) {

return v_Result;

} else if (match (v_GSMMsg, cr_NG_PDU_SESSION_RELEASE_REQUEST (cr_NG_PDU_SessionId(-, p_PDU_SessionId), // @sic R5s200564 sic@

p_PTI,

p_Cause,

p_ExtdPCO))) {

v_Result := true;

} else {  // @sic R5s190944 sic@

v_Result := false;

}

// Default message contents specifies this must match an existing session establishment id

for (i := 0; i < lengthof(p_PDUSessionGlobalInfo); i := i+1) {

if (match (v_GSMMsg.pdu_Session_Release_Request.pduSessionId.sessionId, p_PDUSessionGlobalInfo[i].SessionId)) {

v_ExistingId := true;

}

}

if (not v_ExistingId) {

v_Result := v_ExistingId;

}

// Store info for this release

p_GSM_MobilityInfo.SessionId := v_GSMMsg.pdu_Session_Release_Request.pduSessionId.sessionId; // @sic R5s200564 sic@

p_GSM_MobilityInfo.PTI := v_GSMMsg.pdu_Session_Release_Request.procedureTransactionIdentifier;

p_GSM_MobilityInfo.SessionType := omit;

p_GSM_MobilityInfo.EPS_Bearer := omit; // @sic R5s201533 sic@

p_GSM_MobilityInfo.PdnIndex := omit; // @sic R5s201533 sic@

if (ispresent (p_ReceivedMsgs.Msg.ul_Nas_Transport.dnn)) {

p_GSM_MobilityInfo.DNN := p_ReceivedMsgs.Msg.ul_Nas_Transport.dnn;

} else {

p_GSM_MobilityInfo.DNN := omit;

}

p_GSM_MobilityInfo.PDUType := f_GetPDNTypeFromDNN(p_GSM_MobilityInfo.DNN);  // @sic R5-206296 sic@

p_GSM_MobilityInfo.AlwaysOn := omit;

if (ispresent (v_GSMMsg.pdu_Session_Release_Request.extdProtocolConfigurationOptions)) {

p_GSM_MobilityInfo.PCO := v_GSMMsg.pdu_Session_Release_Request.extdProtocolConfigurationOptions;

} else {

p_GSM_MobilityInfo.PCO := omit;

}

p_GSM_MobilityInfo.NSSAI := omit; // @sic R5s210408 sic@

return v_Result;

}

After Change:

function f_Check_NG_PDUSessionReleaseReq( out GSM_MobilityInfo_Type p_GSM_MobilityInfo,

PDUSessionInfoList_Type p_PDUSessionGlobalInfo,

NG_NAS_UL_Pdu_Type p_ReceivedMsgs,

template (present) O1_Type p_PDU_SessionId,

template (present) ProcedureTransactionIdentifier p_PTI := ?,

template GMM_GSM_Cause p_Cause := *,

template ExtdProtocolConfigOptions p_ExtdPCO := *,

template NG_PDU_SessionId p_OldPDU_SessionId  := *,

template NG_Request_Type p_RequestType  := *,

template S_NSSAI_Type p_S_NSSAI  := *,

template DNN p_DNN  := *,

template AdditionalInformation p_AdditionalInfo  := *,

template MA_PDUSessionInfo p_MA_PDUSessionInfo := *,      // @sic R5s201387 Baseline Moving sic@

template ReleaseAssistanceInd p_ReleaseAssistanceInd := *) return boolean

{

var boolean v_Result := false;

var boolean v_ExistingId := false;

var NG_NAS_UL_Message_Type v_GSMMsg := p_ReceivedMsgs.PiggybackedPduList[0].Msg;

var integer i;

// Check NAS Transport msg first

v_Result := f_Check_NG_ULNASTransport (p_ReceivedMsgs.Msg,

cr_PayloadContainerType(tsc_PayloadContainerESMMsg), // @sic R5w190208 sic@

p_PDU_SessionId,

p_OldPDU_SessionId,

p_RequestType,

p_S_NSSAI,

p_DNN,

p_AdditionalInfo,

p_MA_PDUSessionInfo,   // @sic R5s201387 Baseline Moving sic@

p_ReleaseAssistanceInd);

if (not v_Result) {

return v_Result;

} else if (match (v_GSMMsg, cr_NG_PDU_SESSION_RELEASE_REQUEST (cr_NG_PDU_SessionId(-, p_PDU_SessionId), // @sic R5s200564 sic@

p_PTI,

p_Cause,

p_ExtdPCO))) {

v_Result := true;

} else {  // @sic R5s190944 sic@

v_Result := false;

return v_Result;

}

// Default message contents specifies this must match an existing session establishment id

for (i := 0; i < lengthof(p_PDUSessionGlobalInfo); i := i+1) {

if (match (v_GSMMsg.pdu_Session_Release_Request.pduSessionId.sessionId, p_PDUSessionGlobalInfo[i].SessionId)) {

v_ExistingId := true;

}

}

if (not v_ExistingId) {

v_Result := v_ExistingId;

}

// Store info for this release

p_GSM_MobilityInfo.SessionId := v_GSMMsg.pdu_Session_Release_Request.pduSessionId.sessionId; // @sic R5s200564 sic@

p_GSM_MobilityInfo.PTI := v_GSMMsg.pdu_Session_Release_Request.procedureTransactionIdentifier;

p_GSM_MobilityInfo.SessionType := omit;

p_GSM_MobilityInfo.EPS_Bearer := omit; // @sic R5s201533 sic@

p_GSM_MobilityInfo.PdnIndex := omit; // @sic R5s201533 sic@

if (ispresent (p_ReceivedMsgs.Msg.ul_Nas_Transport.dnn)) {

p_GSM_MobilityInfo.DNN := p_ReceivedMsgs.Msg.ul_Nas_Transport.dnn;

} else {

p_GSM_MobilityInfo.DNN := omit;

}

p_GSM_MobilityInfo.PDUType := f_GetPDNTypeFromDNN(p_GSM_MobilityInfo.DNN);  // @sic R5-206296 sic@

p_GSM_MobilityInfo.AlwaysOn := omit;

if (ispresent (v_GSMMsg.pdu_Session_Release_Request.extdProtocolConfigurationOptions)) {

p_GSM_MobilityInfo.PCO := v_GSMMsg.pdu_Session_Release_Request.extdProtocolConfigurationOptions;

} else {

p_GSM_MobilityInfo.PCO := omit;

}

p_GSM_MobilityInfo.NSSAI := omit; // @sic R5s210408 sic@

return v_Result;

}