3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220990

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2706 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 6.3.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - After Step 17 due to the setting of SOR, it’s possible that the UE will perform the reselection after the N1 signaling releasing to obtain service on higher priority PLMN. i.e., TS 23.122 , section C.2 If the UE has a list of available PLMNs in the area and based on this list the UE determines that there is a higher priority PLMN than the currently camped chosen VPLMN and the UE is in automatic network selection mode, then the UE shall attempt to obtain service on a higher priority PLMN as specified in subclause 4.4.3.3 by acting as if timer T that controls periodic attempts has expired after the release of the N1 NAS signalling connection. So, this may cause an error before the postamble procedure as UE may start pdu establishment on a new rrc connection after Step 26b3-26b5a1. In order to accommodate this rrc connection needs to be released as well after mobility registration at step 26b3-26b5a1. - Following registration procedure SRB2 is not established so PDU session establishment is expected on SRB1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Reworked the TTCN and added steps to allow UE to perform PDU session establishment based on pc_noOf_PDUsSameConnection > 0 or pc_noOf_PDUsNewConnection > 0 - Pass SRB1 parameter into PDU session establishment function instead of SRB2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | R5-225167 is raised for this and will be persented at RAN5#96e |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to fl_TC_6_3_1_3_TestBody	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to fl_TC_6_3_1_3_TestBody

| Function name | fl_TC_6_3_1_3_TestBody |
| --- | --- |
| Reason for change | - After Step 17 due to the setting of SOR, it’s possible that the UE will perform the reselection after the N1 signaling releasing to obtain service on higher priority PLMN. i.e., TS 23.122 , section C.2 If the UE has a list of available PLMNs in the area and based on this list the UE determines that there is a higher priority PLMN than the currently camped chosen VPLMN and the UE is in automatic network selection mode, then the UE shall attempt to obtain service on a higher priority PLMN as specified in subclause 4.4.3.3 by acting as if timer T that controls periodic attempts has expired after the release of the N1 NAS signalling connection. So, this may cause an error before the postamble procedure as UE may start pdu establishment on a new rrc connection after Step 26b3-26b5a1. In order to accommodate this rrc connection needs to be released after mobility registration at step 26b3-26b5a1. NOTE: A Prose CR shall be raised at the next RAN5. - Following registration procedure SRB2 is not established so PDU session establishment is expected on SRB1. |
| Summary of change | - Reworked the TTCN and dded steps to allow UE to perform PDU session establishment based on pc_noOf_PDUsSameConnection > 0 or pc_noOf_PDUsNewConnection > 0 - Pass SRB1 parameter into PDU session establishment function instead of SRB2. |
| TTCN module | \ NR5GC\6_3_1\SOR_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_6_3_1_3_TestBody () runs on NR5GC_PTC { var GMM_MobilityInfo_Type v_GMM_MobilityInfo;//@sic R5-202599 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg;//@sic R5-202599 sic@ var TypeOfRegistration_Type v_TypeOfRegistration;//@sic R5-202599 sic@ var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var NAS_PlmnId v_PlmnID_3 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12));//@sic R5-204543 sic@ var O2_Type v_AccessTechId :='0800'O;//@sic R5s200680 sic@ var octetstring v_Data:=v_PlmnID_2 & v_AccessTechId; var octetstring v_Data_2:=v_PlmnID_3 & v_AccessTechId;//@sic R5-204543 sic@ var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B); timer t_Tmax; timer t_Pdu:= 5.0; //@siclog "Step 1" siclog@ //Power on the UE f_UT_SwitchOnUE(UT); //@siclog "Steps 2-14" siclog@ //Steps 1 to 13 of the registration procedure described in TS 38.508 subclause 4.5.2.2-2 are performed on NR Cell 13siclog "Steps 2-15" siclog@ //@siclog "Step 15" siclog@ //The SS transmits an DLInformationTransfer message and an REGISTRATION ACCEPT message containing //steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception f_NR5GC_SorRegistration(nr_Cell13,v_Data,v_Header,true,-,(tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));//@sic R5s210658 R5s211298 sic@ //@siclog "Step 16" siclog@ //SS starts timer of tmax =(6 minutes + cell selection time) (Note 1, 2 and 3) t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell13, nonProtocolTimer, 660.0)); //@siclog "Step 17" siclog@ //The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without Steering of Roaming Transparent container. f_NR5GC_Registration_Complete(nr_Cell13); //@siclog "Steps 18-21a1 - Void" siclog@ //@sic R5-202599 sic@ //@siclog "Step 22" siclog@ //SS perform the local release.//@sic R5-202599 sic@ f_NR_RRCRelease_Local(nr_Cell13, f_NR_GetNextSendOccasion_DL(nr_Cell13)); //@siclog "Step 23" siclog@ //Check: Does the UE transmits an RRCSetupRequest on NR Cell 12 before tmax expires? alt { //@sic R5s201466 sic@ [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_Tmax.stop; f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 23 PASS"); } [] t_Tmax.timeout { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Step 23 Failed"); } } //@siclog "Step 24" siclog@ //The SS transmits an RRCSetup message on NR Cell 12.//@sic R5-202599 sic@ //@siclog "Step 25" siclog@ //The UE transmits an RRCSetupComplete message and a REGISTRATION REQUEST message on NR Cell 12.//@sic R5-202599 sic@ v_TypeOfRegistration := f_NR5GC_InitialOrMobilityRegistrationRequest (v_GMM_MobilityInfo, v_ReceivedMsg, nr_Cell12); if (v_TypeOfRegistration == Initial_Secure) { //@siclog "Steps 26a1-26a15a1" siclog@//@sic R5-202599 R5-204543 sic@ // IF 5GS registration type is set as Initial Registration in step 25, THEN steps 5 to 20a1 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 12. f_NR5GC_SorRegistration_Step5_20(nr_Cell12, v_Data_2, v_Header, crs_NG_SOR_Header('0'B,'0'B,'0'B,'1'B),//@sic R5-205368 sic@ -, Initial_Secure, v_GMM_MobilityInfo, v_ReceivedMsg); } else if(v_TypeOfRegistration == Mobility){ // @siclog "Steps 26b1-26b2a1" siclog@//@sic R5-202599 sic@ //ELSE IF 5GS registration type is set as Mobility Registration in step 25, THEN steps 2 to 6a1 of the generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 are performed on NR Cell 12. f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell12,v_GMM_MobilityInfo); //exception:The UE performs the generic procedure for UE-requested PDU session(s) establishment, specified in TS 38.508-1 [4] subclause 4.5A.2, performing establishment of UE-requested PDU session(s). t_Pdu.start; alt { [] SRB.check(receive(car_NR_SRB_NasPdu_IND(nr_Cell12, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered))))) { t_Pdu.stop; f_NR5GC_PDUSessionEstablishment (nr_Cell12); } [] t_Pdu.timeout { } } } //@siclog "Steps 27-28a1 Void" siclog@ }//End of fl_TC_6_3_1_3_TestBody

After Change:

function fl_TC_6_3_1_3_TestBody() runs on NR5GC_PTC { var GMM_MobilityInfo_Type v_GMM_MobilityInfo;//@sic R5-202599 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg;//@sic R5-202599 sic@ var TypeOfRegistration_Type v_TypeOfRegistration;//@sic R5-202599 sic@ var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var NAS_PlmnId v_PlmnID_3 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12));//@sic R5-204543 sic@ var O2_Type v_AccessTechId :='0800'O;//@sic R5s200680 sic@ var octetstring v_Data:=v_PlmnID_2 & v_AccessTechId; var octetstring v_Data_2:=v_PlmnID_3 & v_AccessTechId;//@sic R5-204543 sic@ var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B); timer t_Tmax; //timer t_Pdu:= 5.0; //WA#WI=987519 //@siclog "Step 1" siclog@ //Power on the UE f_UT_SwitchOnUE(UT); //@siclog "Steps 2-14" siclog@ //Steps 1 to 13 of the registration procedure described in TS 38.508 subclause 4.5.2.2-2 are performed on NR Cell 13siclog "Steps 2-15" siclog@ //@siclog "Step 15" siclog@ //The SS transmits an DLInformationTransfer message and an REGISTRATION ACCEPT message containing //steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception f_NR5GC_SorRegistration(nr_Cell13,v_Data,v_Header,true,-,(tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));//@sic R5s210658 R5s211298 sic@ //@siclog "Step 16" siclog@ //SS starts timer of tmax =(6 minutes + cell selection time) (Note 1, 2 and 3) t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell13, nonProtocolTimer, 660.0)); //@siclog "Step 17" siclog@ //The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without Steering of Roaming Transparent container. f_NR5GC_Registration_Complete(nr_Cell13); //@siclog "Steps 18-21a1 - Void" siclog@ //@sic R5-202599 sic@ //@siclog "Step 22" siclog@ //SS perform the local release.//@sic R5-202599 sic@ f_NR_RRCRelease_Local(nr_Cell13, f_NR_GetNextSendOccasion_DL(nr_Cell13)); //@siclog "Step 23" siclog@ //Check: Does the UE transmits an RRCSetupRequest on NR Cell 12 before tmax expires? alt { //@sic R5s201466 sic@ [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_Tmax.stop; f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 23 PASS"); } [] t_Tmax.timeout { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Step 23 Failed"); } } //@siclog "Step 24" siclog@ //The SS transmits an RRCSetup message on NR Cell 12.//@sic R5-202599 sic@ //@siclog "Step 25" siclog@ //The UE transmits an RRCSetupComplete message and a REGISTRATION REQUEST message on NR Cell 12.//@sic R5-202599 sic@ v_TypeOfRegistration := f_NR5GC_InitialOrMobilityRegistrationRequest (v_GMM_MobilityInfo, v_ReceivedMsg, nr_Cell12); if (v_TypeOfRegistration == Initial_Secure) { //@siclog "Steps 26a1-26a15a1" siclog@//@sic R5-202599 R5-204543 sic@ // IF 5GS registration type is set as Initial Registration in step 25, THEN steps 5 to 20a1 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 12. f_NR5GC_SorRegistration_Step5_20(nr_Cell12, v_Data_2, v_Header, crs_NG_SOR_Header('0'B,'0'B,'0'B,'1'B),//@sic R5-205368 sic@ -, Initial_Secure, v_GMM_MobilityInfo, v_ReceivedMsg); } else if(v_TypeOfRegistration == Mobility) { // @siclog "Steps 26b1-26b2a1" siclog@//@sic R5-202599 sic@ //ELSE IF 5GS registration type is set as Mobility Registration in step 25, THEN steps 2 to 6a1 of the generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 are performed on NR Cell 12. f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell12,v_GMM_MobilityInfo, -, -, noRrcConnectionRelease); //WA#WI=987519 //exception:The UE performs the generic procedure for UE-requested PDU session(s) establishment, specified in TS 38.508-1 [4] subclause 4.5A.2, performing establishment of UE-requested PDU session(s). if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (nr_Cell12,-,tsc_NR_RbId_SRB1); //WA#WI=987519 f_NR_RRCRelease(nr_Cell12); } if (pc_noOf_PDUsNewConnection > 0 ){ f_NR_RRCRelease(nr_Cell12); f_NR5GC_RRC_Idle_Extension(nr_Cell12, TESTMode_OFF) //WA#WI=987519, and release the connection. } } //@siclog "Steps 27-28a1 Void" siclog@ }//End of fl_TC_6_3_1_3_TestBody