3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221013

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2708 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC testcase 6.5.2.3 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR5GC test case 6.5.2.3 to the IWD_22wk23 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 6.5.2.3 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | R5-224388 is raised for this and will be presented at RAN5#96e |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to test case 6.5.2.3	4

2.1	f_TC_6_5_2_3_NR5GC	4

2.2	TC_6_5_2_3_NR5GC()	5

3	Branches executed	7

4	Execution Log Files	7

4.1	Hisilicon Balong 5000 UE	7

5	References	7

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 6.5.2.3 which is part of the NR5GC test suite in iWD_TTCN3-B2020-09_D22wk23.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	6.5.2.3.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D22wk23

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Hisilicon Balong 5000 UE

Verification Status:	PASS

# Corrections required to test case 6.5.2.3

## f_TC_6_5_2_3_NR5GC

| Function name | f_TC_6_5_2_3_NR5GC |
| --- | --- |
| Reason for change | In the current TTCN, NR Cell 2 and NR Cell 4 are CAG cells. The current TTCN does not set the IE cellReservedForOtherUse to TRUE in SIB1 for these. As a result, a UE may camp on these cells simply because of S criterion, not because these are CAG. This needs to be addressed. |
| Summary of change | Called the function f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(nr_Cell2, true_) and f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(nr_Cell4, true_) in order to set the IE cellReservedForOtherUse to TRUE in SIB1. A Prose CR will be raised for this. |
| TTCN module | CAG_Selection_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> //Cell2 and Cell4 are CAG Cells. default CAG-ID value used for Cell2, value 2 used for Cell4 f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell2); f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell4, 2); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell2); //CAG Cell f_NR_CellConfig_Def(nr_Cell4); //CAG Cell f_NR_CellConfig_Def(nr_Cell11); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //Cell2 and Cell4 are CAG Cells. default CAG-ID value used for Cell2, value 2 used for Cell4 f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell2); f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell4, 2); f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(nr_Cell4, true_);//WA#6_5_2_3 f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(nr_Cell2, true_);//WA#6_5_2_3 //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell2); //CAG Cell f_NR_CellConfig_Def(nr_Cell4); //CAG Cell f_NR_CellConfig_Def(nr_Cell11); <<SKIPPED CODE>>

## TC_6_5_2_3_NR5GC()

| Function name | TC_6_5_2_3_NR5GC() |
| --- | --- |
| Reason for change | Current guardtimer value of 300s is insufficient for this testcase. This needs to be addressed. |
| Summary of change | Increased the guard timer to 600s. |
| TTCN module | NR5GC_TestSuite.ttcn |
| MCC160 Comment |  |

Before Change:

testcase TC_6_5_2_3_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // CAG / Limited Service / No Suitable cell var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(300); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_6_5_2_3_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

After Change:

testcase TC_6_5_2_3_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // CAG / Limited Service / No Suitable cell var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(600); //WA#6_5_2_3 300-->600 v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_6_5_2_3_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n41” using NIA1 Integrity and NEA1 ciphering algorithms

# Execution Log Files

## Hisilicon Balong 5000 UE

The Hisilicon Balong 5000 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
tc_6_5_2_3_Hisilicon.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s221014: Supporting information for addition of NR5GC testcase 6.5.2.3 in FR1 |
| --- | --- |