3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221019

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2711 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC RRC NPN test case 8.1.7.1.1 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR5GC SNPN test case 8.1.7.1.1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC RRC NPN test case 8.1.7.1.1 for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.7.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR TS38-523-1, TS38.508-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	6

2.3	Change 3	7

2.4	Change 4	7

3	Branches executed	8

4	Execution Log Files	8

4.1	Qualcomm Snapdragon X70 5G Modem-RF System	8

5	References	9

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.7.1.1 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	8.1.7.1.1

ATS Version:	iwd-TTCN3-B2020-09_D22wk23

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR,

UE used:	Qualcomm Snapdragon X70 5G Modem-RF System 5G UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | function f_TC_8_1_7_1_1_NR5GC() |
| --- | --- |
| Reason for change | Since NR Cell 2 is broadcasting plmn-IdentityInfoList, it must be included in the MR along with frequencyBandList. [TS 38.331, clause 5.5.5.1] 3> if the cell indicated by cellForWhichToReportCGI is an NR cell: 4> if plmn-IdentityInfoList of the cgi-Info for the concerned cell has been obtained: 5> include the plmn-IdentityInfoList including plmn-IdentityList, trackingAreaCode (if available), ranac (if available), cellIdentity and cellReservedForOperatorUse for each entry of the plmn-IdentityInfoList; 5> include frequencyBandList if available; 4> if nr-CGI-Reporting-NPN is supported by the UE and npn-IdentityInfoList of the cgi-Info for the concerned cell has been obtained: 5> include the npn-IdentityInfoList including npn-IdentityList, trackingAreaCode, ranac (if available), cellIdentity and cellReservedForOperatorUse for each entry of the npn-IdentityInfoList; 5> include cellReservedForOtherUse if available; |
| Summary of change | Include plmn-IdentityInfoList and frequencyBandList in the checking of Step 3 measurement report Note: An associated prose CR(R5-224346) on 38.523-1 is raised at RAN5#96 |
| TTCN module | RRC_NPN_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_7_1_1_NR5GC() runs on NR5GC_PTC { //Measurement configuration control and reporting / CGI reporting of NR NPN cell var PhysCellId v_PhysCellIdServing; var PhysCellId v_PhysCellIdNeighbour; var NPN_IdentityInfoList_r16 v_NPN_IdentityInfoList; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var float v_Wait; f_NR5GC_Init(NR_2); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -88, -82); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell2); //CAG cell2 f_NR_CellConfig_Def(nr_Cell2); f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A); f_NR_SetCellPower (nr_Cell2, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2); f_NR_TestBody_Set(true); //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCReconfiguration message including measConfig including reportCGI for Cell 2 //and sufficient idle periods for UE to acquire the relevant system information from Cell 2 //The UE transmits an RRCReconfigurationComplete message. v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1)}; v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(nr_Cell2); v_ReportConfigList := {cs_NR_ReportConfigCGI(tsc_NR_IdReportConfigId1, v_PhysCellIdNeighbour)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1} v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList); f_NR_SendRRCReconfiguration_MeasNR_CellGroupConfig_DRX(nr_Cell1, v_MeasConfig, omit, cs_38508_MAC_CellGroupConfig_DRX); //@siclog "Step 3" siclog@ //Check: Does the UE transmit a MeasurementReport message //with CGI of NR Cell 2 within 2 sec if Cell 2 is in FR1 (or within 16 sec if Cell 2 is in FR2)? if(f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_Wait := 2.0 } else { v_Wait := 16.0 } v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_NPN_IdentityInfoList := f_NR_GetSIB1_NPN_IdentityInfoList(nr_Cell2); f_NR_CheckReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId1, v_Wait, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults_NPN(v_PhysCellIdNeighbour, cr_CGI_InfoNR_NPN_IdentityInfoList(v_NPN_IdentityInfoList))}),cr_NR_MeasQuantityResults_Any); f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 3"); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }// End of f_TC_8_1_7_1_1_NR5GC()

After Change:

function f_TC_8_1_7_1_1_NR5GC() runs on NR5GC_PTC { //Measurement configuration control and reporting / CGI reporting of NR NPN cell var PhysCellId v_PhysCellIdServing; var PhysCellId v_PhysCellIdNeighbour; var NPN_IdentityInfoList_r16 v_NPN_IdentityInfoList; var PLMN_IdentityInfoList v_PLMN_IdentityInfoList; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var float v_Wait; var FreqBandIndicatorNR v_FreqBandIndicatorNR; f_NR5GC_Init(NR_2); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -88, -82); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell2); //CAG cell2 f_NR_CellConfig_Def(nr_Cell2); f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A); f_NR_SetCellPower (nr_Cell2, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2); f_NR_TestBody_Set(true); //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCReconfiguration message including measConfig including reportCGI for Cell 2 //and sufficient idle periods for UE to acquire the relevant system information from Cell 2 //The UE transmits an RRCReconfigurationComplete message. v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1)}; v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(nr_Cell2); v_ReportConfigList := {cs_NR_ReportConfigCGI(tsc_NR_IdReportConfigId1, v_PhysCellIdNeighbour)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1} v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList); f_NR_SendRRCReconfiguration_MeasNR_CellGroupConfig_DRX(nr_Cell1, v_MeasConfig, omit, cs_38508_MAC_CellGroupConfig_DRX); //@siclog "Step 3" siclog@ //Check: Does the UE transmit a MeasurementReport message //with CGI of NR Cell 2 within 2 sec if Cell 2 is in FR1 (or within 16 sec if Cell 2 is in FR2)? if(f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_Wait := 2.0 } else { v_Wait := 16.0 } v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_NPN_IdentityInfoList := f_NR_GetSIB1_NPN_IdentityInfoList(nr_Cell2); v_PLMN_IdentityInfoList := f_NR_GetSIB1_PLMN_IdentityInfoList(nr_Cell2); v_FreqBandIndicatorNR := f_NR_CellInfo_GetBandIndicator(nr_Cell2); f_NR_CheckReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId1, v_Wait, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults_NPN(v_PhysCellIdNeighbour, cr_CGI_InfoNR_NPN_IdentityInfoList(v_NPN_IdentityInfoList, v_PLMN_IdentityInfoList, {v_FreqBandIndicatorNR}))}),cr_NR_MeasQuantityResults_Any); f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 3"); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }// End of f_TC_8_1_7_1_1_NR5GC()

## Change 2

| Function name | function f_NR_GetSIB1_PLMN_IdentityInfoList() |
| --- | --- |
| Reason for change | A function for retrieving the PLMN of a given cell in the form of ‘PLMN_IdentityInfoList’ data type is needed for CGI information checking |
| Summary of change | Define new function |
| TTCN module | RRC_NPN_NR5GC |
| MCC160 Comment |  |

New Function:

function f_NR_GetSIB1_PLMN_IdentityInfoList(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC return PLMN_IdentityInfoList { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); if (not isbound(v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.plmn_IdentityInfoList)) { FatalError(__FILE__, __LINE__, "SIB1 not found"); } return v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.plmn_IdentityInfoList; }

## Change 3

| Function name | template CGI_InfoNR cr_CGI_InfoNR_NPN_IdentityInfoList() |
| --- | --- |
| Reason for change | Same as Change 1 |
| Summary of change | Update the CGI Info checking template to include plmn-IdentityInfoList, frequencyBandList, and cellReservedForOtherUse Note: An associated prose CR(R5-224326) on 38.508-1 is raised at RAN5#96 |
| TTCN module | RRC_NPN_NR5GC |
| MCC160 Comment |  |

Before Change:

template CGI_InfoNR cr_CGI_InfoNR_NPN_IdentityInfoList (NPN_IdentityInfoList_r16 p_NPN_IdentityInfoList_r16) := cr_CGI_InfoNR_Base(-,-,-,p_NPN_IdentityInfoList_r16);

After Change:

template CGI_InfoNR cr_CGI_InfoNR_NPN_IdentityInfoList (NPN_IdentityInfoList_r16 p_NPN_IdentityInfoList_r16, PLMN_IdentityInfoList p_PLMN_IdentityInfoList, MultiFrequencyBandListNR p_MultiFrequencyBandListNR) := cr_CGI_InfoNR_Base(p_PLMN_IdentityInfoList, p_MultiFrequencyBandListNR,-,p_NPN_IdentityInfoList_r16, true_);

## Change 4

| Function name | function f_GetTestcaseAttrib_NR_SNPN() |
| --- | --- |
| Reason for change | 8.1.7.1.1 does not configure any SNPN-only cell, so it should not be included in the SNPN list |
| Summary of change | Remove 8.1.7.1.1 from the SNPN TC list |
| TTCN module | NR5GC\Common\TestcaseProperties.ttcn |
| MCC160 Comment |  |

Before Change:

function f_GetTestcaseAttrib_NR_SNPN(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_6_5_1_1_NR5GC") { return true; } case ("TC_6_5_1_2_NR5GC") { return true; } case ("TC_6_5_1_3_NR5GC") { return true; } case ("TC_8_1_7_1_1_NR5GC") { return true; } case ("TC_9_1_11_1_NR5GC") { return true; } case ("TC_9_1_11_2_NR5GC") { return true; } case ("TC_9_1_11_3_NR5GC") { return true; } } return false; }

After Change:

function f_GetTestcaseAttrib_NR_SNPN(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_6_5_1_1_NR5GC") { return true; } case ("TC_6_5_1_2_NR5GC") { return true; } case ("TC_6_5_1_3_NR5GC") { return true; } // REMOVED case ("TC_8_1_7_1_1_NR5GC") { return true; } case ("TC_9_1_11_1_NR5GC") { return true; } case ("TC_9_1_11_2_NR5GC") { return true; } case ("TC_9_1_11_3_NR5GC") { return true; } } return false; }

# Branches executed

On Anritsu, the test case was executed on NR band n41 using NR ciphering nea2 and integrity algorithm nia2.

# Execution Log Files

## Qualcomm Snapdragon X70 5G Modem-RF System

The Snapdragon X70 5G Modem-RF System, Qualcomm Technologies’ 5th generation modem-to-antenna UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\ TC_8_1_7_1_1_LOG.xml

- PICS/PIXIT parameter file:

Anritsu\ TC_8_1_7_1_1_PIXIT.txt

# References

| [1] | R5s221020 - Supporting information for addition of NR5GC RRC NPN test case 8.1.7.1.1 in FR1 |
| --- | --- |