3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221030

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2717 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC NAS SNPN test case 9.1.11.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR5GC NAS SNPN test case 9.1.11.1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC NAS SNPN test case 9.1.11.1 for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.11.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR TS38-523-1, TS38-508-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

3	Branches executed	11

4	Execution Log Files	11

4.1	Qualcomm Snapdragon X70 5G Modem-RF System	11

5	References	11

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 9.1.11.1 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	9.1.11.1

ATS Version:	iwd-TTCN3-B2020-09_D22wk23

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR,

UE used:	Qualcomm Snapdragon X70 5G Modem-RF System 5G UE

Verification Status:	PASS

# Corrections required

Note: Changes in R5s220753 are also applied for this test case .

## Change 1

| Function name | function f_TC_9_1_11_1_NR5GC() |
| --- | --- |
| Reason for change | For SNPN specific test cases, the cells need to be configured as NPN-only cells TS 38.331: NPN-only Cell: A cell that is only available for normal service for NPNs' subscriber. An NPN-capable UE determines that a cell is NPN-only Cell by detecting that the cellReservedForOtherUse IE is set to true while the npn-IdentityInfoList IE is present in CellAccessRelatedInfo. |
| Summary of change | Set SIB1 cellReservedForOtherUSE IE to True Note: An associated prose CR(R5-224326) on 38.508-1 will be raised at RAN5#96 |
| TTCN module | SNPN_NAS_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_9_1_11_1_NR5GC() runs on NR5GC_PTC { //SNPN / Initial registration / Rejected / Temporarily not authorized for this SNPN var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); f_NR5GC_Init_NAS(NR_12); // Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); f_NR_CellConfig_Def(v_NGC_NASCellB); f_UT_SwitchOnUE(UT); f_UT_ProvideSubscriberDataList(UT, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), 1); f_UT_ProvideSubscriberDataList(UT, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellB)), 2); f_UT_ClearRegisteredSNPN_Select(UT); f_UT_AutomaticSNPN_Select(UT); f_UT_SwitchOffUE(UT, false); f_NR5GC_Preamble (v_NGC_NASCellA, STATE_OFF_0A); f_NR_TestBody_Set(true); fl_TC_9_1_11_1_Body(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(v_NGC_NASCellA, STATE_IDLE_1A); }

After Change:

function f_TC_9_1_11_1_NR5GC() runs on NR5GC_PTC { //SNPN / Initial registration / Rejected / Temporarily not authorized for this SNPN var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var template (omit) SIB1.cellAccessRelatedInfo.cellReservedForOtherUse v_Sib1_cellReservedForOtherUse := true_; f_NR5GC_Init_NAS(NR_12); f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(v_NGC_NASCellA, valueof(v_Sib1_cellReservedForOtherUse)); f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(v_NGC_NASCellB, valueof(v_Sib1_cellReservedForOtherUse)); // Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); f_NR_CellConfig_Def(v_NGC_NASCellB); f_UT_SwitchOnUE(UT); f_UT_ProvideSubscriberDataList(UT, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), 1); f_UT_ProvideSubscriberDataList(UT, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellB)), 2); f_UT_ClearRegisteredSNPN_Select(UT); f_UT_AutomaticSNPN_Select(UT); f_UT_SwitchOffUE(UT, false); f_NR5GC_Preamble (v_NGC_NASCellA, STATE_OFF_0A); f_NR_TestBody_Set(true); fl_TC_9_1_11_1_Body(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(v_NGC_NASCellA, STATE_IDLE_1A); }

## Change 2

| Function name | function f_TC_9_1_11_1_NR5GC_TestBody() |
| --- | --- |
| Reason for change | - At step 28, the SS sends a REGISTRATION REJECT message to the UE including an appropriate 5GMM cause value #74 (Temporarily not authorized for this SNPN) so at step 31, UE is not expected to send REGISTRATION REQUEST message on NGC Cell A. Wait time is missing at Step 31 to check for the Registration message from the UE. - Step 33 is not aligned with spec. UE has been rejected on NGC Cell B so will not need deregistration. |
| Summary of change | - Step 31 is updated to ensure that UE does not send Registration Request since NGC Cell A has not yet been cleared from “temporarily forbidden SNPN list” Note: An associated prose CR on 38.523-1(R5-224344) will be raised at RAN5#96 - Switch Off UE without Deregistration |
| TTCN module | SNPN_NAS_NR5GC |
| MCC160 Comment |  |

Before Change:

function fl_TC_9_1_11_1_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); var NasCount_Type v_NasCount; timer t_Wait1 := 3600.0; timer t_Wait2 := 60.0; //@siclog "Step 1" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_NonSuitableCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2); //@siclog "Step 2" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 3-14" siclog@ //Steps 2-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_NoSecurity); //@siclog "Steps 15" siclog@ //The SS transmits a REGISTRATION REJECT message, 5GMM cause value = #74 "Temporarily not authorized for this SNPN". SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, '01001010'B))))); //@siclog "Step 16" siclog@ //The SS starts timer 1 = 60 min. t_Wait1.start; //@siclog "Step 17" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 18" siclog@ //Check: Does the UE transmit the REGISTRATION REQUEST message on NGC Cell A in the next 60 seconds? t_Wait2.start; alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(v_NGC_NASCellA, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) { t_Wait2.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 12: RRCSetupRequest received"); } [] t_Wait2.timeout {} } //@siclog "Step 19" siclog@ f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 20" siclog@ //Check: Does the UE transmit the REGISTRATION REQUEST message on NGC Cell B? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellB, tsc_NR_RRC_TI_Def, ?, tsc_SHT_NoSecurityProtection); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity, cr_NG_MobileIdentitySUCI(?,?), //Check the SUCI tsc_NasKsi_NoKey, //check the NAS key set identifier -, -, -, -, -, omit)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 20"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 20"); //@siclog "Step 21-27" siclog@ //Steps 5 to 11 from procedure in TS 38.508-1 [4] Table 4.5.2.2-2 are performed. v_NasCount := f_NR5GC_RRC_Idle_Steps5_9_AKA(v_GMM_MobilityInfo, v_NGC_NASCellB, v_ReceivedMsg, v_PLMN, Initial_NoSecurity); f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); f_NR_RRC_ActivateSecurity (v_NGC_NASCellB, v_NasCount); //@siclog "Steps 28 siclog@ //The SS transmits a REGISTRATION REJECT message, 5GMM cause value = #74 " Temporarily not authorized for this SNPN". SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellB, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, '01001010'B))))); //@siclog "Step 29" siclog@ f_NR_RRCRelease(v_NGC_NASCellB); //@siclog "Step 30" siclog@ f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_NonSuitableCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2); //@siclog "Step 31" siclog@ //Check: Does the UE transmit the REGISTRATION REQUEST message on NGC Cell A? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_NoSecurityProtection); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity, cr_NG_MobileIdentitySUCI(?,?), //Check the SUCI tsc_NasKsi_NoKey, //check the NAS key set identifier -, -, -, -, -, omit)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 31"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 31"); //@siclog "Step 32" siclog@ t_Wait1.stop; //@siclog "Step 33" siclog@ f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellB, STATE_IDLE_1A); //@siclog "Step 34" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 35" siclog@ //Check: Does the UE transmit the REGISTRATION REQUEST message on NGC Cell A? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_NoSecurityProtection); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity, cr_NG_MobileIdentitySUCI(?,?), //Check the SUCI tsc_NasKsi_NoKey, //check the NAS key set identifier -, -, -, -, -, omit)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 35"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 35"); //@siclog "Step 36-51" siclog@ //Steps 5-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NGC Cell A. f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, TESTMode_OFF, -, Initial_NoSecurity); } //End of fl_TC_9_1_11_1_Body

After Change:

function fl_TC_9_1_11_1_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); var NasCount_Type v_NasCount; timer t_Wait1 := 3600.0; timer t_Wait2 := 60.0; //@siclog "Step 1" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_NonSuitableCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2); //@siclog "Step 2" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 3-14" siclog@ //Steps 2-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_NoSecurity); //@siclog "Steps 15" siclog@ //The SS transmits a REGISTRATION REJECT message, 5GMM cause value = #74 "Temporarily not authorized for this SNPN". SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, '01001010'B))))); //@siclog "Step 16" siclog@ //The SS starts timer 1 = 60 min. t_Wait1.start; //@siclog "Step 17" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 18" siclog@ //Check: Does the UE transmit the REGISTRATION REQUEST message on NGC Cell A in the next 60 seconds? t_Wait2.start; alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(v_NGC_NASCellA, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) { t_Wait2.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 12: RRCSetupRequest received"); } [] t_Wait2.timeout {} } //@siclog "Step 19" siclog@ f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 20" siclog@ //Check: Does the UE transmit the REGISTRATION REQUEST message on NGC Cell B? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellB, tsc_NR_RRC_TI_Def, ?, tsc_SHT_NoSecurityProtection); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity, cr_NG_MobileIdentitySUCI(?,?), //Check the SUCI tsc_NasKsi_NoKey, //check the NAS key set identifier -, -, -, -, -, omit)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 20"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 20"); //@siclog "Step 21-27" siclog@ //Steps 5 to 11 from procedure in TS 38.508-1 [4] Table 4.5.2.2-2 are performed. v_NasCount := f_NR5GC_RRC_Idle_Steps5_9_AKA(v_GMM_MobilityInfo, v_NGC_NASCellB, v_ReceivedMsg, v_PLMN, Initial_NoSecurity); f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); f_NR_RRC_ActivateSecurity (v_NGC_NASCellB, v_NasCount); //@siclog "Steps 28 siclog@ //The SS transmits a REGISTRATION REJECT message, 5GMM cause value = #74 " Temporarily not authorized for this SNPN". SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellB, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, '01001010'B))))); //@siclog "Step 29" siclog@ f_NR_RRCRelease(v_NGC_NASCellB); //@siclog "Step 30" siclog@ f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_NonSuitableCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2); //@siclog "Step 31" siclog@ //Check: Does the UE transmit the REGISTRATION REQUEST message on NGC Cell A in the next 60 seconds? t_Wait2.start; alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(v_NGC_NASCellA, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) { t_Wait2.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 12: RRCSetupRequest received"); } [] t_Wait2.timeout {} } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 31"); //@siclog "Step 32" siclog@ t_Wait1.stop; //@siclog "Step 33" siclog@ //f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellB, STATE_IDLE_1A); // REMOVED: At this point NASCellB is still temporarily forbidden f_UT_SwitchOffUE(UT, false); //@siclog "Step 34" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 35" siclog@ //Check: Does the UE transmit the REGISTRATION REQUEST message on NGC Cell A? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_NoSecurityProtection); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity, cr_NG_MobileIdentitySUCI(?,?), //Check the SUCI tsc_NasKsi_NoKey, //check the NAS key set identifier -, -, -, -, -, omit)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 35"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 35"); //@siclog "Step 36-51" siclog@ //Steps 5-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NGC Cell A. f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, TESTMode_OFF, -, Initial_NoSecurity); } //End of fl_TC_9_1_11_1_Body

# Branches executed

On Anritsu, the test case was executed on NR band n41 using NR ciphering nea2 and integrity algorithm nia2.

# Execution Log Files

## Qualcomm Snapdragon X70 5G Modem-RF System

The Snapdragon X70 5G Modem-RF System, Qualcomm Technologies’ 5th generation modem-to-antenna UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\ TC_9_1_11_1_LOG.xml

- PICS/PIXIT parameter file:

Anritsu\ TC_9_1_11_1_PIXIT.txt

# References

| [1] | R5s221031 - Supporting information for addition of NR5GC NAS SNPN test case 9.1.11.1 |
| --- | --- |