3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221041

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0166 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current implementation for f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList does not take into account that following parameters are optional (from 3GPP 37.355): maxNumOfDL-PRS-ResProcessedPerSlot-r16 SEQUENCE { scs15-r16 ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64} OPTIONAL, scs30-r16 ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64} OPTIONAL, scs60-r16 ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64} OPTIONAL, scs120-r16 ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64} OPTIONAL, Current implementation requires UE to report all 4 SCS combinations. This leads into test case failures when testing FR1, since UE will not report SCS 120 for FR1 bands. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the implementation of f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList to follow the OPTIONAL requirement for scs15-r16, scs30-r16, scs60-r16 and scs120-r16 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.2.19s, 9.3.1.2.20s, 9.3.1.2.21s, 9.3.4.1.20s, 9.3.4.1.21s, 9.3.4.2.19s, 9.3.4.2.20s, 9.3.4.2.21s, 9.3.4.3.20s, 9.3.4.3.21s, 9.3.4.4.19s, 9.3.4.4.20s, 9.3.4.4.21s |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

1.1 Correction to f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

## 1.1 Correction to f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList

| Function name | f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList |
| --- | --- |
| Reason for change | Current implementation for f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList does not take into account that following parameters are optional (from 3GPP 37.355): maxNumOfDL-PRS-ResProcessedPerSlot-r16 SEQUENCE { scs15-r16 ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64} OPTIONAL, scs30-r16 ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64} OPTIONAL, scs60-r16 ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64} OPTIONAL, scs120-r16 ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64} OPTIONAL, Current implementation requires UE must report all 4 SCS combinations. This leads into test case failures when testing FR1, since UE will not report SCS 120 for FR1 bands. |
| Summary of change | In the final if statement, report at least one of the 4 SCS combinations, replacing and by or |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

function f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList (NR_DL_PRS_ProcessingCapability_r16.prs_ProcessingCapabilityBandList_r16 p_NrDlPrsProcessingCapabilityBandList) runs on NR_BASE_PTC

{ //@sic POS Rel-16 R5-213638 sic@

var integer v_Index;

var boolean v_Found_SupportedBandwidthPRS_FR1 := false;

var boolean v_Found_SupportedBandwidthPRS_FR2 := false;

var boolean v_Found_Dl_PRS_BufferType := false;

var boolean v_Found_DurationOfPRS_ProcessingSymbols := false;

var boolean v_Found_DurationOfPRS_ProcessingSymbolsInEveryTms := false;

var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS15 := false;

var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS30 := false;

var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS60 := false;

var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS120 := false;

for (v_Index := 0; v_Index < lengthof(p_NrDlPrsProcessingCapabilityBandList); v_Index:=v_Index+1) {

fl_CheckNRBandCapability(p_NrDlPrsProcessingCapabilityBandList[v_Index].freqBandIndicatorNR_r16); // checks freqBandIndicatorNR-r16 against capability PICS

if ((ischosen(p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr1)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr1 == pc_supportedBandwidthPRS_FR1)) {

v_Found_SupportedBandwidthPRS_FR1 := true;

}

if ((ischosen(p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr2)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr2 == pc_supportedBandwidthPRS_FR2)) {

v_Found_SupportedBandwidthPRS_FR2 := true;

}

if (p_NrDlPrsProcessingCapabilityBandList[v_Index].dl_PRS_BufferType_r16 == pc_dl_PRS_BufferType) {

v_Found_Dl_PRS_BufferType := true;

}

if (p_NrDlPrsProcessingCapabilityBandList[v_Index].durationOfPRS_Processing_r16.durationOfPRS_ProcessingSysmbols_r16 == pc_durationOfPRS_ProcessingSymbols) {

v_Found_DurationOfPRS_ProcessingSymbols := true;

}

if (p_NrDlPrsProcessingCapabilityBandList[v_Index].durationOfPRS_Processing_r16.durationOfPRS_ProcessingSymbolsInEveryTms_r16 == pc_PRS_ProcessingSymbolsInEveryTms) {

v_Found_DurationOfPRS_ProcessingSymbolsInEveryTms := true;

}

if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs15_r16)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs15_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS15)) {

v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS15 := true;

}

if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs30_r16)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs30_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS30)) {

v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS30 := true;

}

if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs60_r16)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs60_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS60)) {

v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS60 := true;

}

if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs120_r16)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs120_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS120)) {

v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS120 := true;

}

}

if (not ((v_Found_SupportedBandwidthPRS_FR1 or v_Found_SupportedBandwidthPRS_FR2) and v_Found_Dl_PRS_BufferType and v_Found_DurationOfPRS_ProcessingSymbols and

v_Found_DurationOfPRS_ProcessingSymbolsInEveryTms and v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS15 and v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS30 and

v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS60 and v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS120)) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for Prs_ProcessingCapabilityBandList");

}

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_POS_NR_CheckCapabilities_Prs_ProcessingCapabilityBandList (NR_DL_PRS_ProcessingCapability_r16.prs_ProcessingCapabilityBandList_r16 p_NrDlPrsProcessingCapabilityBandList) runs on NR_BASE_PTC

{ //@sic POS Rel-16 R5-213638 sic@

var integer v_Index;

var boolean v_Found_SupportedBandwidthPRS_FR1 := false;

var boolean v_Found_SupportedBandwidthPRS_FR2 := false;

var boolean v_Found_Dl_PRS_BufferType := false;

var boolean v_Found_DurationOfPRS_ProcessingSymbols := false;

var boolean v_Found_DurationOfPRS_ProcessingSymbolsInEveryTms := false;

var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS15 := false;

var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS30 := false;

var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS60 := false;

var boolean v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS120 := false;

for (v_Index := 0; v_Index < lengthof(p_NrDlPrsProcessingCapabilityBandList); v_Index:=v_Index+1) {

fl_CheckNRBandCapability(p_NrDlPrsProcessingCapabilityBandList[v_Index].freqBandIndicatorNR_r16); // checks freqBandIndicatorNR-r16 against capability PICS

if ((ischosen(p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr1)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr1 == pc_supportedBandwidthPRS_FR1)) {

v_Found_SupportedBandwidthPRS_FR1 := true;

}

if ((ischosen(p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr2)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].supportedBandwidthPRS_r16.fr2 == pc_supportedBandwidthPRS_FR2)) {

v_Found_SupportedBandwidthPRS_FR2 := true;

}

if (p_NrDlPrsProcessingCapabilityBandList[v_Index].dl_PRS_BufferType_r16 == pc_dl_PRS_BufferType) {

v_Found_Dl_PRS_BufferType := true;

}

if (p_NrDlPrsProcessingCapabilityBandList[v_Index].durationOfPRS_Processing_r16.durationOfPRS_ProcessingSysmbols_r16 == pc_durationOfPRS_ProcessingSymbols) {

v_Found_DurationOfPRS_ProcessingSymbols := true;

}

if (p_NrDlPrsProcessingCapabilityBandList[v_Index].durationOfPRS_Processing_r16.durationOfPRS_ProcessingSymbolsInEveryTms_r16 == pc_PRS_ProcessingSymbolsInEveryTms) {

v_Found_DurationOfPRS_ProcessingSymbolsInEveryTms := true;

}

if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs15_r16)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs15_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS15)) {

v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS15 := true;

}

if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs30_r16)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs30_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS30)) {

v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS30 := true;

}

if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs60_r16)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs60_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS60)) {

v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS60 := true;

}

if ((ispresent(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs120_r16)) and

(p_NrDlPrsProcessingCapabilityBandList[v_Index].maxNumOfDL_PRS_ResProcessedPerSlot_r16.scs120_r16 == pc_maxNumOfDL_PRS_ResProcessedPerSlot_SCS120)) {

v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS120 := true;

}

}

if (not ((v_Found_SupportedBandwidthPRS_FR1 or v_Found_SupportedBandwidthPRS_FR2) and v_Found_Dl_PRS_BufferType and v_Found_DurationOfPRS_ProcessingSymbols and

v_Found_DurationOfPRS_ProcessingSymbolsInEveryTms and (v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS15 or v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS30 or

v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS60 or v_Found_MaxNumOfDL_PRS_ResProcessedPerSlot_SCS120))) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for Prs_ProcessingCapabilityBandList");

}

}

<<SKIPPED CODE>>