3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221054

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2724 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 11.3.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation if a UE follows the Steps 6Ba1, then it fails the testcase. This is because after receiving the rrcSetupREQ on NR Cell 12, the SS is ot starting the UL grant. This needs to be fixed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | After receiving the rrcSetupREQ at Step 6Ba1, called the function f_NR_ULGrantConfiguration_Start. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.9.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to fl_TC_11_3_9_TestBody	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to fl_TC_11_3_9_TestBody

| Function name | fl_TC_11_3_9_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation if a UE follows the Steps 6Ba1, then it fails the testcase. This is because after receiving the rrcSetupREQ on NR Cell 12, the SS is ot starting the UL grant. This needs to be fixed. |
| Summary of change | After receiving the rrcSetupREQ at Step 6Ba1, called the function f_NR_ULGrantConfiguration_Start. |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> // @siclog "Steps 6A - 6Bb1" siclog@ t_WaitTime.start; alt{ [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_MoData, ?))) { t_WaitTime.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6Ba1"); } [] t_WaitTime.timeout { //@siclog "Step 6Bb2" siclog@ // Make the UE attempt to establish a PDU session for the DNN defined in Table 11.3.9.3.3-1. This can be done by an AT/MMI command. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_DNNValue); //@siclog "Step 6Bb3" siclog@ //Check: Does the UE transmit a RRCSetupRequest message and establishmentCause is set to mo-Data? v_ReceiveRRCSetupRequest := f_NR_RRC_SetupRequest_Common( nr_Cell12, t_Wait, cr_NR_EstablishmentCause_MoData); if (v_ReceiveRRCSetupRequest) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6Bb3"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6Bb3"); } } } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> // @sic R5-214783 sic@ // @siclog "Steps 6A - 6Bb1" siclog@ t_WaitTime.start; alt{ [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_MoData, ?))) { t_WaitTime.stop; f_NR_ULGrantConfiguration_Start(nr_Cell12, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); //WA#11_3_9 f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6Ba1"); } [] t_WaitTime.timeout { //@siclog "Step 6Bb2" siclog@ // Make the UE attempt to establish a PDU session for the DNN defined in Table 11.3.9.3.3-1. This can be done by an AT/MMI command. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_DNNValue); //@siclog "Step 6Bb3" siclog@ //Check: Does the UE transmit a RRCSetupRequest message and establishmentCause is set to mo-Data? v_ReceiveRRCSetupRequest := f_NR_RRC_SetupRequest_Common( nr_Cell12, t_Wait, cr_NR_EstablishmentCause_MoData); if (v_ReceiveRRCSetupRequest) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6Bb3"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6Bb3"); } } } <<SKIPPED CODE>>