3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221055

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4687 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EUTRA RRC HeNB test case 8.3.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2022-08-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation, when the function f_TC_8_3_4_1_EUTRA -->f_EUTRA_CellConfig_SRBs_Only is called, the attenuation of the cell is not OFF. Before the function f_TC_8_3_4_1_EUTRA -->f_EUTRA_CellConfig_SRBs_Only is called, the UE has been asked to manually select CSG ID of cell 2 in function call f_EUTRA_Manual_CSG_Selection. In this function, the attenuation for this cell is set to a value other than tsc_CellAttenuation_Off, and this is stored. This is then retrieved when the following call is made : f_TC_8_3_4_1_EUTRA -->f_EUTRA_CellConfig_SRBs_Only -->f_EUTRA_CellConfig_Def -->f_EUTRA_SS_ConfigureActiveCell --> v_CellInfo := f_EUTRA_CellInfo_Get(p_CellId); Because this cell was set as serving cell power before, v_CellInfo.ReferencePower.Attenuation is already having some value. This v_CellInfo is then passed to cas_CellConfig_Def_REQ : f_EUTRA_SS_CommonCellConfig(p_CellId, cas_CellConfig_Def_REQ(p_CellId, cs_TimingInfo_Now, v_StaticCellInfo, v_SiSchedul, v_CellInfo, v_ContentionResolution_ContainedRlcPdu, v_PUCCH_ConfigDedicated, p_CA_CellInitialConfig, v_Bcch_BRConfig, v_PRACH_Config_v1310, v_EpdcchConfig, v_UplinkGrantCE_Mode, v_CE_Mode)); Within the call to cas_CellConfig_Def_REQ, It can be seen that : InitialCellPower := { MaxReferencePower := p_CellInfo.ReferencePower.MaxReferencePower, Attenuation := p_CellInfo.ReferencePower.Attenuation } The TTCN only expects Attenuation = tsc_CellAttenuation_Off. This causes a runtime error, and needs to be addressed |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to set the Attenuation to Off in initial cell power before calling function f_EUTRA_CellConfig_SRBs_Only |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC will cause a runtime error |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

| Function name | f_TC_8_3_4_1_EUTRA |
| --- | --- |
| Reason for change | In the current TTCN implementation, when the function f_TC_8_3_4_1_EUTRA -->f_EUTRA_CellConfig_SRBs_Only is called, the attenuation of the cell is not OFF. Before the function f_TC_8_3_4_1_EUTRA -->f_EUTRA_CellConfig_SRBs_Only is called, the UE has been asked to manually select CSG ID of cell 2 in function call f_EUTRA_Manual_CSG_Selection. In this function, the attenuation for this cell is set to a value other than tsc_CellAttenuation_Off, and this is stored. This is then retrieved when the following call is made : f_TC_8_3_4_1_EUTRA -->f_EUTRA_CellConfig_SRBs_Only -->f_EUTRA_CellConfig_Def -->f_EUTRA_SS_ConfigureActiveCell --> v_CellInfo := f_EUTRA_CellInfo_Get(p_CellId); Because this cell was set as serving cell power before, v_CellInfo.ReferencePower.Attenuation is already having some value. This v_CellInfo is then passed to cas_CellConfig_Def_REQ : f_EUTRA_SS_CommonCellConfig(p_CellId, cas_CellConfig_Def_REQ(p_CellId, cs_TimingInfo_Now, v_StaticCellInfo, v_SiSchedul, v_CellInfo, v_ContentionResolution_ContainedRlcPdu, v_PUCCH_ConfigDedicated, p_CA_CellInitialConfig, v_Bcch_BRConfig, v_PRACH_Config_v1310, v_EpdcchConfig, v_UplinkGrantCE_Mode, v_CE_Mode)); Within the call to cas_CellConfig_Def_REQ, It can be seen that : InitialCellPower := { MaxReferencePower := p_CellInfo.ReferencePower.MaxReferencePower, Attenuation := p_CellInfo.ReferencePower.Attenuation } The TTCN only expects Attenuation = tsc_CellAttenuation_Off. This causes a runtime error, and needs to be addressed |
| Summary of change | Set the Attenuation to Off in initial cell power before calling function f_EUTRA_CellConfig_SRBs_Only. Please see below. |
| TTCN module | \ LTE\8_3\RRC_Measurement.ttcn |
| MCC160 Comment |  |

Before Change:

/* * @desc REFERENCE TS 36.523-1 clause 8.3.4.1 * @status APPROVED (LTE) */ function f_TC_8_3_4_1_EUTRA() runs on EUTRA_PTC { //Intra-frequency SI acquisition / CSG cell and non-CSG cell var template (value) CellPowerList_Type v_CellPowerList_T0; //@sic R5s140289 sic@ var template (value) CellPowerList_Type v_CellPowerList_T1; var template (value) CellPowerList_Type v_CellPowerList_T2; var template (value) CellPowerList_Type v_CellPowerList_T3; << CODE SKIPPED >> if (pc_Manual_CSG_Selection) { f_EUTRA_CellConfig_Def(eutra_Cell2); //@sic R5s140594 sic@ f_UT_USIM_Insert (UT, "USIM with service n86 available"); // Empty UE Allowed CSG list in USIM f_EUTRA_EmptyACSGL(eutra_Cell2, {v_CSG_Identity_Cell2, v_CSG_Identity_Cell4}); //@sic R5s150991 sic@ // Broadcast correct CSG id f_EUTRA_CellInfo_SetSysInfo_Csg_Id(eutra_Cell2, v_CSG_Identity_Cell2); f_EUTRA_SS_ConfigureSysinfo(eutra_Cell2); // Add CSG2 in the Alllowed CSG list in USIM f_EUTRA_Manual_CSG_Selection( eutra_Cell2, v_CSG_Identity_Cell2); f_EUTRA_SetCellConfigCapability(eutra_Cell2, broadcastOnlyCell); //@sic R5s140594 sic@ f_EUTRA_CellConfig_SRBs_Only(eutra_Cell2); //@sic R5s140594 sic@ f_EUTRA_SetCellPowerList (v_CellPowerList_T0); } else { << CODE SKIPPED >>

After Change:

/* * @desc REFERENCE TS 36.523-1 clause 8.3.4.1 * @status APPROVED (LTE) */ function f_TC_8_3_4_1_EUTRA() runs on EUTRA_PTC { //Intra-frequency SI acquisition / CSG cell and non-CSG cell var template (value) CellPowerList_Type v_CellPowerList_T0; //@sic R5s140289 sic@ var template (value) CellPowerList_Type v_CellPowerList_T1; var template (value) CellPowerList_Type v_CellPowerList_T2; var template (value) CellPowerList_Type v_CellPowerList_T3; … var template (value) EUTRA_CellInfo_Type v_CellInfo; << CODE SKIPPED >> if (pc_Manual_CSG_Selection) { f_EUTRA_CellConfig_Def(eutra_Cell2); //@sic R5s140594 sic@ f_UT_USIM_Insert (UT, "USIM with service n86 available"); // Empty UE Allowed CSG list in USIM f_EUTRA_EmptyACSGL(eutra_Cell2, {v_CSG_Identity_Cell2, v_CSG_Identity_Cell4}); //@sic R5s150991 sic@ // Broadcast correct CSG id f_EUTRA_CellInfo_SetSysInfo_Csg_Id(eutra_Cell2, v_CSG_Identity_Cell2); f_EUTRA_SS_ConfigureSysinfo(eutra_Cell2); // Add CSG2 in the Alllowed CSG list in USIM f_EUTRA_Manual_CSG_Selection( eutra_Cell2, v_CSG_Identity_Cell2); v_CellInfo := f_EUTRA_CellInfo_Get(eutra_Cell2); v_CellInfo.ReferencePower.Attenuation.Off := true; f_EUTRA_CellInfo_Set(eutra_Cell2, v_CellInfo); f_EUTRA_SetCellConfigCapability(eutra_Cell2, broadcastOnlyCell); //@sic R5s140594 sic@ f_EUTRA_CellConfig_SRBs_Only(eutra_Cell2); //@sic R5s140594 sic@ f_EUTRA_SetCellPowerList (v_CellPowerList_T0); } else { << CODE SKIPPED >>