3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221113

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2762 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_ReleaseCallWithOptionalDeregisterOrRelease |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | While running testcase 11.4.9, fl_TC_11_4_9_NR5GC_TestBody fl_ReleaseCallWithOptionalDeregisterOrRelease f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2], cms_IMS_IPCAN_Trigger("Trigger MO Call Release")); is called, and after this the NR5GC PTC waits for a coordination message from IMS PTC. On the IMS PTC side, f_TC_11_4_9_NR5GC_IMS2 f_IMS_EmergencyCallAndReleaseWithoutRegistration f_IMS_EmergencyCallRelease is called once the coordination message from NR5GC PTC to release call is received. This function then calls f_IMS_EmergencyCallRelease f_IMS_CallReleaseMO a_IMS_ReceiveByeSendOK. Thus, until the UE sends a BYE, and the SS has sent a 200OK followed by TCP close, the call f_TC_11_4_9_NR5GC_IMS2 f_IMS_EmergencyCallAndReleaseWithoutRegistration f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("IMS Call Release Msgs Sent")); will not happen. The problem here is that as soon as the UE sends a BYE, it will also send a PDU Session Release REQ which needs to be handled in the NR5GC PTC. But at this stage, the NR5GC PTC is waiting for a coordination message "IMS Call Release Msgs Sent". This results in the PDU Session Release REQ not being handled which causes testacse failure. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Moved the reception of coordination message IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger ("IMS Call Release Msgs Sent")); to the same level as reception of PDU Session Release REQ in function fl_ReleaseCallWithOptionalDeregisterOrRelease. Please see below for detailed change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.9.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to the function fl_ReleaseCallWithOptionalDeregisterOrRelease	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the function fl_ReleaseCallWithOptionalDeregisterOrRelease

| Template name | fl_ReleaseCallWithOptionalDeregisterOrRelease |
| --- | --- |
| Reason for change | While running testcase 11.4.9, fl_TC_11_4_9_NR5GC_TestBody fl_ReleaseCallWithOptionalDeregisterOrRelease f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2], cms_IMS_IPCAN_Trigger("Trigger MO Call Release")); is called, and after this the NR5GC PTC waits for a coordination message from IMS PTC. On the IMS PTC side, f_TC_11_4_9_NR5GC_IMS2 f_IMS_EmergencyCallAndReleaseWithoutRegistration f_IMS_EmergencyCallRelease is called once the coordination message from NR5GC PTC to release call is received. This function then calls f_IMS_EmergencyCallRelease f_IMS_CallReleaseMO a_IMS_ReceiveByeSendOK. Thus, until the UE sends a BYE, and the SS has sent a 200OK followed by TCP close, the call f_TC_11_4_9_NR5GC_IMS2 f_IMS_EmergencyCallAndReleaseWithoutRegistration f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("IMS Call Release Msgs Sent")); will not happen. The problem here is that as soon as the UE sends a BYE, it will also send a PDU Session Release REQ which needs to be handled in the NR5GC PTC. But at this stage, the NR5GC PTC is waiting for a coordination message "IMS Call Release Msgs Sent". This results in the PDU Session Release REQ not being handled which causes testacse failure. This needs to be addressed. |
| Summary of change | Moved the reception of coordination message IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger ("IMS Call Release Msgs Sent")); to the same level as reception of PDU Session Release REQ in function fl_ReleaseCallWithOptionalDeregisterOrRelease. Please see below for detailed change. |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_ReleaseCallWithOptionalDeregisterOrRelease (NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); var O1_Type v_PDUSessionId := int2oct(f_Get_PDUSessionIdForDNNType(v_PDUSessionInfoList, Emergency_PDN), 1); var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_UL_Pdu_Type v_RxdMsg; var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; var boolean v_WaitForReleaseComplete := false; timer t_Wait := 5.0; // @sic R5-223439 sic@ //@siclog "Step 5" siclog@ //Make the UE release the emergency call. // @sic R5-223439 sic@ //@siclog "Step 6A-6C" siclog@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2], cms_IMS_IPCAN_Trigger("Trigger MO Call Release")); // @sic R5-225362 sic@ IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger ("IMS Call Release Msgs Sent")); // @sic R5-225362 sic@ sic@ //@siclog "Step 7" siclog@ t_Wait.start; // @sic R5s220560 sic@ //EXCEPTION: In parallel to step 8 below the steps specified in the parallel behaviours may take place alt { [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { v_RxdMsg := v_ReceivedAsp.Signalling.Nas[0].Pdu; // The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration". if (ischosen(v_RxdMsg.Msg.deregistration_RequestMO)) { if (not f_Check_NG_DeregistrationReqMOMsg (v_RxdMsg.Msg, crs_DeregisterType('0'B, '0'B,'01'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request failed"); } //The SS replies with a DEREGISTRATION ACCEPT message. SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB2, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT))); } // @sic R5-223439 sic@ // The UE transmits a PDU SESSION RELEASE REQUEST message else if (ischosen(v_RxdMsg.PiggybackedPduList[0].Msg.pdu_Session_Release_Request)) { if (not f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo, v_PDUSessionInfoList, v_RxdMsg, v_PDUSessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request failed"); } // set flag to expect complete v_WaitForReleaseComplete := true; // Complete the release procedure v_MsgToSend := f_Get_NG_PDUSessionReleaseCommand(v_PDUSessionId, cs_GMM_GSM_Cause (omit, '00011010'B)); SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB2, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); } // The UE transmits a PDU SESSION RELEASE COMPLETE message? else if (v_WaitForReleaseComplete and ischosen(v_RxdMsg.PiggybackedPduList[0].Msg.pdu_Session_Release_Complete)) { if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfoList, v_RxdMsg, v_PDUSessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Complete failed"); } v_WaitForReleaseComplete := false; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Wrong NAS Message Received"); } // Keep going round until timer expires @sic R5-223439 sic@ repeat; } //@siclog "Step 8" siclog@ [] t_Wait.timeout //@sic R5-216198 sic@ { // if Release Complete hasn't arrived before timeout, then need to wait for it so it doesn't cause an error when it does arrive if (v_WaitForReleaseComplete) { SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfoList, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Complete failed"); } } } } }

After Change:

function fl_ReleaseCallWithOptionalDeregisterOrRelease (NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); var O1_Type v_PDUSessionId := int2oct(f_Get_PDUSessionIdForDNNType(v_PDUSessionInfoList, Emergency_PDN), 1); var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_UL_Pdu_Type v_RxdMsg; var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; var boolean v_WaitForReleaseComplete := false; timer t_Wait := 5.0; // @sic R5-223439 sic@ //@siclog "Step 5" siclog@ //Make the UE release the emergency call. // @sic R5-223439 sic@ //@siclog "Step 6A-6C" siclog@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2], cms_IMS_IPCAN_Trigger("Trigger MO Call Release")); // @sic R5-225362 sic@ //WA#11_4_9 MOVED IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger ("IMS Call Release Msgs Sent")); // @sic R5-225362 sic@ sic@ //@siclog "Step 7" siclog@ t_Wait.start; // @sic R5s220560 sic@ //EXCEPTION: In parallel to step 8 below the steps specified in the parallel behaviours may take place alt { [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { v_RxdMsg := v_ReceivedAsp.Signalling.Nas[0].Pdu; // The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration". if (ischosen(v_RxdMsg.Msg.deregistration_RequestMO)) { if (not f_Check_NG_DeregistrationReqMOMsg (v_RxdMsg.Msg, crs_DeregisterType('0'B, '0'B,'01'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request failed"); } //The SS replies with a DEREGISTRATION ACCEPT message. SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB2, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT))); } // @sic R5-223439 sic@ // The UE transmits a PDU SESSION RELEASE REQUEST message else if (ischosen(v_RxdMsg.PiggybackedPduList[0].Msg.pdu_Session_Release_Request)) { if (not f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo, v_PDUSessionInfoList, v_RxdMsg, v_PDUSessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request failed"); } // set flag to expect complete v_WaitForReleaseComplete := true; // Complete the release procedure v_MsgToSend := f_Get_NG_PDUSessionReleaseCommand(v_PDUSessionId, cs_GMM_GSM_Cause (omit, '00011010'B)); SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB2, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); } // The UE transmits a PDU SESSION RELEASE COMPLETE message? else if (v_WaitForReleaseComplete and ischosen(v_RxdMsg.PiggybackedPduList[0].Msg.pdu_Session_Release_Complete)) { if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfoList, v_RxdMsg, v_PDUSessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Complete failed"); } v_WaitForReleaseComplete := false; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Wrong NAS Message Received"); } // Keep going round until timer expires @sic R5-223439 sic@ repeat; } [] IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger ("IMS Call Release Msgs Sent")) // @sic R5-225362 sic@ sic@ //WA#11_4_9 {// Keep going round until timer expires @sic R5-223439 sic@ repeat; } //@siclog "Step 8" siclog@ [] t_Wait.timeout //@sic R5-216198 sic@ { // if Release Complete hasn't arrived before timeout, then need to wait for it so it doesn't cause an error when it does arrive if (v_WaitForReleaseComplete) { SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfoList, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Complete failed"); } } } } }