3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221124

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2772 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for EN-DC RRC measurement test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According e.g. table Table 8.2.5.1.2.3.3-2, measResultFreqList should be “not checked”. Current implementation uses the default value “{cr_MeasResultFreqFailNR} ifpresent” which has expected values of “rsrpResult_r15” and “rsrqResult_r15” set to “0”. Pass “*” to cr_FailureReportSCG_NR_r15 for p_MeasResultFreqFail |  |  |  |  |  |  |  |  |
| Summary of change: |  | Pass “*” to cr_FailureReportSCG_NR_r15 for p_MeasResultFreqFail |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.5.1.1.ENDC, 8.2.5.2.1.ENDC, 8.2.5.3.1.ENDC, 8.2.5.4.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | e.g. fl_TC_8_2_5_1_1_ENDC_EUTRA_TestBody() |
| --- | --- |
| Reason for change | According e.g. table Table 8.2.5.1.2.3.3-2, measResultFreqList should be “not checked”. Current implementation uses the default value “{cr_MeasResultFreqFailNR} ifpresent” which has expected values of “rsrpResult_r15” and “rsrqResult_r15” set to “0”. |
| Summary of change | Pass “*” to cr_FailureReportSCG_NR_r15 for p_MeasResultFreqFail Note : Same correction needed for 8.2.5.2.1.ENDC, 8.2.5.3.1.ENDC, 8.2.5.4.1.ENDC |
| TTCN module | EUTRA_RRC_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_2_5_1_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { // @sic R5-187795 sic@ timer t_Timer := 5.0; f_EUTRA_NR_WaitForCoOrd_Trigger(NR); t_Timer.start; alt{ // @siclog "Step 4" siclog@ // Check: Does the UE transmit in the next 5 sec (arbitrary value) a SCGFailureInformationNR message with failureType set to 'randomAccessProblem'? [] SRB.receive (car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_SCGFailureInformationNR_r15(cr_FailureReportSCG_NR_r15(randomAccessProblem)))) {// verdict pass the UE transmit SCGFailureInformationNR message in the next 5 sec f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); t_Timer.stop; } [] t_Timer.timeout {// verdict fail the UE doesn't transmit SCGFailureInformationNR message in the next 5 sec f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } } }

After Change:

function fl_TC_8_2_5_1_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { // @sic R5-187795 sic@ timer t_Timer := 5.0; f_EUTRA_NR_WaitForCoOrd_Trigger(NR); t_Timer.start; alt{ // @siclog "Step 4" siclog@ // Check: Does the UE transmit in the next 5 sec (arbitrary value) a SCGFailureInformationNR message with failureType set to 'randomAccessProblem'? [] SRB.receive (car_SRB1_RrcPdu_IND(eutra_Cell1, cr_508_SCGFailureInformationNR_r15(cr_FailureReportSCG_NR_r15(randomAccessProblem, *)))) {// verdict pass the UE transmit SCGFailureInformationNR message in the next 5 sec f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); t_Timer.stop; } [] t_Timer.timeout {// verdict fail the UE doesn't transmit SCGFailureInformationNR message in the next 5 sec f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } } }