3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221131

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2775 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the function f_NR_SS_ConfigureSysinfo |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN Implementation of the function f_NR_SS_ConfigureSysinfo() v_CellSysInfo.BcchInfo.Sis is assigned to v_Sis, if the length of v_SI_SchedulingInfo.schedulingInfoList > 0 But this condition will not be sufficient for the test cases where the segmented Sis are present ( PWS notification scenarios ). This could result in a run time issue where there is no presence of v_CellSysInfo.BcchInfo.Sis in case of PWS notification scenarios |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add the condition ispresent(v_CellSysInfo.BcchInfo.SIs[i]) before assigning v_SIs[j] := v_CellSysInfo.BcchInfo.SIs[i]; and also before assigning v_CellSysInfo.BcchInfo.SIs := v_SIs; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.3.1.NR5GC, 8.1.5.3.2.NR5GC, 8.1.5.3.3.NR5GC and 8.1.5.3.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to the function f_NR_SS_ConfigureSysinfo	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the function f_NR_SS_ConfigureSysinfo

| Template name | f_NR_SS_ConfigureSysinfo |
| --- | --- |
| Reason for change | In the current TTCN Implementation of the function f_NR_SS_ConfigureSysinfo() v_CellSysInfo.BcchInfo.Sis is assigned to v_Sis if the length of v_SI_SchedulingInfo.schedulingInfoList > 0 But this condition will not be sufficient for the test cases where the segmented Sis are present ( PWS notificaiton scenarios ). This could result in run time issue where there is no presence of v_CellSysInfo.BcchInfo.Sis in case of PWS notification scenarios |
| Summary of change | It is proposed to add the condition ispresent(v_CellSysInfo.BcchInfo.SIs[i]) before assigning v_SIs[j] := v_CellSysInfo.BcchInfo.SIs[i]; and also before assigning v_CellSysInfo.BcchInfo.SIs := v_SIs; |
| TTCN module | NR5GC_SysinfoPaging.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_SS_ConfigureSysinfo(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template(value) NR_BcchToPdschConfig_Type p_NR_BcchToPdschConfig, boolean p_BroadcastAllSI := false ) runs on NR_BASE_PTC { //@sic R5s220880 : added p_BroadcastAllSI sic@ var NR_CellSysInfo_Type v_CellSysInfo := f_NR_CellInfo_GetSysinfo(p_NR_CellId); var SI_SchedulingInfo v_SI_SchedulingInfo; var NR_SI_List_Type v_SIs; var integer i := 0; var integer j := 0; if (not(p_BroadcastAllSI) and v_CellSysInfo.NR_SysinfoCombination != MibOnly and v_CellSysInfo.NR_SysinfoCombination != NR_1 ) { //@sic R5s220880 sic@ v_SI_SchedulingInfo := f_NR_GetSIB1_SiSchedulingInfo(p_NR_CellId); for (i := 0; i < lengthof(v_SI_SchedulingInfo.schedulingInfoList); i:=i+1) { if ( v_SI_SchedulingInfo.schedulingInfoList[i].si_BroadcastStatus == broadcasting) { v_SIs[j] := v_CellSysInfo.BcchInfo.SIs[i]; j := j+1; } } v_CellSysInfo.BcchInfo.SIs := v_SIs; } f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_ModifySysinfo_Def_REQ(p_NR_CellId, p_TimingInfo, v_CellSysInfo.BcchInfo, /* system information to be scheduled */ p_NR_BcchToPdschConfig)); /* scheduling of SIs */ }

After Change:

function f_NR_SS_ConfigureSysinfo(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template(value) NR_BcchToPdschConfig_Type p_NR_BcchToPdschConfig, boolean p_BroadcastAllSI := false ) runs on NR_BASE_PTC { //@sic R5s220880 : added p_BroadcastAllSI sic@ var NR_CellSysInfo_Type v_CellSysInfo := f_NR_CellInfo_GetSysinfo(p_NR_CellId); var SI_SchedulingInfo v_SI_SchedulingInfo; var NR_SI_List_Type v_SIs; var integer i := 0; var integer j := 0; if (not(p_BroadcastAllSI) and v_CellSysInfo.NR_SysinfoCombination != MibOnly and v_CellSysInfo.NR_SysinfoCombination != NR_1 ) { //@sic R5s220880 sic@ v_SI_SchedulingInfo := f_NR_GetSIB1_SiSchedulingInfo(p_NR_CellId); for (i := 0; i < lengthof(v_SI_SchedulingInfo.schedulingInfoList); i:=i+1) { if ( (v_SI_SchedulingInfo.schedulingInfoList[i].si_BroadcastStatus == broadcasting) and (ispresent(v_CellSysInfo.BcchInfo.SIs[i])) ) { //WA#WI_1006998 v_SIs[j] := v_CellSysInfo.BcchInfo.SIs[i]; j := j+1; } } if(ispresent(v_CellSysInfo.BcchInfo.SIs[i])) //WA#WI_1006998 { v_CellSysInfo.BcchInfo.SIs := v_SIs; } } f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_ModifySysinfo_Def_REQ(p_NR_CellId, p_TimingInfo, v_CellSysInfo.BcchInfo, /* system information to be scheduled */ p_NR_BcchToPdschConfig)); /* scheduling of SIs */ }