3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221132

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2776 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the NR test cases 8.1.5.8.2.1, 8.1.5.8.2.2, 8.1.5.8.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the steps 1-3 , when sending RRC Connection Reconfiguration to configure CA , it is proposed to add Mac Bearer Routing to be in alignment with the other working CA test cases ( referring the implementation from f_NR5GC_AddMod_1SCell ) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Mac bearer routing is added while sending the Peer reconfiguration Message |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.8.2.1.NR5GC, 8.1.5.8.2.2.NR5GC, 8.1.5.8.2.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to the function f_TC_8_1_5_8_2_x_NR5GC_TestBody	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the function f_TC_8_1_5_8_2_x_NR5GC_TestBody

| Template name | f_TC_8_1_1_2_3_NR5GC |
| --- | --- |
| Reason for change | In the steps 1-3 , when sending RRC Connection Reconfiguration to configure CA , it is proposed to add Mac Bearer Routing to be in alignment with the other working CA test cases ( referring the implementation from f_NR5GC_AddMod_1SCell ) |
| Summary of change | Mac bearer routing is added while sending the Peer reconfiguration Message |
| TTCN module | RRC_Others_CA_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_5_8_2_x_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { var template(value) CellGroupConfig v_SCGConfigCA := f_NR_GetSCGConfig_SCellAdd(p_SCell, tsc_SCellIndex_1); var template(value) CellGroupConfig v_SCGConfig; var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfigCA))); var template(value) RadioBearerConfig v_RadioBearerConfig; var template(omit) DRB_ToAddModList v_DRB_ToAddModList := omit; var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(p_PCell); var integer i; var SubFrameTiming_Type v_TimingInfo; //@sic R5s210842 change 1 sic@ var SubFrameTiming_Type v_TimingInfo2; //@sic R5s210842 change 1 sic@ var template(value) NR_RadioBearerList_Type v_SS_Config := f_NR5GC_GetRadioBearerList(v_ExistingDRBs); //@sic R5s220835 sic@ // @siclog "Steps 1-3" siclog@ // The SS transmits an RRCReconfiguration message with sCellToAddModList to add SCell(Cell 3) // The SS starts transmitting a periodic DCI from (16*2microsec+1+TDL) slots after step 1 to schedule PUSCH. // Check: Does the UE transmit an RRCReconfigurationComplete message within (16*2microsec+1+TDL+K2) slots after successful completion of step 1? (Note 1, Note 2, Note 3) //@sic R5s210842 change 1 sic@ v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell,500); v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); // 5 ms later f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell)); f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_NR(v_TimingInfo2)); //@sic R5s220365 sic@ f_NR5GC_RRC_ProcedureDelay(p_PCell, cas_NR_SRB1_RrcPdu_REQ(p_PCell, cs_TimingInfo_NR(v_TimingInfo), cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, -, -, v_V1530Ext)), car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete, - , crs_MacBearerRouting(p_PCell)), 16, //converted in slots inside the function "Step 3"); f_NR_ULGrantConfiguration_Start(p_PCell); //@sic R5s220365 sic@ << SKIPPED CODE >> }

After Change:

function f_TC_8_1_5_8_2_x_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { var template(value) CellGroupConfig v_SCGConfigCA := f_NR_GetSCGConfig_SCellAdd(p_SCell, tsc_SCellIndex_1); var template(value) CellGroupConfig v_SCGConfig; var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfigCA))); var template(value) RadioBearerConfig v_RadioBearerConfig; var template(omit) DRB_ToAddModList v_DRB_ToAddModList := omit; var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(p_PCell); var integer i; var SubFrameTiming_Type v_TimingInfo; //@sic R5s210842 change 1 sic@ var SubFrameTiming_Type v_TimingInfo2; //@sic R5s210842 change 1 sic@ var template(value) NR_RadioBearerList_Type v_SS_Config := f_NR5GC_GetRadioBearerList(v_ExistingDRBs); //@sic R5s220835 sic@ // @siclog "Steps 1-3" siclog@ // The SS transmits an RRCReconfiguration message with sCellToAddModList to add SCell(Cell 3) // The SS starts transmitting a periodic DCI from (16*2microsec+1+TDL) slots after step 1 to schedule PUSCH. // Check: Does the UE transmit an RRCReconfigurationComplete message within (16*2microsec+1+TDL+K2) slots after successful completion of step 1? (Note 1, Note 2, Note 3) //@sic R5s210842 change 1 sic@ v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell,500); v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); // 5 ms later f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell)); f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_NR(v_TimingInfo2)); //@sic R5s220365 sic@ f_NR5GC_RRC_ProcedureDelay(p_PCell, cas_NR_SRB1_RrcPdu_REQ(p_PCell, cs_TimingInfo_NR(v_TimingInfo), cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, -, -, v_V1530Ext), crs_MacBearerRouting(p_PCell)), //WA#WI=1008349 car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete, - , crs_MacBearerRouting(p_PCell)), 16, //converted in slots inside the function "Step 3"); f_NR_ULGrantConfiguration_Start(p_PCell); //@sic R5s220365 sic@ << SKIPPED CODE >> }