3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221229

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2830 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 5GSM test case 10.1.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Since the additional PDU session triggered at step 1 without any DNN name, the TTCN functions may not be able to determine the PDN type and so will treat it according to the setting of PICS pc_APN_Default_Configuration. If this is set to “ims” and the UE has already created an IMS PDU session during the preamble, then the UE may reject the PDU SESSION ESTABLISHMENT ACCEPT due to some invalid parameters, e.g. mapped EPS bearer context id already in use. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Pass the parameter p_DNNType from f_NR5GC_TriggerSinglePDUSession() through to f_NR5GC_PDUSessionEstablishment_InnerLoop () so that the PDU type can be fixed to “internet” when no DNN is received from the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.3.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	8

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NR5GC_TriggerSinglePDUSession() |
| --- | --- |
| Reason for change | Since the additional PDU session triggered at step 1 without any DNN name, the TTCN functions may not be able to determine the PDN type and so will treat it according to the setting of PICS pc_APN_Default_Configuration. If this is set to “ims” and the UE has already created an IMS PDU session during the preamble, then the UE may reject the PDU SESSION ESTABLISHMENT ACCEPT due to some invalid parameters, e.g. mapped EPS bearer context id already in use. |
| Summary of change | Pass the parameter p_DNNType through to f_NR5GC_PDUSessionEstablishment() so that the PDU type can be fixed to “internet” when no DNN is received from the UE. |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_TriggerSinglePDUSession(NR_CellId_Type p_NR_CellId, integer p_PDUSessionId, PDU_PDN_DNN_Type p_DNNType := Internet_DNN, template (omit) charstring p_DNN := "", template (omit) S_NSSAI_Type p_S_NSSAI_InATCmd := omit, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI) runs on NR5GC_PTC { var charstring v_S_NSSAI_ForATCmd := ""; var charstring v_DNN_ForATCmd := ""; var template (omit) DNN v_DNNRx := omit; var template (value) DNN v_DNNTx; var template (omit) QoS_Rules v_QoS_Rules; var charstring v_QoSFlowToBuild; …………………………… f_UT_RequestPDUSession(UT, p_PDUSessionId, v_DNN_ForATCmd, -, -, v_S_NSSAI_ForATCmd); //AT command to trigger a PDU session f_NR5GC_PDUSessionEstablishment (p_NR_CellId, 1, // Only triggering this one session -, -, p_ExpectedS_NSSAI, // Value in UL Req message v_DNNRx, // Value in UL Req message -, v_DNNTx, // Value in DL Accept message v_QoS_Rules, // Value determined by PICS -, p_S_NSSAI, // Value in DL Accept message -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_QoSFlowToBuild); // Value determined by PICS }

After Change:

function f_NR5GC_TriggerSinglePDUSession(NR_CellId_Type p_NR_CellId, integer p_PDUSessionId, PDU_PDN_DNN_Type p_DNNType := Internet_DNN, template (omit) charstring p_DNN := "", template (omit) S_NSSAI_Type p_S_NSSAI_InATCmd := omit, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI) runs on NR5GC_PTC { var charstring v_S_NSSAI_ForATCmd := ""; var charstring v_DNN_ForATCmd := ""; var template (omit) DNN v_DNNRx := omit; var template (value) DNN v_DNNTx; var template (omit) QoS_Rules v_QoS_Rules; var charstring v_QoSFlowToBuild; …………………….. f_UT_RequestPDUSession(UT, p_PDUSessionId, v_DNN_ForATCmd, -, -, v_S_NSSAI_ForATCmd); //AT command to trigger a PDU session f_NR5GC_PDUSessionEstablishment (p_NR_CellId, 1, // Only triggering this one session -, -, p_ExpectedS_NSSAI, // Value in UL Req message v_DNNRx, // Value in UL Req message -, v_DNNTx, // Value in DL Accept message v_QoS_Rules, // Value determined by PICS -, p_S_NSSAI, // Value in DL Accept message -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_QoSFlowToBuild, -,-,-,-,p_DNNType); //KS#R5s22xxxx // Value determined by PICS }

## Change 2

| Function name | f_NR5GC_PDUSessionEstablishment() |
| --- | --- |
| Reason for change | Since the additional PDU session triggered at step 1 without any DNN name, the TTCN functions may not be able to determine the PDN type and so will treat it according to the setting of PICS pc_APN_Default_Configuration. If this is set to “ims” and the UE has already created an IMS PDU session during the preamble, then the UE may reject the PDU SESSION ESTABLISHMENT ACCEPT due to some invalid parameters, e.g. mapped EPS bearer context id already in use. |
| Summary of change | New paramter p_DNNType, passed through to f_NR5GC_PDUSessionEstablishment_InnerLoop() so that the PDU type can be fixed to “internet” when no DNN is received from the UE. |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_PDUSessionEstablishment(NR_CellId_Type p_NR_CellId, integer p_ExpectedNumberOfNewPDUSessions := pc_noOf_PDUsSameConnection, // @sic R5s201358 sic@, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2, template (omit) SDAP_Config p_SDAP_Config := omit, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) O1_Type p_PDU_SessionId := ?, template ProcedureTransactionIdentifier p_PTI := omit, template (omit) PDU_SessionType p_PDU_SessionType := omit, template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@ template NG_UE_SM_Cap p_UECap := *, template MaxNumPacketFilters p_MaxNumPacketFilters := *, template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?, template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *, template SM_PDU_DN_RequestContainer p_PDUReq := *, template ExtdProtocolConfigOptions p_ExtdPCO := *, template NG_PDU_SessionId p_OldPDU_SessionId := *, template NG_Request_Type p_RequestType := cr_NG_Request_Type('001'B), template AdditionalInformation p_AdditionalInfo := *, template (omit) GMM_GSM_Cause p_Cause := omit, template (omit) GPRS_Timer p_RQTimer := omit, template (omit) EAP_Message p_EAP := omit, template (omit) GPRS_Timer3 p_BackOff := omit, boolean p_PDUSessionAuthenticationRequired := false, template (omit) charstring p_TriggerWithThisAPN := omit, boolean p_FailWhenK_LessN := true, template (omit) charstring p_BuildQosFlow := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromLTE := omit, // @sic R5-198184, R5-199075 sic@ boolean p_NormalIMSOperation := true, // @sic R5s210515 sic@ integer p_EstReqAlreadyInDefault := 0, boolean p_OmitDNN := false) runs on NR5GC_PTC // @sic R5-223987 sic@ { var SRBs_AlreadyConfigured v_SRBs := SRB1_2; var NR_RadioBearerId_Type v_SRB := p_SRB; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_K_NumOfReqsProcessed := 0; var integer v_L_NumOfReqsReceivedInParallel := 0; var boolean v_EndLoop := false; var boolean v_RxdMsg; var boolean v_IMS_Configured := false; var PDU_PDN_DNN_Type v_PDUType; var DelayForUserPlaneSignalling_Type v_WaitForUserplaneSignalling := noDelay; // Default: no IP signalling possible var default v_MyDefaultVar := null; ……………………… v_PDUType := f_NR5GC_PDUSessionEstablishment_InnerLoop(p_NR_CellId, v_ReceivedAsp, v_SRBs, p_SDAP_Config, p_ExpectedS_NSSAI, p_ExpectedDNN, p_SSC_Mode, p_DNN, p_QoS_Rules, p_Session_AMBR, p_S_NSSAI, p_QoSFlowDescr, p_PDU_SessionId, p_PTI, p_PDU_SessionType, p_ExpectedSSC_Mode, p_UECap, p_MaxNumPacketFilters, p_IntegrityProtMaxDataRate, p_AlwaysOnPDUSessionReq, p_PDUReq, p_ExtdPCO, p_OldPDU_SessionId, p_RequestType, p_AdditionalInfo, p_Cause, p_RQTimer, p_EAP, p_BackOff, p_PDUSessionAuthenticationRequired, p_BuildQosFlow, p_PduInfoFromLTE, // @sic R5-198184 sic@ p_OmitDNN); // @sic R5-223987 sic@ ………………………….. }

After Change:

function f_NR5GC_PDUSessionEstablishment(NR_CellId_Type p_NR_CellId, integer p_ExpectedNumberOfNewPDUSessions := pc_noOf_PDUsSameConnection, // @sic R5s201358 sic@, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2, template (omit) SDAP_Config p_SDAP_Config := omit, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) O1_Type p_PDU_SessionId := ?, template ProcedureTransactionIdentifier p_PTI := omit, template (omit) PDU_SessionType p_PDU_SessionType := omit, template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@ template NG_UE_SM_Cap p_UECap := *, template MaxNumPacketFilters p_MaxNumPacketFilters := *, template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?, template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *, template SM_PDU_DN_RequestContainer p_PDUReq := *, template ExtdProtocolConfigOptions p_ExtdPCO := *, template NG_PDU_SessionId p_OldPDU_SessionId := *, template NG_Request_Type p_RequestType := cr_NG_Request_Type('001'B), template AdditionalInformation p_AdditionalInfo := *, template (omit) GMM_GSM_Cause p_Cause := omit, template (omit) GPRS_Timer p_RQTimer := omit, template (omit) EAP_Message p_EAP := omit, template (omit) GPRS_Timer3 p_BackOff := omit, boolean p_PDUSessionAuthenticationRequired := false, template (omit) charstring p_TriggerWithThisAPN := omit, boolean p_FailWhenK_LessN := true, template (omit) charstring p_BuildQosFlow := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromLTE := omit, // @sic R5-198184, R5-199075 sic@ boolean p_NormalIMSOperation := true, // @sic R5s210515 sic@ integer p_EstReqAlreadyInDefault := 0, boolean p_OmitDNN := false, template (omit) PDU_PDN_DNN_Type p_DNNType := omit) runs on NR5GC_PTC // @sic R5-223987 sic@ { var SRBs_AlreadyConfigured v_SRBs := SRB1_2; var NR_RadioBearerId_Type v_SRB := p_SRB; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_K_NumOfReqsProcessed := 0; var integer v_L_NumOfReqsReceivedInParallel := 0; var boolean v_EndLoop := false; var boolean v_RxdMsg; var boolean v_IMS_Configured := false; var PDU_PDN_DNN_Type v_PDUType; var DelayForUserPlaneSignalling_Type v_WaitForUserplaneSignalling := noDelay; // Default: no IP signalling possible var default v_MyDefaultVar := null; ……………………………….. v_PDUType := f_NR5GC_PDUSessionEstablishment_InnerLoop(p_NR_CellId, v_ReceivedAsp, v_SRBs, p_SDAP_Config, p_ExpectedS_NSSAI, p_ExpectedDNN, p_SSC_Mode, p_DNN, p_QoS_Rules, p_Session_AMBR, p_S_NSSAI, p_QoSFlowDescr, p_PDU_SessionId, p_PTI, p_PDU_SessionType, p_ExpectedSSC_Mode, p_UECap, p_MaxNumPacketFilters, p_IntegrityProtMaxDataRate, p_AlwaysOnPDUSessionReq, p_PDUReq, p_ExtdPCO, p_OldPDU_SessionId, p_RequestType, p_AdditionalInfo, p_Cause, p_RQTimer, p_EAP, p_BackOff, p_PDUSessionAuthenticationRequired, p_BuildQosFlow, p_PduInfoFromLTE, // @sic R5-198184 sic@ p_OmitDNN, p_DNNType); // @sic R5-223987 sic@ ……………………………….. }

## Change 3

| Function name | f_NR5GC_PDUSessionEstablishment_InnerLoop() |
| --- | --- |
| Reason for change | Since the additional PDU session triggered at step 1 without any DNN name, the TTCN functions may not be able to determine the PDN type and so will treat it according to the setting of PICS pc_APN_Default_Configuration. If this is set to “ims” and the UE has already created an IMS PDU session during the preamble, then the UE may reject the PDU SESSION ESTABLISHMENT ACCEPT due to some invalid parameters, e.g. mapped EPS bearer context id already in use. |
| Summary of change | New paramter p_DNNType, so that the PDU type can be fixed to “internet” when no DNN is received from the UE. |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_PDUSessionEstablishment_InnerLoop(NR_CellId_Type p_NR_CellId, NR_SRB_COMMON_IND p_ReceivedAsp, SRBs_AlreadyConfigured p_SRB := SRB1_2, template (omit) SDAP_Config p_SDAP_Config, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) O1_Type p_PDU_SessionId := ?, template ProcedureTransactionIdentifier p_PTI := omit, template (omit) PDU_SessionType p_PDU_SessionType := omit, template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@ template NG_UE_SM_Cap p_UECap := *, template MaxNumPacketFilters p_MaxNumPacketFilters := *, template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?, template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *, template SM_PDU_DN_RequestContainer p_PDUReq := *, template ExtdProtocolConfigOptions p_ExtdPCO := *, template NG_PDU_SessionId p_OldPDU_SessionId := *, template NG_Request_Type p_RequestType := ?, template AdditionalInformation p_AdditionalInfo := *, template (omit) GMM_GSM_Cause p_Cause := omit, template (omit) GPRS_Timer p_RQTimer := omit, template (omit) EAP_Message p_EAP := omit, template (omit) GPRS_Timer3 p_BackOff := omit, boolean p_PDUSessionAuthenticationRequired := false, template (omit) charstring p_BuildQosFlow := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromLTE := omit, boolean p_OmitDNN := false // @sic R5-223987 sic@ ) runs on NR5GC_PTC return PDU_PDN_DNN_Type // @sic R5-198184, R5-206296 sic@ { // Split function so list of DRBs can be built elsewhere @sic R5-199103 sic@ // Moved UT trigger up to f_NR5GC_PDUSessionEstablishment @sic R5s200751 sic@ var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var GMM_MobilityInfo_Type v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); // @sic R5-195328 sic@ var GSM_MobilityInfo_Type v_GSM_MobilityInfo; …………………… f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } if (match(p_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.requestType.requestValue, '011'B) or match(p_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.requestType.requestValue, '100'B)) { // @sic R5-202550, R5s210518, R5s20883 sic@ // this is the only way to flag it's an emergency PDU v_GSM_MobilityInfo.PDUType := Emergency_PDN; // @sic R5-206296 sic@ v_MediaType := IPCAN_EmergencyCall; // @sic R5s211486 sic@ } …………………………………. }

After Change:

function f_NR5GC_PDUSessionEstablishment_InnerLoop(NR_CellId_Type p_NR_CellId, NR_SRB_COMMON_IND p_ReceivedAsp, SRBs_AlreadyConfigured p_SRB := SRB1_2, template (omit) SDAP_Config p_SDAP_Config, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) O1_Type p_PDU_SessionId := ?, template ProcedureTransactionIdentifier p_PTI := omit, template (omit) PDU_SessionType p_PDU_SessionType := omit, template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@ template NG_UE_SM_Cap p_UECap := *, template MaxNumPacketFilters p_MaxNumPacketFilters := *, template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?, template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *, template SM_PDU_DN_RequestContainer p_PDUReq := *, template ExtdProtocolConfigOptions p_ExtdPCO := *, template NG_PDU_SessionId p_OldPDU_SessionId := *, template NG_Request_Type p_RequestType := ?, template AdditionalInformation p_AdditionalInfo := *, template (omit) GMM_GSM_Cause p_Cause := omit, template (omit) GPRS_Timer p_RQTimer := omit, template (omit) EAP_Message p_EAP := omit, template (omit) GPRS_Timer3 p_BackOff := omit, boolean p_PDUSessionAuthenticationRequired := false, template (omit) charstring p_BuildQosFlow := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromLTE := omit, boolean p_OmitDNN := false, // @sic R5-223987 sic@ template (omit) PDU_PDN_DNN_Type p_DNNType := omit) runs on NR5GC_PTC return PDU_PDN_DNN_Type // @sic R5-198184, R5-206296 sic@ { // Split function so list of DRBs can be built elsewhere @sic R5-199103 sic@ // Moved UT trigger up to f_NR5GC_PDUSessionEstablishment @sic R5s200751 sic@ var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var GMM_MobilityInfo_Type v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); // @sic R5-195328 sic@ var GSM_MobilityInfo_Type v_GSM_MobilityInfo; ………………………………………… f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } if (isvalue(p_DNNType)) { // KS: R5x2xxxx v_PDUType := p_DNNType; v_GSM_MobilityInfo.PDUType := p_DNNType; } if (match(p_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.requestType.requestValue, '011'B) or match(p_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.requestType.requestValue, '100'B)) { // @sic R5-202550, R5s210518, R5s20883 sic@ // this is the only way to flag it's an emergency PDU v_GSM_MobilityInfo.PDUType := Emergency_PDN; // @sic R5-206296 sic@ v_MediaType := IPCAN_EmergencyCall; // @sic R5s211486 sic@ } …………………………….. }