3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220211

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2351 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for EN-DC RRC test case 8.2.1.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. The PICS pc_dynamicPowerSharing indicates the UE support of dynamicPowerSharingENDC as per 38.306 4.2.7.9: This capability is applicable onle for FR1 bands. A UE supporting both FR1 and FR2 bands may operate with the PICS set to TRUE but in FR2 operation, capabilities will be requested only for the current operation band, so the UE may report the associated capability as not supported since it is not applicable for FR2 bands. 2. According to 38.523-1 table 8.2.1.1.1.3.3-3F, all fields of EUTRA-5GC-Parameters-v1610 are “not checked”. Since all fields are optional, an R16 UE not supporting these capabilities may omit the structure altogether. This is currently not allowed in the implementation of higher template. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Check the setting of DynamicPowerSharingENDC for FR1 operation only. - Allow eutra_5GC_Parameters_v1610 IE to be omitted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_Check_RF_ParamsMRDC_r15() |
| --- | --- |
| Reason for change | The PICS pc_dynamicPowerSharing indicates the UE support of dynamicPowerSharingENDC as per 38.306 4.2.7.9: This capability is applicable onle for FR1 bands. A UE supporting both FR1 and FR2 bands may operate with the PICS set to TRUE but in FR2 operation, capabilities will be requested only for the current operation band, so the UE may report the associated capability as not supported since it is not applicable for FR2 bands. |
| Summary of change | Check the setting of DynamicPowerSharingENDC for FR1 operation only. |
| TTCN module | RRC_Other_CapabilityFunctions_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_Check_RF_ParamsMRDC_r15(UE_MRDC_Capability p_MRDC_Capability) { //@sic R5-194844 R5-201123 sic@ var UE_MRDC_Capability v_Received_MRDCCapabilityMsg; var integer v_Index := 0; var boolean v_Found_IsDynamicPowerSharing := false; v_Received_MRDCCapabilityMsg := p_MRDC_Capability; if (isvalue(v_Received_MRDCCapabilityMsg.rf_ParametersMRDC)) { // rf-ParametersMRDC is mandatory, so need to check first if (isvalue(v_Received_MRDCCapabilityMsg.rf_ParametersMRDC.supportedBandCombinationList[0])) { // check the array if it has contents while ((v_Index < lengthof(v_Received_MRDCCapabilityMsg.rf_ParametersMRDC.supportedBandCombinationList)) and (not v_Found_IsDynamicPowerSharing)) { // run over the array until content found or until it ends if (isvalue(v_Received_MRDCCapabilityMsg.rf_ParametersMRDC.supportedBandCombinationList[v_Index].mrdc_Parameters.dynamicPowerSharingENDC )) { /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ v_Found_IsDynamicPowerSharing := true; break; } else { v_Index := v_Index +1; } } } if (not f_NR_CompareCapability_vs_PICS(pc_dynamicPowerSharing, v_Found_IsDynamicPowerSharing)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_dynamicPowerSharing not correctly set"); } } else { f_SetVerdict(fail,__FILE__, __LINE__, "Mandatory rf_ParametersMRDC not included on UE Capability message Rx"); } } // fl_Check_RF_ParamsMRDC_r15

After Change:

function fl_Check_RF_ParamsMRDC_r15(UE_MRDC_Capability p_MRDC_Capability) { //@sic R5-194844 R5-201123 sic@ var UE_MRDC_Capability v_Received_MRDCCapabilityMsg; var integer v_Index := 0; var boolean v_Found_IsDynamicPowerSharing := false; v_Received_MRDCCapabilityMsg := p_MRDC_Capability; if (isvalue(v_Received_MRDCCapabilityMsg.rf_ParametersMRDC)) { // rf-ParametersMRDC is mandatory, so need to check first if(f_NR_IsBandFR1(f_ENDC_GetNR_Band(px_ENDC_BandCombination)))//KS#21xxx { if (isvalue(v_Received_MRDCCapabilityMsg.rf_ParametersMRDC.supportedBandCombinationList[0])) { // check the array if it has contents while ((v_Index < lengthof(v_Received_MRDCCapabilityMsg.rf_ParametersMRDC.supportedBandCombinationList)) and (not v_Found_IsDynamicPowerSharing)) { // run over the array until content found or until it ends if (isvalue(v_Received_MRDCCapabilityMsg.rf_ParametersMRDC.supportedBandCombinationList[v_Index].mrdc_Parameters.dynamicPowerSharingENDC )) { /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ v_Found_IsDynamicPowerSharing := true; break; } else { v_Index := v_Index +1; } } } if (not f_NR_CompareCapability_vs_PICS(pc_dynamicPowerSharing, v_Found_IsDynamicPowerSharing)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_dynamicPowerSharing not correctly set"); } } } else { f_SetVerdict(fail,__FILE__, __LINE__, "Mandatory rf_ParametersMRDC not included on UE Capability message Rx"); } } // fl_Check_RF_ParamsMRDC_r15

## Change 2

| Function name | template UE_EUTRA_Capability_v1550_IEs |
| --- | --- |
| Reason for change | According to 38.523-1 table 8.2.1.1.1.3.3-3F, all fields of EUTRA-5GC-Parameters-v1610 are “not checked”. Since all fields are optional, an R16 UE not supporting these capabilities may omit the structure altogether. This is currently not allowed in the implementation of higher template. |
| Summary of change | Allow eutra_5GC_Parameters_v1610 IE to be omitted. |
| TTCN module | RRC_UECapability_ENDC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

template UE_EUTRA_Capability_v1550_IEs cr_UE_EUTRA_Capability_v1550_IEs := // @sic R5s201387 BASELINE MOVING 2020 sic@ { /* @status APPROVED (ENDC) */ neighCellSI_AcquisitionParameters_v1550 := *, phyLayerParameters_v1550 := ?, mac_Parameters_v1550 := ?, fdd_Add_UE_EUTRA_Capabilities_v1550 := ?, tdd_Add_UE_EUTRA_Capabilities_v1550 := ?, nonCriticalExtension := { pdcp_ParametersNR_v1560 := ?, irat_ParametersNR_v1560 := ?, appliedCapabilityFilterCommon_r15 := *, fdd_Add_UE_EUTRA_Capabilities_v1560 := ?, tdd_Add_UE_EUTRA_Capabilities_v1560 := ?, nonCriticalExtension := { rf_Parameters_v1570 := *, irat_ParametersNR_v1570 := *, nonCriticalExtension := { neighCellSI_AcquisitionParameters_v15a0 := ?, eutra_5GC_Parameters_r15 := *, fdd_Add_UE_EUTRA_Capabilities_v15a0 := *, tdd_Add_UE_EUTRA_Capabilities_v15a0 := *, nonCriticalExtension := { highSpeedEnhParameters_v1610 := *, neighCellSI_AcquisitionParameters_v1610 := *, mbms_Parameters_v1610 := *, pdcp_Parameters_v1610 := *, mac_Parameters_v1610 := *, phyLayerParameters_v1610 := *, measParameters_v1610 := *, pur_Parameters_r16 := *, eutra_5GC_Parameters_v1610 := { // @sic R5-211501 sic@ ce_InactiveState_r16 := *, ce_EUTRA_5GC_r16 := * }, otherParameters_v1610 := *, dl_DedicatedMessageSegmentation_r16 := *, mmtel_Parameters_v1610 := ?, irat_ParametersNR_v1610 := { // @sic R5-206430 sic@ nr_HO_ToEN_DC_r16 := *, //checked in detail on fl_Derive_EUTRA_nr_HO_ToEN_DC_r16 ce_EUTRA_5GC_HO_ToNR_FDD_FR1_r16 := *, ce_EUTRA_5GC_HO_ToNR_TDD_FR1_r16 := *, ce_EUTRA_5GC_HO_ToNR_FDD_FR2_r16 := *, ce_EUTRA_5GC_HO_ToNR_TDD_FR2_r16 := * } ifpresent, // IRAT_ParametersNR_v1610 rf_Parameters_v1610 := *, mobilityParameters_v1610 := *, ue_BasedNetwPerfMeasParameters_v1610 := ?, sl_Parameters_v1610 := *, fdd_Add_UE_EUTRA_Capabilities_v1610 := { // @sic R5-206430 sic@ phyLayerParameters_v1610 := *, pur_Parameters_r16 := *, measParameters_v1610 := *, eutra_5GC_Parameters_v1610 := { // @sic R5-211501 sic@ ce_InactiveState_r16 := *, ce_EUTRA_5GC_r16 := * }, irat_ParametersNR_v1610 := { nr_HO_ToEN_DC_r16 := *, //checked in detail on fl_Derive_EUTRA_nr_HO_ToEN_DC_r16 ce_EUTRA_5GC_HO_ToNR_FDD_FR1_r16 := *, ce_EUTRA_5GC_HO_ToNR_TDD_FR1_r16 := *, ce_EUTRA_5GC_HO_ToNR_FDD_FR2_r16 := *, ce_EUTRA_5GC_HO_ToNR_TDD_FR2_r16 := * } ifpresent, neighCellSI_AcquisitionParameters_v1610 := *, mobilityParameters_v1610 := * } ifpresent, //UE_EUTRA_CapabilityAddXDD_Mode_v1610 tdd_Add_UE_EUTRA_Capabilities_v1610 := { // @sic R5-206430 sic@ phyLayerParameters_v1610 := *, pur_Parameters_r16 := *, measParameters_v1610 := *, eutra_5GC_Parameters_v1610 := { // @sic R5-211501 sic@ ce_InactiveState_r16 := *, ce_EUTRA_5GC_r16 := * }, irat_ParametersNR_v1610 := { nr_HO_ToEN_DC_r16 := *, //checked in detail on fl_Derive_EUTRA_nr_HO_ToEN_DC_r16 ce_EUTRA_5GC_HO_ToNR_FDD_FR1_r16 := *, ce_EUTRA_5GC_HO_ToNR_TDD_FR1_r16 := *, ce_EUTRA_5GC_HO_ToNR_FDD_FR2_r16 := *, ce_EUTRA_5GC_HO_ToNR_TDD_FR2_r16 := * } ifpresent, neighCellSI_AcquisitionParameters_v1610 := *, mobilityParameters_v1610 := * } ifpresent, //UE_EUTRA_CapabilityAddXDD_Mode_v1610 nonCriticalExtension := * } ifpresent // UE-EUTRA-Capability-v1610-IEs } ifpresent // UE-EUTRA-Capability-v15a0-IEs } ifpresent // UE-EUTRA-Capability-v1570-IEs } ifpresent // UE-EUTRA-Capability-v1560-IEs } ifpresent; // UE-EUTRA-Capability-v1550-IEs

After Change:

template UE_EUTRA_Capability_v1550_IEs cr_UE_EUTRA_Capability_v1550_IEs := // @sic R5s201387 BASELINE MOVING 2020 sic@ { /* @status APPROVED (ENDC) */ neighCellSI_AcquisitionParameters_v1550 := *, phyLayerParameters_v1550 := ?, mac_Parameters_v1550 := ?, fdd_Add_UE_EUTRA_Capabilities_v1550 := ?, tdd_Add_UE_EUTRA_Capabilities_v1550 := ?, nonCriticalExtension := { pdcp_ParametersNR_v1560 := ?, irat_ParametersNR_v1560 := ?, appliedCapabilityFilterCommon_r15 := *, fdd_Add_UE_EUTRA_Capabilities_v1560 := ?, tdd_Add_UE_EUTRA_Capabilities_v1560 := ?, nonCriticalExtension := { rf_Parameters_v1570 := *, irat_ParametersNR_v1570 := *, nonCriticalExtension := { neighCellSI_AcquisitionParameters_v15a0 := ?, eutra_5GC_Parameters_r15 := *, fdd_Add_UE_EUTRA_Capabilities_v15a0 := *, tdd_Add_UE_EUTRA_Capabilities_v15a0 := *, nonCriticalExtension := { highSpeedEnhParameters_v1610 := *, neighCellSI_AcquisitionParameters_v1610 := *, mbms_Parameters_v1610 := *, pdcp_Parameters_v1610 := *, mac_Parameters_v1610 := *, phyLayerParameters_v1610 := *, measParameters_v1610 := *, pur_Parameters_r16 := *, eutra_5GC_Parameters_v1610 := { // @sic R5-211501 sic@ ce_InactiveState_r16 := *, ce_EUTRA_5GC_r16 := * }ifpresent, otherParameters_v1610 := *, dl_DedicatedMessageSegmentation_r16 := *, mmtel_Parameters_v1610 := ?, irat_ParametersNR_v1610 := { // @sic R5-206430 sic@ nr_HO_ToEN_DC_r16 := *, //checked in detail on fl_Derive_EUTRA_nr_HO_ToEN_DC_r16 ce_EUTRA_5GC_HO_ToNR_FDD_FR1_r16 := *, ce_EUTRA_5GC_HO_ToNR_TDD_FR1_r16 := *, ce_EUTRA_5GC_HO_ToNR_FDD_FR2_r16 := *, ce_EUTRA_5GC_HO_ToNR_TDD_FR2_r16 := * } ifpresent, // IRAT_ParametersNR_v1610 rf_Parameters_v1610 := *, mobilityParameters_v1610 := *, ue_BasedNetwPerfMeasParameters_v1610 := ?, sl_Parameters_v1610 := *, fdd_Add_UE_EUTRA_Capabilities_v1610 := { // @sic R5-206430 sic@ phyLayerParameters_v1610 := *, pur_Parameters_r16 := *, measParameters_v1610 := *, eutra_5GC_Parameters_v1610 := { // @sic R5-211501 sic@ ce_InactiveState_r16 := *, ce_EUTRA_5GC_r16 := * }ifpresent, irat_ParametersNR_v1610 := { nr_HO_ToEN_DC_r16 := *, //checked in detail on fl_Derive_EUTRA_nr_HO_ToEN_DC_r16 ce_EUTRA_5GC_HO_ToNR_FDD_FR1_r16 := *, ce_EUTRA_5GC_HO_ToNR_TDD_FR1_r16 := *, ce_EUTRA_5GC_HO_ToNR_FDD_FR2_r16 := *, ce_EUTRA_5GC_HO_ToNR_TDD_FR2_r16 := * } ifpresent, neighCellSI_AcquisitionParameters_v1610 := *, mobilityParameters_v1610 := * } ifpresent, //UE_EUTRA_CapabilityAddXDD_Mode_v1610 tdd_Add_UE_EUTRA_Capabilities_v1610 := { // @sic R5-206430 sic@ phyLayerParameters_v1610 := *, pur_Parameters_r16 := *, measParameters_v1610 := *, eutra_5GC_Parameters_v1610 := { // @sic R5-211501 sic@ ce_InactiveState_r16 := *, ce_EUTRA_5GC_r16 := * }ifpresent, irat_ParametersNR_v1610 := { nr_HO_ToEN_DC_r16 := *, //checked in detail on fl_Derive_EUTRA_nr_HO_ToEN_DC_r16 ce_EUTRA_5GC_HO_ToNR_FDD_FR1_r16 := *, ce_EUTRA_5GC_HO_ToNR_TDD_FR1_r16 := *, ce_EUTRA_5GC_HO_ToNR_FDD_FR2_r16 := *, ce_EUTRA_5GC_HO_ToNR_TDD_FR2_r16 := * } ifpresent, neighCellSI_AcquisitionParameters_v1610 := *, mobilityParameters_v1610 := * } ifpresent, //UE_EUTRA_CapabilityAddXDD_Mode_v1610 nonCriticalExtension := * } ifpresent // UE-EUTRA-Capability-v1610-IEs } ifpresent // UE-EUTRA-Capability-v15a0-IEs } ifpresent // UE-EUTRA-Capability-v1570-IEs } ifpresent // UE-EUTRA-Capability-v1560-IEs } ifpresent; // UE-EUTRA-Capability-v1550-IEs