3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220268

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2280 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 7.1.1.11.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The function f_NR_CellInfo_SetMAC_CellGroupConfigL2 is not required to be called before preamble as it is called later on during the test after preamble. Having this function before preamable will cause problem with BSR timer related issues due to UE capability being too long it is proposed to update the BSR time after the preamble (see R5s210862) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the function call for f_NR_CellInfo_SetMAC_CellGroupConfigL2 before the preamble |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.11.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required for test case 7.1.1.11.1	2

1.1	Correction to f_TC_7_1_1_11_1_NR5GC()	2

# Corrections required for test case 7.1.1.11.1

## Correction to f_TC_7_1_1_11_1_NR5GC()

| Function name | f_TC_7_1_1_11_1_NR5GC() |
| --- | --- |
| Reason for change | The function f_NR_CellInfo_SetMAC_CellGroupConfigL2 is not required to be called before preamble as it is called later on during the test after preamble. Having this function before preamable will cause problem with BSR timer related issues due to UE capability being too long it is proposed to update the BSR time after the preamble (see R5s210862) |
| Summary of change | Removed the function call for f_NR_CellInfo_SetMAC_CellGroupConfigL2 before the preamble |
| TTCN module |  |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_11_1_NR5GC() runs on NR5GC_PTC { //DC power headroom reporting / PSCell activation and DL pathloss change reporting var template (value) CellGroupConfig v_SCGConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var boolean v_RRC_MsgRcvd; var boolean v_PHR_Rcvd; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP ; var NR_SYSTEM_IND v_NR_SYSTEM_IND; var SubFrameTiming_Type v_TimingInfo; //@sic R5s211698 R5-217806 sic@ var template (value) CellGroupConfig v_MCGConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig_SCG; var boolean v_PHR_PSCell_Rcvd; var SubFrameTiming_Type v_TimingInfo_PSCell; var NR_ResourceAllocation_Type v_NR_ResourceAllocation_SCG; var NR_UplinkBWP_Type v_NR_UplinkBWP_SCG ; f_NR5GC_Init_NRDC(NR_1, {nr_Cell1, nr_Cell10}); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -82, -82); //@sic R5s211698 R5-217806 sic@ f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell10); //Create and configure NR cell1 and NR cell 10. f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell10, -, tsc_NRDC_SCG_Cell); f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A);// f_NR_SetCellPower (nr_Cell1, -82, -82); //@sic R5s211698 R5-217806 sic@ f_NR_SetCellPower (nr_Cell10, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5s211698 R5-217806 sic@ f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell10); f_NR_RRCReconfig_AddSCG_DRBs_NRDC(nr_Cell1, nr_Cell10, MCG_SCG);// f_NR_TestBody_Set(true);

After Change:

function f_TC_7_1_1_11_1_NR5GC() runs on NR5GC_PTC { //DC power headroom reporting / PSCell activation and DL pathloss change reporting var template (value) CellGroupConfig v_SCGConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var boolean v_RRC_MsgRcvd; var boolean v_PHR_Rcvd; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP ; var NR_SYSTEM_IND v_NR_SYSTEM_IND; var SubFrameTiming_Type v_TimingInfo; //@sic R5s211698 R5-217806 sic@ var template (value) CellGroupConfig v_MCGConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig_SCG; var boolean v_PHR_PSCell_Rcvd; var SubFrameTiming_Type v_TimingInfo_PSCell; var NR_ResourceAllocation_Type v_NR_ResourceAllocation_SCG; var NR_UplinkBWP_Type v_NR_UplinkBWP_SCG ; f_NR5GC_Init_NRDC(NR_1, {nr_Cell1, nr_Cell10}); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -82, -82); //@sic R5s211698 R5-217806 sic@ // removed SetMAC_CellGroupConfigL2 as it is configured later after preamble //f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); //f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell10); //Create and configure NR cell1 and NR cell 10. f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell10, -, tsc_NRDC_SCG_Cell); f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A);// f_NR_SetCellPower (nr_Cell1, -82, -82); //@sic R5s211698 R5-217806 sic@ f_NR_SetCellPower (nr_Cell10, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5s211698 R5-217806 sic@ f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell10); f_NR_RRCReconfig_AddSCG_DRBs_NRDC(nr_Cell1, nr_Cell10, MCG_SCG);// f_NR_TestBody_Set(true);