3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220472

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2448 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.1.6.1.2.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to RAN5#94e agreed prose CR R5-221564, RRC Reconfiguration message at Step 16 shall be based on condition RBConfig_KeyChange, currently TTCN is not providing the IE masterKeyUpdate |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified the TTCN to include the the IE masterKeyUpdate and also the nextHopChainingCount IE to local end configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.8.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required to test case 8.1.6.1.2.8.NR5GC	2

1.1	Correction to f_TC_8_1_6_1_2_8_TestBody	2

# Corrections required to test case 8.1.6.1.2.8.NR5GC

## Correction to f_TC_8_1_6_1_2_8_TestBody

| Function name | f_TC_8_1_6_1_2_8_TestBody |
| --- | --- |
| Reason for change | According to RAN5#94e agreed prose CR R5-221564, RRC Reconfiguration message at Step 16 shall be based on condition RBConfig_KeyChange, currently TTCN is not providing the IE masterKeyUpdate |
| Summary of change | Modified the TTCN to include the the IE masterKeyUpdate and also the nextHopChainingCount IE to local end configuration |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_6_1_2_8_TestBody() runs on NR5GC_PTC { var AbsoluteTimeInfo_r16 v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var CellIdentity v_CellIdentity_NRCell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var TrackingAreaCode v_TAC_NRCell1 := f_NR_CellInfo_GetTAC (nr_Cell1); var ARFCN_ValueNR v_NRf_SSB_NRCell2 := f_NR_CellInfo_GetFrequencySSB(nr_Cell2); var PhysCellId v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); var PhysCellId v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(nr_Cell2); var template (present) MeasQuantityResults v_MeasQuantityResults; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var NR_AbsoluteCellPower_Type v_CellPower_Serving_FR1; var NR_AbsoluteCellPower_Type v_CellPower_Serving_FR2; //@sic R5-216308 sic@ var NR_AbsoluteCellPower_Type v_CellPower_EntryCondition_FR1; var NR_AbsoluteCellPower_Type v_CellPower_EntryCondition_FR2; //@sic R5-216308 sic@ var NR_AbsoluteCellPower_Type v_CellPower_Leaving_FR1; var NR_AbsoluteCellPower_Type v_CellPower_Leaving_FR2; //@sic R5-216308 sic@ var RSRP_Range v_RsrpThres := 2; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var template (value) DL_DCCH_Message v_RRCReconfigurationHO; var NR_SRB_COMMON_IND v_ReceivedAsp; var LogMeasInfo_r16 v_LogMeasInfo_r16; var integer v_FieldCnt; var integer i:=0; //Power levels of NR cell 1 and cell 2 FR1 acc. Table 8.1.6.1.2.8.3.2-1 /* ¦ Cell 1 ¦ Cell 2 * ------------------------------- * T0 ¦ -85 ¦ -91 * ------------------------------- * T1 ¦ -85 ¦ -79 * ------------------------------- */ //@sic R5-216308 sic@ //Power levels of NR cell 1 and cell 2 FR2 acc. Table 8.1.6.1.2.8.3.2-2 /* ¦ Cell 1 ¦ Cell 2 * ------------------------------- * T0 ¦ -82 ¦ -91 * ------------------------------- * T1 ¦ -82 ¦ -75 * ------------------------------- */ v_CellPower_Serving_FR1 := -85; v_CellPower_Serving_FR2 := -82; //@sic R5-216308 sic@ v_CellPower_Leaving_FR1 := -91; v_CellPower_Leaving_FR2 := -91; //@sic R5-216308 sic@ v_CellPower_EntryCondition_FR1 := -79; v_CellPower_EntryCondition_FR2 := -75; //@sic R5-216308 sic@ v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, v_CellPower_Serving_FR1, v_CellPower_Serving_FR2), //@sic R5-216308 sic@ cs_NR_CellPower(nr_Cell2, v_CellPower_Leaving_FR1, v_CellPower_Leaving_FR2)}; //@sic R5-216308 sic@ v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell2, v_CellPower_EntryCondition_FR1, v_CellPower_EntryCondition_FR2)}; //@sic R5-216308 sic@ f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Step 1" siclog@ //The SS transmits a LoggedMeasurementConfiguration message on NR Cell 1. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo_r16, cs_LoggedMeasurementConfiguration_reportType_Periodical(ms10240)))); //@siclog "Step 2" siclog@ //The SS transmits a RRCRelease message to release the RRC connection. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 3" siclog@ //Wait 30 seconds for the UE to perform the logging at regular time intervals. f_Delay(30.0); //@siclog "Step 4-11" siclog@ //Steps 1 to 8 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, -, -, -, -, -, -, -, -, -, -, cr_RRCSetupComplete_v1610_IEs(-,-,cr_UE_MeasurementsAvailable_r16(true_))); //@siclog "Step 12" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig to setup intra NR measurement and reporting for inter-frequency event A3. //@siclog "Step 13" siclog@ //The UE transmits an RRCReconfigurationComplete message. v_ReportConfigList := {cs_NR_ReportConfigEventA3(tsc_NR_IdReportConfigId1, v_RsrpThres, r1, -, -, cs_NR_MeasReportQuantity_Rsrp)}; v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1}; v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList ); f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig); //@siclog "Step 14" siclog@ //The SS changes NR Cell 1 and NR Cell 2 parameters according to the row "T1" in tables 8.1.6.1.2.8.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 15" siclog@ //The UE transmit a MeasurementReport message to report event A3 with the measured RSRP value for NR Cell 2? f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId1, -, -, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour, cr_NR_MeasQuantityResults_Rsrp)})); //@siclog "Step 16" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform intra frequency handover to NR Cell 2 //@siclog "Step 17" siclog@ //Check: Does the UE transmit an RRCReconfigurationComplete message on NR Cell 2 with logMeasAvailable set to true to confirm the successful completion of the intra frequency handover? v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2); //@sic R5s220074 sic@ f_NR5GC_508RRC_IntraNR_HO_InterCell_Common(nr_Cell1, nr_Cell2, -, -, v_RRCReconfigurationHO, f_NR_GetRACHProcedureConfigPSCell(nr_Cell2), f_NR5GC_MobileInfo_GetDRBInfoList(), -, -, cr_38508_RRCReconfigurationComplete(cr_RRCReconfigurationComplete_v1530(-, cr_RRCReconfigurationComplete_v1560(-,cr_RRCReconfigurationComplete_v1610_LogMeas)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 17 PASS"); //@siclog "Step 18" siclog@ //he SS sends an UEInformationRequest message to get logMeasReport. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell2, -, cs_NR_UEInformationRequest_LOG));

After Change:

function f_TC_8_1_6_1_2_8_TestBody() runs on NR5GC_PTC { var AbsoluteTimeInfo_r16 v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var CellIdentity v_CellIdentity_NRCell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var TrackingAreaCode v_TAC_NRCell1 := f_NR_CellInfo_GetTAC (nr_Cell1); var ARFCN_ValueNR v_NRf_SSB_NRCell2 := f_NR_CellInfo_GetFrequencySSB(nr_Cell2); var PhysCellId v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); var PhysCellId v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(nr_Cell2); var template (present) MeasQuantityResults v_MeasQuantityResults; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var NR_AbsoluteCellPower_Type v_CellPower_Serving_FR1; var NR_AbsoluteCellPower_Type v_CellPower_Serving_FR2; //@sic R5-216308 sic@ var NR_AbsoluteCellPower_Type v_CellPower_EntryCondition_FR1; var NR_AbsoluteCellPower_Type v_CellPower_EntryCondition_FR2; //@sic R5-216308 sic@ var NR_AbsoluteCellPower_Type v_CellPower_Leaving_FR1; var NR_AbsoluteCellPower_Type v_CellPower_Leaving_FR2; //@sic R5-216308 sic@ var RSRP_Range v_RsrpThres := 2; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var template (value) DL_DCCH_Message v_RRCReconfigurationHO; var NR_SRB_COMMON_IND v_ReceivedAsp; var LogMeasInfo_r16 v_LogMeasInfo_r16; var integer v_FieldCnt; var integer i:=0; var template (value) MasterKeyUpdate v_MasterKeyUpdate; // var NextHopChainingCount v_NCC; // //Power levels of NR cell 1 and cell 2 FR1 acc. Table 8.1.6.1.2.8.3.2-1 /* ¦ Cell 1 ¦ Cell 2 * ------------------------------- * T0 ¦ -85 ¦ -91 * ------------------------------- * T1 ¦ -85 ¦ -79 * ------------------------------- */ //@sic R5-216308 sic@ //Power levels of NR cell 1 and cell 2 FR2 acc. Table 8.1.6.1.2.8.3.2-2 /* ¦ Cell 1 ¦ Cell 2 * ------------------------------- * T0 ¦ -82 ¦ -91 * ------------------------------- * T1 ¦ -82 ¦ -75 * ------------------------------- */ v_CellPower_Serving_FR1 := -85; v_CellPower_Serving_FR2 := -82; //@sic R5-216308 sic@ v_CellPower_Leaving_FR1 := -91; v_CellPower_Leaving_FR2 := -91; //@sic R5-216308 sic@ v_CellPower_EntryCondition_FR1 := -79; v_CellPower_EntryCondition_FR2 := -75; //@sic R5-216308 sic@ v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, v_CellPower_Serving_FR1, v_CellPower_Serving_FR2), //@sic R5-216308 sic@ cs_NR_CellPower(nr_Cell2, v_CellPower_Leaving_FR1, v_CellPower_Leaving_FR2)}; //@sic R5-216308 sic@ v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell2, v_CellPower_EntryCondition_FR1, v_CellPower_EntryCondition_FR2)}; //@sic R5-216308 sic@ f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Step 1" siclog@ //The SS transmits a LoggedMeasurementConfiguration message on NR Cell 1. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo_r16, cs_LoggedMeasurementConfiguration_reportType_Periodical(ms10240)))); //@siclog "Step 2" siclog@ //The SS transmits a RRCRelease message to release the RRC connection. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 3" siclog@ //Wait 30 seconds for the UE to perform the logging at regular time intervals. f_Delay(30.0); //@siclog "Step 4-11" siclog@ //Steps 1 to 8 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, -, -, -, -, -, -, -, -, -, -, cr_RRCSetupComplete_v1610_IEs(-,-,cr_UE_MeasurementsAvailable_r16(true_))); //@siclog "Step 12" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig to setup intra NR measurement and reporting for inter-frequency event A3. //@siclog "Step 13" siclog@ //The UE transmits an RRCReconfigurationComplete message. v_ReportConfigList := {cs_NR_ReportConfigEventA3(tsc_NR_IdReportConfigId1, v_RsrpThres, r1, -, -, cs_NR_MeasReportQuantity_Rsrp)}; v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1}; v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList ); f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig); //@siclog "Step 14" siclog@ //The SS changes NR Cell 1 and NR Cell 2 parameters according to the row "T1" in tables 8.1.6.1.2.8.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 15" siclog@ //The UE transmit a MeasurementReport message to report event A3 with the measured RSRP value for NR Cell 2? f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId1, -, -, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour, cr_NR_MeasQuantityResults_Rsrp)})); //@siclog "Step 16" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform intra frequency handover to NR Cell 2 //@siclog "Step 17" siclog@ //Check: Does the UE transmit an RRCReconfigurationComplete message on NR Cell 2 with logMeasAvailable set to true to confirm the successful completion of the intra frequency handover? v_MasterKeyUpdate := cs_MasterKeyUpdate(false, 0); // v_NCC := valueof(v_MasterKeyUpdate.nextHopChainingCount); // v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2, -, -, -, -, -, v_MasterKeyUpdate); f_NR5GC_508RRC_IntraNR_HO_InterCell_Common(nr_Cell1, nr_Cell2, -, -, v_RRCReconfigurationHO, f_NR_GetRACHProcedureConfigPSCell(nr_Cell2), f_NR5GC_MobileInfo_GetDRBInfoList(), -, -, cr_38508_RRCReconfigurationComplete(cr_RRCReconfigurationComplete_v1530(-, cr_RRCReconfigurationComplete_v1560(-,cr_RRCReconfigurationComplete_v1610_LogMeas))), v_NCC); f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 17 PASS"); //@siclog "Step 18" siclog@ //he SS sends an UEInformationRequest message to get logMeasReport. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell2, -, cs_NR_UEInformationRequest_LOG));