3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220476

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2452 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR MAC test case 7.1.1.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The variables v_SrbIndDummy and v_SysIndDummy are maintained and passed by reference into the altstep a_NR_RacingCond_AwaitRrcMessageAndSysInd() several times, so their values should be reset to omit between each use, otherwise they will still refer to ASPs received at previous steps. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Initiate v_SrbIndDummy and v_SysIndDummy to omit after use of altstep a_NR_RacingCond_AwaitRrcMessageAndSysInd() at step 2-4,steps 6-8 and steps 10-12. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not execute correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.6.ENDC, 7.1.1.1.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_1_6_NR_TestBody |
| --- | --- |
| Reason for change | The variables v_SrbIndDummy and v_SysIndDummy are maintained and passed by reference into the altstep a_NR_RacingCond_AwaitRrcMessageAndSysInd() several times, so their values should be reset to omit between each use, otherwise they will still refer to ASPs received at previous steps. |
| Summary of change | Initiate v_SrbIndDummy and v_SysIndDummy to omit after use of altstep a_NR_RacingCond_AwaitRrcMessageAndSysInd() at step 2-4,steps 6-8 and steps 10-12. |
| TTCN module | MAC_NR5GC |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_1_6_NR_TestBody() runs on NR_BASE_PTC return NG_NAS_MSG_Indication_Type { //@sic R5s210091 R5s211304 sic@ var NG_NAS_MSG_Indication_Type v_NasInd; var template (omit) NR_SRB_COMMON_IND v_SrbIndDummy := omit; var template (omit) NR_SYSTEM_IND v_SysIndDummy := omit; timer t_Watchdog := 10.0; //@sic r5s190764 sic@ t_Watchdog.start; // @siclog "Step 3" siclog@ //Preconfigured at SS // @siclog "Step 2-4" siclog@ a_NR_RacingCond_AwaitRrcMessageAndSysInd(nr_Cell1, SysIndBeforeRrcMsg, v_SrbIndDummy, v_SysIndDummy, car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest), car_NR_PRACH_Preamble_IND(nr_Cell1, -, (0..31))) ; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2 4"); // @siclog "Step 5" siclog@ //Preconfigured at SS // @siclog "Step 7" siclog@ //Preconfigured at SS // @siclog "Step 6 8" siclog@ a_NR_RacingCond_AwaitRrcMessageAndSysInd(nr_Cell1, SysIndBeforeRrcMsg, v_SrbIndDummy, v_SysIndDummy, car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest), car_NR_PRACH_Preamble_IND(nr_Cell1, -, (0..31))) ; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6 8"); // @siclog "Step 9" siclog@ //Preconfigured at SS // @siclog "Step 11" siclog@ //Preconfigured at SS // @siclog "Step 10-12" siclog@ alt { [] a_NR_RacingCond_AwaitRrcMessageAndSysInd(nr_Cell1, SysIndBeforeRrcMsg, v_SrbIndDummy, car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest), car_NR_PRACH_Preamble_IND(nr_Cell1, -, (0..31))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10 12"); } [] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10: UE is sending RRCSetupComplete"); } }; // @siclog "Step 13" siclog@ //Preconfigured at SS // @siclog "Step 14" siclog@ v_NasInd := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def)); ...... }

After Change

function f_TC_7_1_1_1_6_NR_TestBody() runs on NR_BASE_PTC return NG_NAS_MSG_Indication_Type { //@sic R5s210091 R5s211304 sic@ var NG_NAS_MSG_Indication_Type v_NasInd; var template (omit) NR_SRB_COMMON_IND v_SrbIndDummy := omit; var template (omit) NR_SYSTEM_IND v_SysIndDummy := omit; timer t_Watchdog := 10.0; //@sic r5s190764 sic@ t_Watchdog.start; // @siclog "Step 3" siclog@ //Preconfigured at SS // @siclog "Step 2-4" siclog@ a_NR_RacingCond_AwaitRrcMessageAndSysInd(nr_Cell1, SysIndBeforeRrcMsg, v_SrbIndDummy, v_SysIndDummy, car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest), car_NR_PRACH_Preamble_IND(nr_Cell1, -, (0..31))) ; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2 4"); v_SrbIndDummy := omit; v_SysIndDummy := omit; // @siclog "Step 5" siclog@ //Preconfigured at SS // @siclog "Step 7" siclog@ //Preconfigured at SS // @siclog "Step 6 8" siclog@ a_NR_RacingCond_AwaitRrcMessageAndSysInd(nr_Cell1, SysIndBeforeRrcMsg, v_SrbIndDummy, v_SysIndDummy, car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest), car_NR_PRACH_Preamble_IND(nr_Cell1, -, (0..31))) ; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6 8"); v_SrbIndDummy := omit; v_SysIndDummy := omit; // @siclog "Step 9" siclog@ //Preconfigured at SS // @siclog "Step 11" siclog@ //Preconfigured at SS // @siclog "Step 10-12" siclog@ alt { [] a_NR_RacingCond_AwaitRrcMessageAndSysInd(nr_Cell1, SysIndBeforeRrcMsg, v_SrbIndDummy, car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest), car_NR_PRACH_Preamble_IND(nr_Cell1, -, (0..31))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10 12"); } [] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10: UE is sending RRCSetupComplete"); } }; v_SrbIndDummy := omit; v_SysIndDummy := omit; // @siclog "Step 13" siclog@ //Preconfigured at SS // @siclog "Step 14" siclog@ v_NasInd := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def)); ...... }