3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220498

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2471 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR RLC test case 7.1.2.3.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the 38.523-1 Table 7.1.2.3.7.3.2-1: Step 12: After 200 ms the SS transmits 2 AMD PDUs with SN=3, SN=7. The SS sets the P field of all the AMD PDUs to 0 except for that of the AMD PDU with SN=7. A time spacing of 20 ms is applied. Considering the test purpose of this case, the 200ms here should be after step 10 instead of the current system time, as the current system time is obtained later, and risk the t-StatusProhibit = 300ms expired before receiving the missing PDUs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Calculate v_TimingDL for step 12 based on Tc |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not execute correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_2_3_7_NR_TestBody |
| --- | --- |
| Reason for change | According to the 38.523-1 Table 7.1.2.3.7.3.2-1: Step 12: After 200 ms the SS transmits 2 AMD PDUs with SN=3, SN=7. The SS sets the P field of all the AMD PDUs to 0 except for that of the AMD PDU with SN=7. A time spacing of 20 ms is applied. Considering the test purpose of this case, the 200ms here should be after step 10 instead of the current system time, as the current system time is obtained later, and risk the t-StatusProhibit = 300ms expired before receiving the missing PDUs. |
| Summary of change | Calculate v_TimingDL for step 12 based on Tc |
| TTCN module | RLC_TC_Common_NR |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_2_3_7_NR_TestBody(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { … //@siclog "Step 10" siclog@ // Check 1: Does the UE transmit a Status Report with NACK_SN=3 and ACK_SN=7? // Record time TC // Check 2: (TC - TB) = t-StatusProhibit? p_RLC_Rec.AM_RX_Next_Ack := 7; //@sic R5-197030 sic@ v_NR_NackSN18Bit_List := { crs_NR_RLC_NACK_SN18_Simple(3) }; v_SubFrameTimingTc := f_NR_RxSTATUS_PDU_GetTimingInfo(p_RLC_Rec, p_NR_DRB_Id, v_NR_NackSN18Bit_List); v_Duration := f_SubFrameTiming_Duration( v_SubFrameTimingTb, v_SubFrameTimingTc); if (f_NR_SubFrameTimingCheckDuration( nr_Cell1, l2Timer, v_Duration, float2int(p_RLC_Rec.t_StatusProhibit * 1000.0))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10 Check 2"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10 Check 2"); } //@siclog "Step 11" siclog@ // The SS ignores scheduling requests unless otherwise specified and does not allocate any uplink grant. f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); // @siclog "Step 12" siclog@ // After 200 ms the SS transmits 2 AMD PDUs with SN=3, SN=7. The SS sets the P field of all the AMD PDUs to 0 except for that of the AMD PDU with SN=7. A time spacing of 20 ms is applied. v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 200); //@sic R5-197030 sic@ v_TimingDL1 := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 20); p_RLC_Rec.AM_TX_Next := 3; f_NR_TxAMD_SDU(p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_DataList[p_RLC_Rec.AM_TX_Next], tsc_NR_P_NoPoll, cs_TimingInfo_NR(v_TimingDL)); p_RLC_Rec.AM_TX_Next := 7; f_NR_TxAMD_SDU(p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_DataList[p_RLC_Rec.AM_TX_Next], tsc_NR_P_Poll, cs_TimingInfo_NR(v_TimingDL1)); //@siclog "Step 13" siclog@ ……… }

After Change

function f_TC_7_1_2_3_7_NR_TestBody(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { … //@siclog "Step 10" siclog@ // Check 1: Does the UE transmit a Status Report with NACK_SN=3 and ACK_SN=7? // Record time TC // Check 2: (TC - TB) = t-StatusProhibit? p_RLC_Rec.AM_RX_Next_Ack := 7; //@sic R5-197030 sic@ v_NR_NackSN18Bit_List := { crs_NR_RLC_NACK_SN18_Simple(3) }; v_SubFrameTimingTc := f_NR_RxSTATUS_PDU_GetTimingInfo(p_RLC_Rec, p_NR_DRB_Id, v_NR_NackSN18Bit_List); v_Duration := f_SubFrameTiming_Duration( v_SubFrameTimingTb, v_SubFrameTimingTc); if (f_NR_SubFrameTimingCheckDuration( nr_Cell1, l2Timer, v_Duration, float2int(p_RLC_Rec.t_StatusProhibit * 1000.0))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10 Check 2"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10 Check 2"); } //@siclog "Step 11" siclog@ // The SS ignores scheduling requests unless otherwise specified and does not allocate any uplink grant. f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); // @siclog "Step 12" siclog@ // After 200 ms the SS transmits 2 AMD PDUs with SN=3, SN=7. The SS sets the P field of all the AMD PDUs to 0 except for that of the AMD PDU with SN=7. A time spacing of 20 ms is applied. v_TimingDL.HSFN.Number := v_SubFrameTimingTc.HSFN.Number + (v_SubFrameTimingTc.SFN.Number + 20) / 1024; v_TimingDL.SFN.Number := (v_SubFrameTimingTc.SFN.Number + 20 ) mod 1024; v_TimingDL1 := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 20); p_RLC_Rec.AM_TX_Next := 3; f_NR_TxAMD_SDU(p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_DataList[p_RLC_Rec.AM_TX_Next], tsc_NR_P_NoPoll, cs_TimingInfo_NR(v_TimingDL)); p_RLC_Rec.AM_TX_Next := 7; f_NR_TxAMD_SDU(p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_DataList[p_RLC_Rec.AM_TX_Next], tsc_NR_P_Poll, cs_TimingInfo_NR(v_TimingDL1)); //@siclog "Step 13" siclog@ ……… }