3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220500

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0805 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for IMS 5GS call control test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Corrections for IMS call control video addition / removal test cases. - Request URI in 200OK at step 9 shall be the same as provided by the UE in the Contact header field of the INVITE creating the dialog - SIP UPDATE at step 6 is received from the UE, and hence the SIP URL expectation for Contact header should be done using the RX function f_SIP_BuildSipUri_RX() - The preliminary pass verdict at step 1 has incorrect step number - The UE may also include a port number in a URI - Incorrect operation code is being used for the QoS rules IE in the PDU SESSION MODIFICATION COMMAND for removing the video stream |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected the SIP URI sent in 200OK of step 9 - Used the correct function for the expected value of the URI in contact header - Corrected the preliminary pass verdict step number at step 1 - Changed the default value for port to ‘anyOrOmit’ - Removed the check on v_RemoveVoice so that cs_QoS_DeleteRule template is used Note : An associated prose CR on 38.508-1 will be raised at RAN5#95 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.20.NR5GC, 7.22.NR5GC, 7.23.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.508-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	7

2.3	Change 3	8

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_22_NR5GC_IMS1() |
| --- | --- |
| Reason for change | - Request URI in 200OK at step 9 shall be the same as provided by the UE in the Contact header field of the INVITE creating the dialog - SIP UPDATE at step 6 is received from the UE, and hence the SIP URL expectation for Contact header should be done using the RX function f_SIP_BuildSipUri_RX() - The preliminary pass verdict at step 1 has incorrect step number |
| Summary of change | - Corrected the SIP URI sent in 200OK of step 9 - Used the correct function for the expected value of the URI in contact header - Corrected the preliminary pass verdict step number at step 1 |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_22_NR5GC_IMS1() runs on IMS_PTC { // MTSI MT Voice Call / add video and remove video / with preconditions at both originating UE and terminating UE / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var SDP_Message v_LatestSdpOfferFromUE; var template (value) SDP_Message v_LatestSdpAnswerFromSS; var charstring v_CallIdentification; var boolean v_DontIncrementSessVers := true; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); f_IMS_MTCallSetup_NR5GC_A_5_1(); v_CallIdentification := f_UT_GetCallId(MMI); f_IMS_TestBody_Set(true); f_UT_AddVideoToIMSCall(MMI, v_CallIdentification); // @siclog "Step 1" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_Video_Step2(AddVideo, -, tsc_IMS_SDP_AudioCodecList_EVS_only, tsc_IMS_SDP_VideoCodecList_H265_H264); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 21"); // @siclog "Step 2 - 3" siclog@ 100 Trying, 183 Session Progress v_ResponseWithSdp := f_IMS_MOCallSetup_Video_Step3_4(AddVideo, v_InviteRequestWithSdp, EVS_16000, H265_90000); // @siclog "Step 4 - 5" siclog@ PRACK, 200 OK f_IMS_MOCallSetup_ReceivePRACK_Send200OK(v_InviteRequestWithSdp, v_ResponseWithSdp.Response.msgHeader); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5A" siclog@ f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MO_AddVideo); // @siclog "Step 6 - 7" siclog@ UPDATE, 200 OK v_LatestSdpOfferFromUE := f_IMS_MOCallSetup_NR5GC_Video_Step6_7(AddVideo, v_InviteRequestWithSdp); v_LatestSdpAnswerFromSS := f_IMS_BuildSDP_UpdatedAnswer_TX(v_LatestSdpOfferFromUE, v_DontIncrementSessVers); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); f_IMS_IPCAN_WaitForTrigger(IPCAN); // @siclog "Step 8 - 9" siclog@ 200 OK, ACK f_IMS_MOCallSetup_Send200OK_ReceiveACK(v_InviteRequestWithSdp); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 9"); // @siclog "Step 10 - 14" siclog@ Release Video f_IMS_VideoCall_NR5GC_ReleaseVideoMT(v_LatestSdpAnswerFromSS); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 12"); f_IMS_TestBody_Set(false); // Call release and postamble: f_IMS_CC_PostambleWithCallReleaseMT(IPCAN_MT_SpeechCall); }

After Change:

… … import from IMS_CommonParameters all; … … function f_TC_7_22_NR5GC_IMS1() runs on IMS_PTC { // MTSI MT Voice Call / add video and remove video / with preconditions at both originating UE and terminating UE / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var SDP_Message v_LatestSdpOfferFromUE; var template (value) SDP_Message v_LatestSdpAnswerFromSS; var charstring v_CallIdentification; var boolean v_DontIncrementSessVers := true; var charstring v_ContactUrlStrSS; var SipUrl v_ContactSipUrlSS; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); f_IMS_MTCallSetup_NR5GC_A_5_1(); v_CallIdentification := f_UT_GetCallId(MMI); f_IMS_TestBody_Set(true); f_UT_AddVideoToIMSCall(MMI, v_CallIdentification); // @siclog "Step 1" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_Video_Step2(AddVideo, -, tsc_IMS_SDP_AudioCodecList_EVS_only, tsc_IMS_SDP_VideoCodecList_H265_H264); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 1"); v_ContactSipUrlSS := f_MessageHeader_GetContactSipUrl(v_InviteRequestWithSdp.Invite.msgHeader); v_ContactUrlStrSS := f_SIP_SipUrl2Str(v_ContactSipUrlSS); // @siclog "Step 2 - 3" siclog@ 100 Trying, 183 Session Progress v_ResponseWithSdp := f_IMS_MOCallSetup_Video_Step3_4(AddVideo, v_InviteRequestWithSdp, EVS_16000, H265_90000); // @siclog "Step 4 - 5" siclog@ PRACK, 200 OK f_IMS_MOCallSetup_ReceivePRACK_Send200OK(v_InviteRequestWithSdp, v_ResponseWithSdp.Response.msgHeader); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5A" siclog@ f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MO_AddVideo); // @siclog "Step 6 - 7" siclog@ UPDATE, 200 OK v_LatestSdpOfferFromUE := f_IMS_MOCallSetup_NR5GC_Video_Step6_7(AddVideo, v_InviteRequestWithSdp, f_SIP_BuildSipUri_RX(px_IMS_CalleeContactUri, *)); v_LatestSdpAnswerFromSS := f_IMS_BuildSDP_UpdatedAnswer_TX(v_LatestSdpOfferFromUE, v_DontIncrementSessVers); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); f_IMS_IPCAN_WaitForTrigger(IPCAN); // @siclog "Step 8 - 9" siclog@ 200 OK, ACK f_IMS_MOCallSetup_Send200OK_ReceiveACK(v_InviteRequestWithSdp, v_ContactUrlStrSS); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 9"); // @siclog "Step 10 - 14" siclog@ Release Video f_IMS_VideoCall_NR5GC_ReleaseVideoMT(v_LatestSdpAnswerFromSS); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 12"); f_IMS_TestBody_Set(false); // Call release and postamble: f_IMS_CC_PostambleWithCallReleaseMT(IPCAN_MT_SpeechCall); }

## Change 2

| Function name | f_SIP_BuildSipUri_RX() |
| --- | --- |
| Reason for change | The UE may also include a port number in a URI |
| Summary of change | Changed the default value for port to ‘anyOrOmit’ |
| TTCN module | IMS_SIP_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

function f_SIP_BuildSipUri_RX(charstring p_URI, template integer p_Port := omit, template SemicolonParam_List p_UrlParameters := *, template AmpersandParam_List p_Headers := omit) return template (present) SipUrl { var template (present) SipUrl v_SipUrl := f_SIP_ParseURI(p_URI); if (ischosen(v_SipUrl.components.sip)) { /* @sic R5s130195 BASELINE MOVING 2013: change of SipUrl sic@ @sic R5s140020 change 11: p_Port may be * therefore shall not be checked to be present sic@ */ v_SipUrl.components.sip.hostPort.portField := p_Port; } v_SipUrl.urlParameters := p_UrlParameters; v_SipUrl.headers := p_Headers; /* @sic R5s150382 change 4 sic@ */ return v_SipUrl; }

After Change:

function f_SIP_BuildSipUri_RX(charstring p_URI, template integer p_Port := *, template SemicolonParam_List p_UrlParameters := *, template AmpersandParam_List p_Headers := omit) return template (present) SipUrl { var template (present) SipUrl v_SipUrl := f_SIP_ParseURI(p_URI); if (ischosen(v_SipUrl.components.sip)) { /* @sic R5s130195 BASELINE MOVING 2013: change of SipUrl sic@ @sic R5s140020 change 11: p_Port may be * therefore shall not be checked to be present sic@ */ v_SipUrl.components.sip.hostPort.portField := p_Port; } v_SipUrl.urlParameters := p_UrlParameters; v_SipUrl.headers := p_Headers; /* @sic R5s150382 change 4 sic@ */ return v_SipUrl; }

## Change 3

| Function name | fl_NR5GC_ModifyPDUSession_RemoveIMS_Media() |
| --- | --- |
| Reason for change | Incorrect operation code is being used for the QoS rules IE in the PDU SESSION MODIFICATION COMMAND for removing the video stream |
| Summary of change | Removed the check on v_RemoveVoice so that cs_QoS_DeleteRule template is used Note : An associated prose CR on 38.508-1 will be raised at RAN5#95 |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_NR5GC_ModifyPDUSession_RemoveIMS_Media(NR_CellId_Type p_NR_CellId, ListOfMedia_Type p_MediaType) runs on NR5GC_PTC { … … // @sic R5-216150 sic@ if (v_RemoveVoice) { // @sic R5-217798 sic@ v_QoS_Rules[j] := cs_QoS_DeleteRule('03'O); // @sic R5s210073, R5-213439 sic@ v_QoS_Flows[j] := cs_QoS_DeleteFlow('000111'B); // @sic R5-213439 sic@ j := j+1; } if (v_RemoveVideo) { if (v_RemoveVoice) { v_QoS_Rules[j] := cs_QoS_DeleteRule('04'O); } else { // @sic R5-217798 sic@ FFS: error in prose v_QoS_Rules[j] := cs_QoS_ModifyRule('04'O); } v_QoS_Flows[j] := cs_QoS_DeleteFlow('001000'B); } … … }

After Change:

function fl_NR5GC_ModifyPDUSession_RemoveIMS_Media(NR_CellId_Type p_NR_CellId, ListOfMedia_Type p_MediaType) runs on NR5GC_PTC { … … // @sic R5-216150 sic@ if (v_RemoveVoice) { // @sic R5-217798 sic@ v_QoS_Rules[j] := cs_QoS_DeleteRule('03'O); // @sic R5s210073, R5-213439 sic@ v_QoS_Flows[j] := cs_QoS_DeleteFlow('000111'B); // @sic R5-213439 sic@ j := j+1; } if (v_RemoveVideo) { //REMOVED if (v_RemoveVoice){ v_QoS_Rules[j] := cs_QoS_DeleteRule('04'O); /* REMOVED } else { // @sic R5-217798 sic@ FFS: error in prose v_QoS_Rules[j] := cs_QoS_ModifyRule('04'O); }*/ v_QoS_Flows[j] := cs_QoS_DeleteFlow('001000'B); } … … }