3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220613

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2523 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for EN-DC NR PDCP test case 7.1.3.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-05-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Coordination triggers from EUTRA PTC to NR PTC are required following each reconfiguration step, to trigger NR PTC to start the data path checking steps. - For step 0 when we configure MCG for primary Path , the logical channel id should be 4 instead of 5 for EUTRA Radio bearer - PDCP state variables are updated inside some functions |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add EUTRA->NR coordination triggers before steps 2, 3C and 7 - Modified the Drb_identity in cs_NR_PDCP_Config_UlPath for step 0-0B - Change type of parameter p_NR_PDCP_State to “inout” in some functions so that updated PDCP state is used in subsequent test steps |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.2.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

2.2 Change 2	6

2.3 Change 3	7

2.4 Change 4	10

2.5 Change 5	13

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_7_1_3_5_2_NR_TestBody_ENDC() |
| --- | --- |
| Reason for change | - Coordination triggers from EUTRA PTC to NR PTC are required following each reconfiguration step, to trigger NR PTC to start the data path checking steps. - For step 0 when we configure MCG for primary Path , the logical channel id should be 4 instead of 5 for EUTRA Radio bearer |
| Summary of change | - Add wait for Coordination message before steps 2, 3C and 7 - Modified the Drb_identity in cs_NR_PDCP_Config_UlPath for step 0-0B |
| TTCN module | PDCP_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_7_1_3_5_2_NR_TestBody_ENDC(NR_PDCP_SS_State_Type p_NR_PDCP_State,integer p_DRB_Id) runs on ENDC_NR_PTC {// @sic R5-220641 sic@ var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) DRB_ToAddModList v_DRB_ToAddModList; //@siclog "Step 0A-0B" siclog@ //SS transmits NR RRCReconfiguration message to configure the primary path on the MCG Cell. v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(p_DRB_Id, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits,tsc_NR_CellGroupId_MCG))};//@sic R5s211471 sic@ v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, omit);//@siclog "Step 0B" siclog@ //SCG NR Cell1, Primary path Eutra Cell 1, Secondary path NR Cell 1. f_TC_7_1_3_5_2_NR_CheckULPrimaryPath(p_NR_PDCP_State, cs_RlcBearerRouting_NR(nr_Cell1), crs_RlcBearerRouting_Eutra(eutra_Cell1), cs_RlcBearerRouting_NR(nr_Cell1), "Step2"); //@siclog "Step 2A-2B" siclog@ //SS transmits NR RRCReconfiguration message to configure the primary path on the SCG Cell. v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(p_DRB_Id, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits,tsc_NR_CellGroupId_SCG))};//@sic R5s211471 sic@ v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, omit); //SCG NR Cell 1, Primary path Eutra Cell 1, Secondary path NR Cell 1 f_TC_7_1_3_5_2_NR_CheckULPrimaryPath(p_NR_PDCP_State, cs_RlcBearerRouting_NR(nr_Cell1), cs_RlcBearerRouting_NR(nr_Cell1), crs_RlcBearerRouting_Eutra(eutra_Cell1), "Step2D"); f_TC_7_1_3_5_2_Steps3to4b2(p_NR_PDCP_State, cs_RlcBearerRouting_NR(nr_Cell1), crs_RlcBearerRouting_Eutra(eutra_Cell1)); //@siclog "Step 5-6" siclog@ //SS transmits NR RRCReconfiguration message to configure new split DRB parameters, where the ul-DataSplitThreshold is equal to 0 bytes v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(p_DRB_Id, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits,tsc_NR_CellGroupId_SCG,b0))};//@sic R5s211471 sic@ v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, omit); //SCG NR Cell1, Primary path Eutra Cell 1, Secondary path NR Cell 1. f_TC_7_1_3_5_2_Steps7to8b2(p_NR_PDCP_State, cs_RlcBearerRouting_NR(nr_Cell1), crs_RlcBearerRouting_Eutra(eutra_Cell1)); f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); }

After Change

function fl_TC_7_1_3_5_2_NR_TestBody_ENDC(NR_PDCP_SS_State_Type p_NR_PDCP_State,integer p_DRB_Id) runs on ENDC_NR_PTC {// @sic R5-220641 sic@ var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) DRB_ToAddModList v_DRB_ToAddModList; //@siclog "Step 0A-0B" siclog@ //SS transmits NR RRCReconfiguration message to configure the primary path on the MCG Cell. v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(p_DRB_Id, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits,tsc_NR_CellGroupId_MCG,-,-,-,tsc_NR_DRB1))};//@sic R5s211471 sic@ v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, omit);//@siclog "Step 0B" siclog@ //SCG NR Cell1, Primary path Eutra Cell 1, Secondary path NR Cell 1. f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_TC_7_1_3_5_2_NR_CheckULPrimaryPath(p_NR_PDCP_State, cs_RlcBearerRouting_NR(nr_Cell1), crs_RlcBearerRouting_Eutra(eutra_Cell1), cs_RlcBearerRouting_NR(nr_Cell1), "Step2"); //@siclog "Step 2A-2B" siclog@ //SS transmits NR RRCReconfiguration message to configure the primary path on the SCG Cell. v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(p_DRB_Id, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits,tsc_NR_CellGroupId_SCG))};//@sic R5s211471 sic@ v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, omit); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); //SCG NR Cell 1, Primary path Eutra Cell 1, Secondary path NR Cell 1 f_TC_7_1_3_5_2_NR_CheckULPrimaryPath(p_PDCP_state, cs_RlcBearerRouting_NR(nr_Cell1), cs_RlcBearerRouting_NR(nr_Cell1), crs_RlcBearerRouting_Eutra(eutra_Cell1), "Step2D");//orig f_TC_7_1_3_5_2_Steps3to4b2(p_PDCP_state, cs_RlcBearerRouting_NR(nr_Cell1), crs_RlcBearerRouting_Eutra(eutra_Cell1)); //@siclog "Step 5-6" siclog@ //SS transmits NR RRCReconfiguration message to configure new split DRB parameters, where the ul-DataSplitThreshold is equal to 0 bytes v_DRB_ToAddModList := {cs_38508_DRB_ToAddMod(p_DRB_Id, cs_NR_PDCP_Config_UlPath(ms500, len18bits, len18bits,tsc_NR_CellGroupId_SCG,b0))};//@sic R5s211471 sic@ v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, v_DRB_ToAddModList); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, omit); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); //SCG NR Cell1, Primary path Eutra Cell 1, Secondary path NR Cell 1. f_TC_7_1_3_5_2_Steps7to8b2(p_PDCP_state, cs_RlcBearerRouting_NR(nr_Cell1), crs_RlcBearerRouting_Eutra(eutra_Cell1)); f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); }

## 2.2 Change 2

| Function name | function fl_TC_7_1_3_5_2_EUTRA_TestBody() |
| --- | --- |
| Reason for change | Coordination triggers from EUTRA PTC to NR PTC are required following each reconfiguration step, to trigger NR PTC to start the data path checking steps. |
| Summary of change | Add coordination trigger sending after steps 0B, 3B and 6 |
| TTCN module | PDCP_ENDC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_7_1_3_5_2_EUTRA_TestBody()runs on EUTRA_5GS_PTC { //@siclog "Step 0B" siclog@ f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, RBConfig_Only); //@sic R5-204426 sic@ f_EUTRA_NR_WaitForCoOrd_Trigger(NR); //@siclog "Step 2B" siclog@ f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, RBConfig_Only); //@sic R5-220641 sic@ f_EUTRA_NR_WaitForCoOrd_Trigger(NR); //@siclog "Step 6" siclog@ f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, RBConfig_Only); //@sic R5-220641 sic@ f_EUTRA_NR_WaitForCoOrd_Trigger(NR); }

After Change

function fl_TC_7_1_3_5_2_EUTRA_TestBody()runs on EUTRA_5GS_PTC { //@siclog "Step 0B" siclog@ f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, RBConfig_Only); //@sic R5-204426 sic@ f_EUTRA_NR_SendCoOrd(NR,cms_EUTRA_NR_Trigger); //@siclog "Step 2B" siclog@ f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, RBConfig_Only); //@sic R5-220641 sic@ f_EUTRA_NR_SendCoOrd(NR,cms_EUTRA_NR_Trigger); //@siclog "Step 6" siclog@ f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, RBConfig_Only); //@sic R5-220641 sic@ f_EUTRA_NR_SendCoOrd(NR,cms_EUTRA_NR_Trigger); f_EUTRA_NR_WaitForCoOrd_Trigger(NR); }

## 2.3 Change 3

| Function name | f_TC_7_1_3_5_2_NR_CheckULPrimaryPath() |
| --- | --- |
| Reason for change | PDCP state variables are updated inside the function |
| Summary of change | Change type of parameter p_NR_PDCP_State to “inout” so that updated PDCP state is used in subsequent test steps |
| TTCN module | PDCP_ENDC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_3_5_2_NR_CheckULPrimaryPath(NR_PDCP_SS_State_Type p_NR_PDCP_State, template (value) RlcBearerRouting_Type p_SCG_RoutingRlcRouting, template (value) RlcBearerRouting_Type p_PrimaryPath, template (value) RlcBearerRouting_Type p_SecondaryPath, charstring p_TestStep) runs on NR_BASE_PTC {// @sic R5-220641 sic@ var NR_PDCP_SDUList_Type v_SduList; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu; timer t_WaitForPdu:= 2.0; v_SduList := f_GenerateSDU (p_NR_PDCP_State, {64, 164}); //@siclog "Step 1 or 3C" siclog@ //The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell Data PDU = 64 bytes. f_NR_PDCP_Send_SCGCell(p_NR_PDCP_State,nr_Cell1,v_SduList[0], p_SCG_RoutingRlcRouting); v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT, v_SduList[0]); t_WaitForPdu.start; alt{ [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id , p_PrimaryPath, -, {v_ExpectedPdu})) { //@siclog "Step 2 or 3D" siclog@ Does UE transmit a PDCP Data PDU on the AM RLC entity configured for SCG on NR Cell 1 (PSCell)? f_NR_PreliminaryPass(__FILE__, __LINE__, p_TestStep); p_NR_PDCP_State.RX_NEXT := p_NR_PDCP_State.RX_NEXT + 1; t_WaitForPdu.stop; } [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id , p_SecondaryPath, -, {v_ExpectedPdu})) { //Does UE transmit a PDCP Data PDU on the AM RLC secondary entity in next two seconds? f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestStep); } [] t_WaitForPdu.timeout{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestStep); } } if(p_TestStep =="3D")//@siclog "Step 3E" siclog@ { //The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell.Data PDU = 164 bytes. f_NR_PDCP_Send_SCGCell(p_NR_PDCP_State,nr_Cell1,v_SduList[1], p_SCG_RoutingRlcRouting); } }

After Change

function f_TC_7_1_3_5_2_NR_CheckULPrimaryPath(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, template (value) RlcBearerRouting_Type p_SCG_RoutingRlcRouting, template (value) RlcBearerRouting_Type p_PrimaryPath, template (value) RlcBearerRouting_Type p_SecondaryPath, charstring p_TestStep) runs on NR_BASE_PTC {// @sic R5-220641 sic@ var NR_PDCP_SDUList_Type v_SduList; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu; timer t_WaitForPdu:= 2.0; v_SduList := f_GenerateSDU (p_NR_PDCP_State, {64, 164}); //@siclog "Step 1 or 3C" siclog@ //The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell Data PDU = 64 bytes. f_NR_PDCP_Send_SCGCell(p_NR_PDCP_State,nr_Cell1,v_SduList[0], p_SCG_RoutingRlcRouting); v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT, v_SduList[0]); t_WaitForPdu.start; alt{ [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id , p_PrimaryPath, -, {v_ExpectedPdu})) { //@siclog "Step 2 or 3D" siclog@ Does UE transmit a PDCP Data PDU on the AM RLC entity configured for SCG on NR Cell 1 (PSCell)? f_NR_PreliminaryPass(__FILE__, __LINE__, p_TestStep); p_NR_PDCP_State.RX_NEXT := p_NR_PDCP_State.RX_NEXT + 1; t_WaitForPdu.stop; } [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id , p_SecondaryPath, -, {v_ExpectedPdu})) { //Does UE transmit a PDCP Data PDU on the AM RLC secondary entity in next two seconds? f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestStep); } [] t_WaitForPdu.timeout{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestStep); } } if(p_TestStep =="3D")//@siclog "Step 3E" siclog@ { //The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell.Data PDU = 164 bytes. f_NR_PDCP_Send_SCGCell(p_NR_PDCP_State,nr_Cell1,v_SduList[1], p_SCG_RoutingRlcRouting); } }

## 2.4 Change 4

| Function name | f_TC_7_1_3_5_2_Steps3to4b2() |
| --- | --- |
| Reason for change | PDCP state variables are updated inside the function |
| Summary of change | Change type of parameter p_NR_PDCP_State to “inout” so that updated PDCP state is used in subsequent test steps |
| TTCN module | PDCP_ENDC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_3_5_2_Steps3to4b2(NR_PDCP_SS_State_Type p_NR_PDCP_State, template (value) RlcBearerRouting_Type p_PrimaryPath, template (value) RlcBearerRouting_Type p_SecondaryPath ) runs on NR_BASE_PTC { var NR_PDCP_SDUList_Type v_SduList; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu; var boolean v_ReceivedPdu := false; timer t_WaitForPdu:= 2.0; v_SduList := f_GenerateSDU (p_NR_PDCP_State, {164}); //@siclog "Step 3" siclog@ //The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell. Data PDU=164 bytes. f_NR_PDCP_Send_SCGCell(p_NR_PDCP_State,nr_Cell1,v_SduList[0], p_PrimaryPath); //Primary path is configured on SCG @sic R5-220641 sic@ v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT,v_SduList[0]); t_WaitForPdu.start; alt{ //@siclog "Step 4a1-4b2" siclog@ [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id , p_PrimaryPath, -, {v_ExpectedPdu})) {//Does UE transmit a PDCP Data PDU on the AM RLC primary entity? if (v_ReceivedPdu){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4a1"); }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4a1"); p_NR_PDCP_State.RX_NEXT := p_NR_PDCP_State.RX_NEXT + 1; //@sic R5s210275 sic@ v_ReceivedPdu := true; } repeat; } [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id , p_SecondaryPath, -, {v_ExpectedPdu})) //@sic R5s190401 sic@ { //Does UE transmit a PDCP Data PDU on the AM RLC secondary entity? if (v_ReceivedPdu){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4b1"); }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4b2"); p_NR_PDCP_State.RX_NEXT := p_NR_PDCP_State.RX_NEXT + 1; //@sic R5s210275 sic@ v_ReceivedPdu := true; } repeat; } [] t_WaitForPdu.timeout{ if (v_ReceivedPdu){//@sic R5s210065 sic@ v_ReceivedPdu := false } else{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4a1-4b2"); } } } }

After Change

function f_TC_7_1_3_5_2_Steps3to4b2(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, template (value) RlcBearerRouting_Type p_PrimaryPath, template (value) RlcBearerRouting_Type p_SecondaryPath ) runs on NR_BASE_PTC { var NR_PDCP_SDUList_Type v_SduList; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu; var boolean v_ReceivedPdu := false; timer t_WaitForPdu:= 2.0; v_SduList := f_GenerateSDU (p_NR_PDCP_State, {164}); //@siclog "Step 3" siclog@ //The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell. Data PDU=164 bytes. f_NR_PDCP_Send_SCGCell(p_NR_PDCP_State,nr_Cell1,v_SduList[0], p_PrimaryPath); //Primary path is configured on SCG @sic R5-220641 sic@ v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT,v_SduList[0]); t_WaitForPdu.start; alt{ //@siclog "Step 4a1-4b2" siclog@ [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id , p_PrimaryPath, -, {v_ExpectedPdu})) {//Does UE transmit a PDCP Data PDU on the AM RLC primary entity? if (v_ReceivedPdu){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4a1"); }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4a1"); p_NR_PDCP_State.RX_NEXT := p_NR_PDCP_State.RX_NEXT + 1; //@sic R5s210275 sic@ v_ReceivedPdu := true; } repeat; } [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id , p_SecondaryPath, -, {v_ExpectedPdu})) //@sic R5s190401 sic@ { //Does UE transmit a PDCP Data PDU on the AM RLC secondary entity? if (v_ReceivedPdu){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4b1"); }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4b2"); p_NR_PDCP_State.RX_NEXT := p_NR_PDCP_State.RX_NEXT + 1; //@sic R5s210275 sic@ v_ReceivedPdu := true; } repeat; } [] t_WaitForPdu.timeout{ if (v_ReceivedPdu){//@sic R5s210065 sic@ v_ReceivedPdu := false } else{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4a1-4b2"); } } } }

## 2.5 Change 5

| Function name | f_TC_7_1_3_5_2_Steps7to8b2 () |
| --- | --- |
| Reason for change | PDCP state variables are updated inside the function |
| Summary of change | Change type of parameter p_NR_PDCP_State to “inout” so that updated PDCP state is used in subsequent test steps |
| TTCN module | PDCP_ENDC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_3_5_2_Steps7to8b2(NR_PDCP_SS_State_Type p_NR_PDCP_State, template (value) RlcBearerRouting_Type p_PrimaryPath, template (value) RlcBearerRouting_Type p_SecondaryPath ) runs on NR_BASE_PTC { var boolean v_ReceivedPdu := false; var NR_PDCP_SDUList_Type v_SduList; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu; timer t_WaitForPdu:= 2.0; v_SduList := f_GenerateSDU (p_NR_PDCP_State, {64}); //@siclog "Step 7" siclog@ //The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell. Data PDU = 64 bytes @sic R5s210065 sic@ f_NR_PDCP_Send_SCGCell(p_NR_PDCP_State,nr_Cell1,v_SduList[0], p_PrimaryPath);//Primary path is configured on SCG @sic R5-220641 sic@ v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT); t_WaitForPdu.start; alt{ //@siclog "Step 8a1-8b2" siclog@ [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id , p_PrimaryPath, -, {v_ExpectedPdu})) {//Does UE transmit a PDCP Data PDU on the AM RLC primary entity? if (v_ReceivedPdu){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8a1"); }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8a1"); v_ReceivedPdu := true; } repeat; } [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id , p_SecondaryPath, -, {v_ExpectedPdu})) { //Does UE transmit a PDCP Data PDU on the AM RLC secondary entity ? if (v_ReceivedPdu){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8b1"); }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8b2"); v_ReceivedPdu := true; } repeat; } [] t_WaitForPdu.timeout{ if (v_ReceivedPdu){ v_ReceivedPdu := false } else{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8a1-8b2"); } } } }

After Change

function f_TC_7_1_3_5_2_Steps7to8b2(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, template (value) RlcBearerRouting_Type p_PrimaryPath, template (value) RlcBearerRouting_Type p_SecondaryPath ) runs on NR_BASE_PTC { var boolean v_ReceivedPdu := false; var NR_PDCP_SDUList_Type v_SduList; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu; timer t_WaitForPdu:= 2.0; v_SduList := f_GenerateSDU (p_NR_PDCP_State, {64}); //@siclog "Step 7" siclog@ //The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell. Data PDU = 64 bytes @sic R5s210065 sic@ f_NR_PDCP_Send_SCGCell(p_NR_PDCP_State,nr_Cell1,v_SduList[0], p_PrimaryPath);//Primary path is configured on SCG @sic R5-220641 sic@ v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT); t_WaitForPdu.start; alt{ //@siclog "Step 8a1-8b2" siclog@ [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id , p_PrimaryPath, -, {v_ExpectedPdu})) {//Does UE transmit a PDCP Data PDU on the AM RLC primary entity? if (v_ReceivedPdu){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8a1"); }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8a1"); v_ReceivedPdu := true; } repeat; } [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id , p_SecondaryPath, -, {v_ExpectedPdu})) { //Does UE transmit a PDCP Data PDU on the AM RLC secondary entity ? if (v_ReceivedPdu){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8b1"); }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8b2"); v_ReceivedPdu := true; } repeat; } [] t_WaitForPdu.timeout{ if (v_ReceivedPdu){ v_ReceivedPdu := false } else{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8a1-8b2"); } } } }