3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220628

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2322 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR test case 7.1.1.3.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In test case 7.1.1.3.7, for FR2 120Khz, based on the UL-DL pattern configuration and K2=4, it is recommended to choose subframe 5, slot 0 for the UL grant scheduling to expect UL Data on subframe 5, slot 4 so that TTCN can control the subframe where the UL Grant is transmitted Similarly, for FR1 30Khz, based on the UL-DL pattern configuration and K2=4, it is recommended to choose subframe 2, slot 0 for the UL grant scheduling to expect the UL Data on subframe 4, slot 0 so that TTCN can control the subframe where the UL Grant is transmitted - In the 7.1.1.3.7 ENDC, TP1 should be tested accurately as per Prose where Steps 3 and 4 can happen in any order. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Changed the subframe to 2 when SCS 30 is configured Changed the subframe to 5 when SCS 120 is configured - Introduced the alt mechanism at Steps 3 and 4 to handle the MAC PDU containing Power Headroom MAC Control Element and RRCReconfigurationComplete as done similarly to Steps 10 and 11 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC implementation will FAIL conformant UEs when testing with SCS 30KHz and SCS 120 Khz and TP1 shall not be tested correctly in 7.1.1.3.7 ENDC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.7.ENDC, 7.1.1.3.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required for test case 7.1.1.3.7	3

1.1	Correction to f_TC_7_1_1_3_7_NR5GC	3

1.2	Correction to f_TC_7_1_1_3_7_ENDC	5

# Corrections required for test case 7.1.1.3.7

## Correction to f_TC_7_1_1_3_7_NR5GC

| Function name | f_TC_7_1_1_3_7_NR5GC |
| --- | --- |
| Reason for change | In test case 7.1.1.3.7, for FR2 120Khz, based on the UL-DL pattern configuration and K2=4, it is recommended to choose subframe 5, slot 0 for the UL grant scheduling to expect UL Data on subframe 5, slot 4 so that TTCN can control the subframe where the UL Grant is transmitted Similarly, for FR1 30Khz, based on the UL-DL pattern configuration and K2=4, it is recommended to choose subframe 2, slot 0 for the UL grant scheduling to expect the UL Data on subframe 4, slot 0 so that TTCN can control the subframe where the UL Grant is transmitted. |
| Summary of change | Changed the subframe to 2 when SCS 30 is configured Changed the subframe to 5 when SCS 120 is configured |
| TTCN module | MAC_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_3_7_NR5GC() runs on NR5GC_PTC { //UE power headroom reporting / Periodic reporting / DL pathloss change reporting <<SKIPPED CODE>> //@sic R5s190767 sic@ var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP ; <<SKIPPED CODE>> //@siclog "Steps 1" siclog@ v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.phr_Config := cs_38508_PHR_Config_Setup(cs_PHR_Config_7137(false, sf500)); //@sic R5-197026 sic@ f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG, omit, omit, v_MAC_CellGroupConfig); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfig))); t_Watchdog.start; //The SS transmits UL grant to the UE at every 10ms in PDCCH occasion. f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation, 10); //@siclog "Steps 2" siclog@ f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,-,false); <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_3_7_NR5GC() runs on NR5GC_PTC { //UE power headroom reporting / Periodic reporting / DL pathloss change reporting <<SKIPPED CODE>> //@sic R5s190767 sic@ var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP ; var SubcarrierSpacing v_SCS; //WA# <<SKIPPED CODE>> //@siclog "Steps 1" siclog@ v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.phr_Config := cs_38508_PHR_Config_Setup(cs_PHR_Config_7137(false, sf500)); //@sic R5-197026 sic@ f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG, omit, omit, v_MAC_CellGroupConfig); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfig))); t_Watchdog.start; //The SS transmits UL grant to the UE at every 10ms in PDCCH occasion. v_SCS:= f_NR_CellInfo_GetSCS(nr_Cell1); //WA# if (v_SCS == kHz30) { f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_SubFrame(2), v_NR_ResourceAllocation, 10); } else { f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_SubFrame(5), v_NR_ResourceAllocation, 10); } //@siclog "Steps 2" siclog@ f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,-,false); <<SKIPPED CODE>>

## Correction to f_TC_7_1_1_3_7_ENDC

| Function name | f_TC_7_1_1_3_7_ENDC |
| --- | --- |
| Reason for change | - In test case 7.1.1.3.7, for FR2 120Khz, based on the UL-DL pattern configuration and K2=4, it is recommended to choose subframe 5, slot 0 for the UL grant scheduling to expect UL Data on subframe 5, slot 4 so that TTCN can control the subframe where the UL Grant is transmitted Similarly, for FR1 30Khz, based on the UL-DL pattern configuration and K2=4, it is recommended to choose subframe 2, slot 0 for the UL grant scheduling to expect the UL Data on subframe 4, slot 0 so that TTCN can control the subframe where the UL Grant is transmitted. - In the 7.1.1.3.7 ENDC, TP1 should be tested accurately as per Prose where Steps 3 and 4 can happen in any order. |
| Summary of change | - Changed the subframe to 2 when SCS 30 is configured Changed the subframe to 5 when SCS 120 is configured - Introduced the alt mechanism at Steps 3 and 4 to handle the MAC PDU containing Power Headroom MAC Control Element and RRCReconfigurationComplete as done similarly to Steps 10 and 11 |
| TTCN module | MAC_ENDC_NR |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_3_7_ENDC_NR() runs on ENDC_NR_PTC { //UE power headroom reporting / Periodic reporting / DL pathloss change reporting <<SKIPPED CODE>> var EUTRA_NR_Coordination_MSG v_ReceivedMsg; var boolean v_RRC_MsgRcvd := false; var boolean v_PHR_Rcvd := false; <<SKIPPED CODE>> //@siclog "Steps 2&4" siclog@ v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.phr_Config := cs_38508_PHR_Config_Setup(cs_PHR_Config_7137(-, sf500)); f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG, omit, omit, v_MAC_CellGroupConfig); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig); // Now send octetstrings to EUTRA //The SS transmits UL grant to the UE at every 10ms in PDCCH occasion. v_TimingUL := f_NR_GetNextSendOccasion_UL(nr_Cell1, 100); //@sic R5s191093 sic@ f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 10); //@siclog "Steps 3" siclog@ SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(nr_Cell1, cr_NR_MAC_CE_MultiEntryPHR(?)))->value v_NR_SYSTEM_IND; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); v_TimingInfo1 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; <<SKIPPED CODE>> //@siclog "Steps 9& 11" siclog@ v_MAC_CellGroupConfig.phr_Config := cs_38508_PHR_Config_Setup(cs_PHR_Config_7137(-,-,dB3)); f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG, omit, omit, v_MAC_CellGroupConfig); // PHR Prohibit Timer v_TimerMax := f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, 1.0); v_TimerMin := f_NR_SetTimerToleranceMin (nr_Cell1, l2Timer, 1.0); t_Watchdog.start; f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig,-,-,false); // Now send octetstrings to EUTRA <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_3_7_ENDC_NR() runs on ENDC_NR_PTC { //UE power headroom reporting / Periodic reporting / DL pathloss change reporting <<SKIPPED CODE>> var EUTRA_NR_Coordination_MSG v_ReceivedMsg; var boolean v_RRC_MsgRcvd := false; var boolean v_PHR_Rcvd := false; var SubcarrierSpacing v_SCS; //WA# <<SKIPPED CODE>> //@siclog "Steps 2&4" siclog@ v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.phr_Config := cs_38508_PHR_Config_Setup(cs_PHR_Config_7137(-, sf500)); f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG, omit, omit, v_MAC_CellGroupConfig); //The SS transmits UL grant to the UE at every 10ms in PDCCH occasion. v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); //WA# if (v_SCS == kHz30) { f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_SubFrame(2), v_NR_ResourceAllocation, 10); } else { f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_SubFrame(5), v_NR_ResourceAllocation, 10); } f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig,-,-,false); // Now send octetstrings to EUTRA WA# //@siclog "Steps 3-4" siclog@ alt { [not(v_RRC_MsgRcvd)] EUTRA.receive(cmr_EUTRA_NR_OctetData ("Complete")) -> value v_ReceivedMsg { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); f_NR_CheckReceived_RRCReconfigurationComplete(valueof(v_ReceivedMsg)); v_RRC_MsgRcvd := true; if (not(v_PHR_Rcvd)) { repeat; } } // The UE transmits a MAC PDU containing Power Headroom MAC Control Element [not(v_PHR_Rcvd)] SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(nr_Cell1, cr_NR_MAC_CE_MultiEntryPHR(?)))->value v_NR_SYSTEM_IND { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); v_TimingInfo1 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; v_PHR_Rcvd := true; if (not(v_RRC_MsgRcvd)) { repeat; } } }; <<SKIPPED CODE>> //@siclog "Steps 9& 11" siclog@ v_MAC_CellGroupConfig.phr_Config := cs_38508_PHR_Config_Setup(cs_PHR_Config_7137(-,-,dB3)); f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG, omit, omit, v_MAC_CellGroupConfig); // PHR Prohibit Timer v_TimerMax := f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, 1.0); v_TimerMin := f_NR_SetTimerToleranceMin (nr_Cell1, l2Timer, 1.0); //WA# v_RRC_MsgRcvd := false; v_PHR_Rcvd := false; t_Watchdog.start; f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig,-,-,false); // Now send octetstrings to EUTRA <<SKIPPED CODE>>