3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220647

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2409 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 8.1.4.1.9.3 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Addition of NR5GC test case 8.1.4.1.9.3 in FR1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC test case 8.1.4.1.9.3 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.9.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS38.508-1，R5-221389 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1 Overview	3

1.1 Verification Test Summary	3

2 Corrections required	3

3 Branches executed	3

4 Execution Log Files	3

4.1 HiSilicon Balong 5000	3

5 References	3

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.4.1.9.3 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Ting Gao
	gaoting@starpointcomm.com

## Verification Test Summary

Test Case: 	8.1.4.1.9.3

ATS Version:	iwd-TTCN3-B2020-09_D12wk49

System Simulator used:	Starpoint SP9500 Test System

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_NR_GetCA_IntraBandFrequency |
| --- | --- |
| Reason for change | UE supporting CA_n78_2A can not be tested. |
| Summary of change | Add case CA_n78_2A, prose cr R5-221389 is submitted to RAN5#94e. |
| TTCN module | NR_CA_FrequencyFunctions.ttcn |
| MCC160 Comment |  |

Before change

function f_NR_GetCA_IntraBandFrequency ( NR_CA_BandCombination_Type p_NR_CA_BandCombination_Type ) return template(value) NR_Frequencyf1tof4_Type

{

var template(value) NR_Frequencyf1tof4_Type v_NR_Frequencyf1tof4 := cs_NR_Frequencyf1tof4_Dummy;

var FreqBandIndicatorNR v_FreqBandIndicatorNR := f_NR_GetCA_PrimaryBand(p_NR_CA_BandCombination_Type);

var P_Max v_P_Max := f_NR_InitP_Max();

...

case( CA_n71_2A ) {

//@sic R5-214929, R5-215970 sic@

v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 123890 , v_FreqBandIndicatorNR , 123464 , 0 , kHz15 , 52 ), 10 , kHz15 , 10 , 0 , 1 , 1 , 0 );

v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ v_FreqBandIndicatorNR }, 132664 , 0 , kHz15 , 52 , v_P_Max), 10 );

v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 128890 , v_FreqBandIndicatorNR , 110320 , 504 , kHz15 , 52 ), 10 , kHz15 , 10 , 0 , 505 , 1 , 0 );

v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ v_FreqBandIndicatorNR }, 137448 , 6 , kHz15 , 52 , v_P_Max), 10 );

}

case( CA_n261_2A ) {

//@sic R5-206306, R5-206861 sic@

v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 2071387 , v_FreqBandIndicatorNR , 2070875 , 0 , kHz120 , 132 ), 100 , kHz120 , 8 , 1 , 22 , 14 , 8 );

v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz120 , 132 , v_P_Max), 100 );

v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 2083771 , v_FreqBandIndicatorNR , 2077325 , 504 , kHz120 , 132 ), 100 , kHz120 , 2 , 1 , 517 , 5 , 8 );

v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 504 , kHz120 , 132 , v_P_Max), 100 );

}

case else { FatalError (__FILE__, __LINE__, "Invalid NR-SA band combination") }

}

return v_NR_Frequencyf1tof4;

}

After change

function f_NR_GetCA_IntraBandFrequency ( NR_CA_BandCombination_Type p_NR_CA_BandCombination_Type ) return template(value) NR_Frequencyf1tof4_Type

{

var template(value) NR_Frequencyf1tof4_Type v_NR_Frequencyf1tof4 := cs_NR_Frequencyf1tof4_Dummy;

var FreqBandIndicatorNR v_FreqBandIndicatorNR := f_NR_GetCA_PrimaryBand(p_NR_CA_BandCombination_Type);

var P_Max v_P_Max := f_NR_InitP_Max();

...

case( CA_n71_2A ) {

//@sic R5-214929, R5-215970 sic@

v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 123890 , v_FreqBandIndicatorNR , 123464 , 0 , kHz15 , 52 ), 10 , kHz15 , 10 , 0 , 1 , 1 , 0 );

v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ v_FreqBandIndicatorNR }, 132664 , 0 , kHz15 , 52 , v_P_Max), 10 );

v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 128890 , v_FreqBandIndicatorNR , 110320 , 504 , kHz15 , 52 ), 10 , kHz15 , 10 , 0 , 505 , 1 , 0 );

v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ v_FreqBandIndicatorNR }, 137448 , 6 , kHz15 , 52 , v_P_Max), 10 );

}

case( CA_n78_2A ) {//50MHz+50MHz

v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 620352 , v_FreqBandIndicatorNR , 620072 , 0 , kHz30 , 133 ), 50 , kHz30 , 16 , 1 , 2 , 0 , 1 );

v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz30 , 133 , v_P_Max), 50 );

v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 650400 , v_FreqBandIndicatorNR , 637974 , 504 , kHz30 , 133 ), 50 , kHz30 , 18 , 3 , 1014 , 0 , 3 );

v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 504 , kHz30 , 133 , v_P_Max), 50 );

}

case( CA_n261_2A ) {

//@sic R5-206306, R5-206861 sic@

v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 2071387 , v_FreqBandIndicatorNR , 2070875 , 0 , kHz120 , 132 ), 100 , kHz120 , 8 , 1 , 22 , 14 , 8 );

v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz120 , 132 , v_P_Max), 100 );

v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 2083771 , v_FreqBandIndicatorNR , 2077325 , 504 , kHz120 , 132 ), 100 , kHz120 , 2 , 1 , 517 , 5 , 8 );

v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 504 , kHz120 , 132 , v_P_Max), 100 );

}

case else { FatalError (__FILE__, __LINE__, "Invalid NR-SA band combination") }

}

return v_NR_Frequencyf1tof4;

}

# Branches executed

The test case was executed on NR band CA_n78_2A using NR ciphering nea2 and integrity algorithm nia2.

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000passed this test case on Starpoint SP9500 Test System. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test Case Execution log file: 
\TC_8_1_4_1_9_3_NR5GC_PASS.spm

- PIXCIT settings used:

\PCT_PICS_PIXIT.sppar

# References

| [1] | R5s220369: Supporting information for agreement of NR5GC test case 8.1.4.1.9.3 in FR1. |
| --- | --- |