3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220804

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2615 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR5GC_EmergencyCallLimitedService |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of testcase 11.4.5, the function f_NR5GC_EmergencyCallLimitedService f_NR5GC_EmergencyCallPDUSession is called which establishes a PDU Session for Emergency. After this the NR PTC sends a coordination message to IMS PTC so that procedure for IMS Emergency Voice call (A.6) is performed. Once this is done, the IMS PTC sends a coordination message to the NR PTC to add voice. After this, the IMS PTC waits for a coordination message from the NR PTC to release the call : f_IMS_EmergencyCallAndReleaseWithoutRegistration f_IMS_IPCAN_WaitForTrigger(IPCAN); Once the NR PTC receives the coordination message from IMS PTC, the function f_NR5GC_ModifyPDUSession_AddVoice is called to add voice (f_NR5GC_EmergencyCallLimitedService f_NR5GC_EmergencyCallPDUSession f_NR5GC_ModifyPDUSession_AddVoice), and then then a coordination message is expected from IMS PTC : f_NR5GC_EmergencyCallLimitedService f_NR5GC_EmergencyCallPDUSession f_NR5GC_ModifyPDUSession_AddVoice f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); At this stage, both, the NR PTC and IMS PTC are waiting for a coordination message and the testcase gets stuck. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | After the NR PTC has added voice by calling function f_NR5GC_ModifyPDUSession_AddVoice, it should send a coordination message to IMS PTC, rather than wait for a coordination message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Thus, instead of calling f_NR5GC_EmergencyCallLimitedService f_NR5GC_EmergencyCallPDUSession f_NR5GC_ModifyPDUSession_AddVoice f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.5.NR5GC and other NR5GC emergency call testcases. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required for function f_NR5GC_EmergencyCallLimitedService	3

2.1	Correction to f_NR5GC_EmergencyCallLimitedService	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required for function f_NR5GC_EmergencyCallLimitedService

## Correction to f_NR5GC_EmergencyCallLimitedService

| Template name | f_NR5GC_EmergencyCallLimitedService |
| --- | --- |
| Reason for change | In the current TTCN implementation of testcase 11.4.5, the function f_NR5GC_EmergencyCallLimitedService f_NR5GC_EmergencyCallPDUSession is called which establishes a PDU Session for Emergency. After this the NR PTC sends a coordination message to IMS PTC so that procedure for IMS Emergency Voice call (A.6) is performed. Once this is done, the IMS PTC sends a coordination message to the NR PTC to add voice. After this, the IMS PTC waits for a coordination message from the NR PTC to release the call : f_IMS_EmergencyCallAndReleaseWithoutRegistration f_IMS_IPCAN_WaitForTrigger(IPCAN); Once the NR PTC receives the coordination message from IMS PTC, the function f_NR5GC_ModifyPDUSession_AddVoice is called to add voice (f_NR5GC_EmergencyCallLimitedService f_NR5GC_EmergencyCallPDUSession f_NR5GC_ModifyPDUSession_AddVoice), and then then a coordination message is expected from IMS PTC : f_NR5GC_EmergencyCallLimitedService f_NR5GC_EmergencyCallPDUSession f_NR5GC_ModifyPDUSession_AddVoice f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); At this stage, both, the NR PTC and IMS PTC are waiting for a coordination message and the testcase gets stuck. This needs to be addressed. |
| Summary of change | After the NR PTC has added voice by calling function f_NR5GC_ModifyPDUSession_AddVoice, it should send a coordination message to IMS PTC, rather than wait for a coordination message. Thus, instead of calling f_NR5GC_EmergencyCallLimitedService f_NR5GC_EmergencyCallPDUSession f_NR5GC_ModifyPDUSession_AddVoice f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); Following should be called : f_NR5GC_EmergencyCallLimitedService f_NR5GC_EmergencyCallPDUSession f_NR5GC_ModifyPDUSession_AddVoice f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_NR5GC_EmergencyCallLimitedService(NR_CellId_Type p_NR_CellId, boolean p_IncludeTAIList := true) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams; var NR_SecurityParams_Type v_RRC_SecurityParams; var template NAS_SecurityReturnType v_NAS_SecurityReturnValue; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (p_NR_CellId); var template (omit) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); if (not p_IncludeTAIList) { // @sic R5s211510 sic@ v_TAIList := omit; } // Steps 1-3 RRC connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, emergency, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Emergency)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Registration Request"); } // Store GMM Mobility Info now f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); // Set all algorithms to 0 v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_RRC_SecurityParams := f_NR_Security_Get(); v_NAS_SecurityParams.NAS_Integrity.Algorithm := '0000'B; // IA0 v_NAS_SecurityParams.NAS_Ciphering.Algorithm := '0000'B; // EA0 v_NAS_SecurityParams.KSIamf := '000'B; f_NR5GC_Security_Set(v_NAS_SecurityParams); v_RRC_SecurityParams.AS_Integrity.Algorithm := nia0; v_RRC_SecurityParams.AS_Ciphering.Algorithm := nea0; f_NR_Security_Set(v_RRC_SecurityParams); //Steps 4-5 v_NAS_SecurityReturnValue := f_NR_NASSecurityModeProcedure(p_NR_CellId, v_NAS_SecurityParams, Initial_NoSecurity, v_GMM_MobilityInfo); // @sic R5s210518 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Step 5"); //Steps 6-7 f_NR_RRC_ActivateSecurity (p_NR_CellId, valueof(v_NAS_SecurityReturnValue.NasCount)); // Use NasCount returned from NAS S.M. Complete f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Step 7"); //Steps 8-9 f_NR_Capability(p_NR_CellId); //Step 10 SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, // @sic R5-206426 sic@ v_GUTIToSend, v_TAIList)))); // Step 11 f_NR5GC_Registration_Complete(p_NR_CellId); // Steps 13-18 f_NR5GC_EmergencyCallPDUSession(p_NR_CellId, tsc_NR_RbId_SRB1); // @sic R5s210518 sic@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); // Emergency call now active f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Established"); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_NR5GC_EmergencyCallLimitedService(NR_CellId_Type p_NR_CellId, boolean p_IncludeTAIList := true) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams; var NR_SecurityParams_Type v_RRC_SecurityParams; var template NAS_SecurityReturnType v_NAS_SecurityReturnValue; var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (p_NR_CellId); var template (omit) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); if (not p_IncludeTAIList) { // @sic R5s211510 sic@ v_TAIList := omit; } // Steps 1-3 RRC connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, emergency, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Emergency)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Registration Request"); } // Store GMM Mobility Info now f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); // Set all algorithms to 0 v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_RRC_SecurityParams := f_NR_Security_Get(); v_NAS_SecurityParams.NAS_Integrity.Algorithm := '0000'B; // IA0 v_NAS_SecurityParams.NAS_Ciphering.Algorithm := '0000'B; // EA0 v_NAS_SecurityParams.KSIamf := '000'B; f_NR5GC_Security_Set(v_NAS_SecurityParams); v_RRC_SecurityParams.AS_Integrity.Algorithm := nia0; v_RRC_SecurityParams.AS_Ciphering.Algorithm := nea0; f_NR_Security_Set(v_RRC_SecurityParams); //Steps 4-5 v_NAS_SecurityReturnValue := f_NR_NASSecurityModeProcedure(p_NR_CellId, v_NAS_SecurityParams, Initial_NoSecurity, v_GMM_MobilityInfo); // @sic R5s210518 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Step 5"); //Steps 6-7 f_NR_RRC_ActivateSecurity (p_NR_CellId, valueof(v_NAS_SecurityReturnValue.NasCount)); // Use NasCount returned from NAS S.M. Complete f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Step 7"); //Steps 8-9 f_NR_Capability(p_NR_CellId); //Step 10 SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, // @sic R5-206426 sic@ v_GUTIToSend, v_TAIList)))); // Step 11 f_NR5GC_Registration_Complete(p_NR_CellId); // Steps 13-18 f_NR5GC_EmergencyCallPDUSession(p_NR_CellId, tsc_NR_RbId_SRB1); // @sic R5s210518 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]);//WA#11_4_5 //WA#11_4_5 COMMENTED OUT f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); // Emergency call now active f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Established"); } <<SKIPPED CODE>>