3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220838

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2643 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 7.1.1.9.1.ENDC test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Starting of the UL grant after step 3 is probably remnant of the old test prose. In the latest version of 38.523-1 the UL grant should be disabled all the way till step 8. Truly, the grant is disabled shortly after this, but it shouldn’t be started here in the first place. So this change improves stability of the test. - Step 5A dictates to ignore SRs, but they are ignored only after step 7 which may be too late and while performing SS configurations an SR may come thus leading to test case failure. This is to increase stability of the test. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed starting the UL grant after step 3 - Activating handler to ignore SR at the beginning of the f_TC_7_1_1_9_1_Step5to9A and deactivating it again prior reception of SR in step 7.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Changes suggested in this document improve stability of the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.9.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | The “before change” already contain R5s220805 which is still subject of MCC review. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

2.2	Change 2	7

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_TC_7_1_1_9_1_ENDC_TestBody |
| --- | --- |
| Reason for change | - Starting of the UL grant after step 3 is probably remnant of the old test prose. In the latest version of 38.523-1 the UL grant should be disabled all the way till step 8. Truly, the grant is disabled shortly after this, but it shouldn’t be started her in the first place. So this change improves stability of the test. |
| Summary of change | - Removed starting the UL grant after step 3 |
| TTCN module | MAC_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_7_1_1_9_1_ENDC_TestBody(DRB_Identity p_NR_DRB_Id) runs on ENDC_NR_PTC { var B6_Type v_LCID := tsc_LCID_SCG_Bearer; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := f_NR_UL_AM_T_PollRetransmit(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var RLC_Config.am.dl_AM_RLC.t_Reassembly v_T_Reassembly:= f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var template (value) NR_RadioBearer_Type v_SS_DRB_Config := cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NormalMode); var template (value) NR_RadioBearerList_Type v_SS_DrbConfigList_TestMode := {cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; var EUTRA_NR_Coordination_MSG v_Coordination_MSG_Value; var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s220669 sic@ var SubFrameTiming_Type v_Timing; f_TC_7_1_1_9_1_Step0to2(p_NR_DRB_Id,v_LCID); //Restrict the SS to use of the HARQ proccess id used by the test cases v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1); f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_NR_DciDlInfo(cs_NR_DciFormat_1_X_ResourceAssignment(-,-,-,-,v_NR_HarqProcessConfig_DL), -, -)));//@sic R5s220669 sic@ //@siclog "Step 3" sigloc@ //SS transmits RRCConnectionReconfiguration message to E-UTRA PTC containing NR RRCReconfiguration message to perform SCG change with reconfigurationWithSync with the same PSCell //Send NR RRCreconfiguration message to E-UTRA PTC //Note: it is assumed that the timing can be achieved without timed sending //Set PDCP parameter discardTimer to 60ms v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(p_NR_DRB_Id, -, v_T_PollRetransmit, -, -, -, -, v_T_Reassembly, -, true_); // @sic R5-197023 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1,{v_SS_DRB_Config});//@sic R5s200855 sic@ f_NR_RLC_ReconfigurationHO_ENDC (nr_Cell1, v_RLC_BearerConfigList, cs_38508_PDCP_Config(-, -, -, omit), true,-,false); //@sic R5s200855 R5-213132 R5s210950 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell1, -, -, -, cs_NR_UplinkTimeAlignment_Stop); //@sic R5s210896 R5-215171 sic@ //EXCEPTION: Void //@sic R5-213132 R5-222044 sic@ EUTRA.receive(cmr_EUTRA_NR_OctetData ("Complete")) -> value v_Coordination_MSG_Value; f_NR_CheckReceived_RRCReconfigurationComplete(valueof(v_Coordination_MSG_Value)); f_TC_7_1_1_9_1_Step5to9A(p_NR_DRB_Id,v_LCID, v_SS_DrbConfigList_TestMode); //@sic R5s200855 sic@ f_TC_7_1_1_9_1_Step16Ato21(p_NR_DRB_Id,v_LCID, v_SS_DrbConfigList_TestMode);//@sic R5s200855 sic@ //@siclog "Step 22-23" siclog@ //The SS transmits NR RRCReconfiguration message to perform SCG change with reconfigurationWithSync with the same PSCell. //@sic R5s210950 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {v_SS_DRB_Config}); f_NR_RLC_ReconfigurationHO_ENDC (nr_Cell1, v_RLC_BearerConfigList, cs_38508_PDCP_Config(-, -, -, omit),true); //@sic R5-197023, R5s200855 sic@ f_TC_7_1_1_9_1_Step24to29(p_NR_DRB_Id, v_LCID,v_SS_DrbConfigList_TestMode);//@sic R5s200855 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {v_SS_DRB_Config}); }

After Change

function fl_TC_7_1_1_9_1_ENDC_TestBody(DRB_Identity p_NR_DRB_Id) runs on ENDC_NR_PTC { var B6_Type v_LCID := tsc_LCID_SCG_Bearer; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var RLC_Config.am.ul_AM_RLC.t_PollRetransmit v_T_PollRetransmit := f_NR_UL_AM_T_PollRetransmit(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var RLC_Config.am.dl_AM_RLC.t_Reassembly v_T_Reassembly:= f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1)); var template (value) NR_RadioBearer_Type v_SS_DRB_Config := cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NormalMode); var template (value) NR_RadioBearerList_Type v_SS_DrbConfigList_TestMode := {cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; var EUTRA_NR_Coordination_MSG v_Coordination_MSG_Value; var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s220669 sic@ var SubFrameTiming_Type v_Timing; f_TC_7_1_1_9_1_Step0to2(p_NR_DRB_Id,v_LCID); //Restrict the SS to use of the HARQ proccess id used by the test cases v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1); f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_NR_DciDlInfo(cs_NR_DciFormat_1_X_ResourceAssignment(-,-,-,-,v_NR_HarqProcessConfig_DL), -, -)));//@sic R5s220669 sic@ //@siclog "Step 3" sigloc@ //SS transmits RRCConnectionReconfiguration message to E-UTRA PTC containing NR RRCReconfiguration message to perform SCG change with reconfigurationWithSync with the same PSCell //Send NR RRCreconfiguration message to E-UTRA PTC //Note: it is assumed that the timing can be achieved without timed sending //Set PDCP parameter discardTimer to 60ms v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(p_NR_DRB_Id, -, v_T_PollRetransmit, -, -, -, -, v_T_Reassembly, -, true_); // @sic R5-197023 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1,{v_SS_DRB_Config});//@sic R5s200855 sic@ f_NR_RLC_ReconfigurationHO_ENDC (nr_Cell1, v_RLC_BearerConfigList, cs_38508_PDCP_Config(-, -, -, omit), true,-,false); //@sic R5s200855 R5-213132 R5s210950 sic@ //REMOVED f_NR_ULGrantConfiguration_Start(nr_Cell1, -, -, -, cs_NR_UplinkTimeAlignment_Stop); //@sic R5s210896 R5-215171 sic@ //EXCEPTION: Void //@sic R5-213132 R5-222044 sic@ EUTRA.receive(cmr_EUTRA_NR_OctetData ("Complete")) -> value v_Coordination_MSG_Value; f_NR_CheckReceived_RRCReconfigurationComplete(valueof(v_Coordination_MSG_Value)); f_TC_7_1_1_9_1_Step5to9A(p_NR_DRB_Id,v_LCID, v_SS_DrbConfigList_TestMode); //@sic R5s200855 sic@ f_TC_7_1_1_9_1_Step16Ato21(p_NR_DRB_Id,v_LCID, v_SS_DrbConfigList_TestMode);//@sic R5s200855 sic@ //@siclog "Step 22-23" siclog@ //The SS transmits NR RRCReconfiguration message to perform SCG change with reconfigurationWithSync with the same PSCell. //@sic R5s210950 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {v_SS_DRB_Config}); f_NR_RLC_ReconfigurationHO_ENDC (nr_Cell1, v_RLC_BearerConfigList, cs_38508_PDCP_Config(-, -, -, omit),true); //@sic R5-197023, R5s200855 sic@ f_TC_7_1_1_9_1_Step24to29(p_NR_DRB_Id, v_LCID,v_SS_DrbConfigList_TestMode);//@sic R5s200855 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {v_SS_DRB_Config}); }

## Change 2

| Function name | f_TC_7_1_1_9_1_Step5to9A |
| --- | --- |
| Reason for change | - Step 5A dictates to ignore SRs, but they are ignored only after step 7 which may be too late and while performing SS configurations an SR may come thus leading to test case failure. This is to increase stability of the test. |
| Summary of change | - Activating handler to ignore SR at the beginning of the f_TC_7_1_1_9_1_Step5to9A and deactivating it again prior reception of SR in step 7. |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_9_1_Step5to9A(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR_BASE_PTC { //@sic R5-206320 sic@ var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var default v_IgnoreNRSR:= null; //@sic R5-204418 sic@ var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s211710 sic@ // @siclog "Step 5" sigloc@ // void // @sic R5-213132 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, p_NR_RadioBearerList); //The SS gets prepared for SR indication and stops default UL grant allocation. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); //@siclog "Step 5A" siclog@ //@sic R5-204418 sic@ //The SS ignores scheduling requests and does not allocate any uplink grant. f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@siclog "Step 6" siclog@ //The SS transmits MAC PDU using same HARQ process and NDI as in step 1. //PDCP SDU = (37B + 3B PDCP Header) + 3B RLC Header + 2B MAC Header = 48B = 384 bits (an allowed TB size as per 38.523-3 Annex B) f_NR_SS_CellConfig_DciDlInfo_ReOrNewtransmission(nr_Cell1,-,cs_NR_TransportBlockRetransmission(tsc_NR_DefaultGrant_Imcs), -, cs_HarqProcessConfig(5));//@sic R5s211710 sic@ v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue (cs_NR_PDCP_PDU_18B(1,crs_NR_PDCP_SDU_37B))); //@sic R5s201457 R5s210896 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_Now, {cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding ( {cs_NR_MAC_SDU_SubPDU_LI8(p_LCID ,v_EncodedRlcPdu1)}, omit)})); //@sic R5s200855 R5s211710 sic@ // @siclog "Step 7 " siclog@ //Check: Does the UE transmit a Scheduling Request SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_SchedReq)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7: UE sent Scheduling Request"); v_IgnoreNRSR := activate(a_NR5GC_SR_Handler(nr_Cell1)); //@sic R5-204418 sic@ //@siclog "Step 8" siclog@ //The SS allocates single UL grant sufficient for one RLC SDU to be loop backed in a TTI, and NDI indicates new transmission redundancy version to be used as 0. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(384, v_NR_UplinkBWP); // 384 bits f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation, cs_HarqProcessConfig(5));//@sic R5s200855 R5s220687 sic@ //@siclog "Step 9" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, {cr_NR_UL_MAC_PDU ( {cr_NR_MAC_SDU_SubPDU(?,?)},*,*)})); //@siclog "Step 9A" siclog@ // The SS transmits a STATUS PDU. // Restrict the SS to use of the HARQ proccess id used by the test case f_NR_SS_CellConfig_DciDlInfo_Automatic(nr_Cell1, -, v_NR_HarqProcessConfig_DL); //@sic R5s211710 R5s220506 sic@ f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 1, p_NR_DRB_Id); deactivate(v_IgnoreNRSR); //@sic R5-204418 R5s220687sic@ //@sic R5-213132 sic@ }

After Change

function f_TC_7_1_1_9_1_Step5to9A(DRB_Identity p_NR_DRB_Id, B6_Type p_LCID, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR_BASE_PTC { //@sic R5-206320 sic@ var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var default v_IgnoreNRSR:= null; //@sic R5-204418 sic@ var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s211710 sic@ v_IgnoreNRSR := activate(a_NR5GC_SR_Handler(nr_Cell1)); // @siclog "Step 5" sigloc@ // void // @sic R5-213132 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, p_NR_RadioBearerList); //The SS gets prepared for SR indication and stops default UL grant allocation. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); //@siclog "Step 5A" siclog@ //@sic R5-204418 sic@ //The SS ignores scheduling requests and does not allocate any uplink grant. f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@siclog "Step 6" siclog@ //The SS transmits MAC PDU using same HARQ process and NDI as in step 1. //PDCP SDU = (37B + 3B PDCP Header) + 3B RLC Header + 2B MAC Header = 48B = 384 bits (an allowed TB size as per 38.523-3 Annex B) f_NR_SS_CellConfig_DciDlInfo_ReOrNewtransmission(nr_Cell1,-,cs_NR_TransportBlockRetransmission(tsc_NR_DefaultGrant_Imcs), -, cs_HarqProcessConfig(5));//@sic R5s211710 sic@ v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue (cs_NR_PDCP_PDU_18B(1,crs_NR_PDCP_SDU_37B))); //@sic R5s201457 R5s210896 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_Now, {cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding ( {cs_NR_MAC_SDU_SubPDU_LI8(p_LCID ,v_EncodedRlcPdu1)}, omit)})); //@sic R5s200855 R5s211710 sic@ deactivate(v_IgnoreNRSR); // @siclog "Step 7 " siclog@ //Check: Does the UE transmit a Scheduling Request SYSIND.receive(car_NR_SYSTEM_IND(nr_Cell1, cr_NR_SystemIndication_SchedReq)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7: UE sent Scheduling Request"); v_IgnoreNRSR := activate(a_NR5GC_SR_Handler(nr_Cell1)); //@sic R5-204418 sic@ //@siclog "Step 8" siclog@ //The SS allocates single UL grant sufficient for one RLC SDU to be loop backed in a TTI, and NDI indicates new transmission redundancy version to be used as 0. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(384, v_NR_UplinkBWP); // 384 bits f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation, cs_HarqProcessConfig(5));//@sic R5s200855 R5s220687 sic@ //@siclog "Step 9" siclog@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, {cr_NR_UL_MAC_PDU ( {cr_NR_MAC_SDU_SubPDU(?,?)},*,*)})); //@siclog "Step 9A" siclog@ // The SS transmits a STATUS PDU. // Restrict the SS to use of the HARQ proccess id used by the test case f_NR_SS_CellConfig_DciDlInfo_Automatic(nr_Cell1, -, v_NR_HarqProcessConfig_DL); //@sic R5s211710 R5s220506 sic@ f_NR_RLC_Status_MAC_PDU_Tx(p_LCID, 1, p_NR_DRB_Id); deactivate(v_IgnoreNRSR); //@sic R5-204418 R5s220687sic@ //@sic R5-213132 sic@ } -,