3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220866

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2657 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC MDT test case 8.1.6.1.4.8 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR5GC test case 8.1.6.1.4.8 in FR1 to the IWD_22wk23 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.1.6.1.4.8 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.4.8.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Correction to f_GetTestcaseAttrib_EarlyContentionResolution	3

2.2	Correction to f_TC_8_1_6_1_4_8_NR5GC	4

3	Branches executed	8

4	Execution Log Files	8

4.1	MediaTek MT6983	8

5	References	8

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.1.6.1.4.8 which is part of the NR5GC test suite in iWD_TTCN3-B2020-09_D22wk23.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	8_1_6_1_4_8_NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D22wk23

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	MediaTek MT6983

Verification Status:	PASS

# Corrections required

## Correction to f_GetTestcaseAttrib_EarlyContentionResolution

| Function name | f_GetTestcaseAttrib_EarlyContentionResolution |
| --- | --- |
| Reason for change | According to the prose there is RRCSetup message sent to the UE after Step 1B |
| Summary of change | Added the test case to this function so that the SS does not transmit RRCSetup message to the UE. |
| TTCN module | TestcaseProperties |
| MCC160 Comment |  |

Before Change:

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { … … case ("TC_8_1_6_1_4_6_NR5GC") { return true; } case ("TC_8_1_6_1_4_7_NR5GC") { return true; } }

After Change:

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_8_1_6_1_4_5_NR5GC") { return true; } case ("TC_8_1_6_1_4_6_NR5GC") { return true; } case ("TC_8_1_6_1_4_7_NR5GC") { return true; } case ("TC_8_1_6_1_4_8_NR5GC") { return true; }

## Correction to f_TC_8_1_6_1_4_8_NR5GC

| Function name | f_TC_8_1_6_1_4_8_NR5GC |
| --- | --- |
| Reason for change | According to the prose at Step 2 the UE shall be able to receive the Paging and then send RRCSetupRequest message. Therefore after Step 1B the additional RACH procedure needs to be ignored by the SS to abort the RRCSetupRequest procedure (Due to unsuccessful Contention Resolution procedure , the UE will try to re trigger the procedure) perRAAttemptInfoList_r16 may contain mulitiple entries and therefore the check needs to done based on receiveing the list and then post processing the list to check for contentionDetected_r16 = true |
| Summary of change | Modified v_NR_RachProcedure to not include contention resolution and ignore subsequent RACH procedure Modified the template cr_PerRAInfo_r16_SSB to receive any value and introduced variables/ for loop to check the template cr_PerRAAttemptInfo_r16 against the received list |
| TTCN module | RRC_MDT_IntraNR_CEF_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_6_1_4_8_NR5GC() runs on NR5GC_PTC { /* Connection Establishment Failure / Logging and reporting / RACH failure report */ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (present) UL_DCCH_Message v_UEInformationResponse; var NR_PhysicalParameters_Type v_NR_PhysicalParameters ; var template(value) DL_CCCH_Message v_RRCSetup; var template(value) NR_RachProcedureMsg4RrcMsg_Type v_RachProcedureMsg4RrcMsg; var octetstring v_EncodedMsg4; var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedure ; var template ConnEstFailReport_r16 v_ConnEstFailReport; var CellIdentity v_CellIdentity; var PLMN_Identity v_PLMN_Identity; f_NR5GC_Init(NR_1); //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); //Preamble //The UE is in state 1N-A on Cell 1 as defined in TS 38.508-1 [4], subclause 4.4A f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); //@siclog "Step 1" siclog@ //SS is configured to transmit MAC control element with non-matched UE Contention Resolution Identity. v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); v_RRCSetup := f_NR_38508_RRCSetup(nr_Cell1); v_EncodedMsg4 := f_NR_RRC_DL_CCCH_Encvalue(v_RRCSetup); v_RachProcedureMsg4RrcMsg := cs_NR_RachProcedureMsg4WithCcchMsg(v_EncodedMsg4); v_NR_RachProcedure := cs_NR_RachProcedureConfig_ContentionResolution_Unmatched(v_NR_PhysicalParameters, v_RachProcedureMsg4RrcMsg); f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); //@siclog "Step 1A" siclog@ //The SS transmits a Paging message. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 1B" siclog@ //The UE transmits an RRCSetupRequest message. f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Step 1C" siclog@ //The SS waits for 2sec to ensure that the UE detects T300 expiry. f_Delay(2.0); //@siclog "Step 1D" siclog@ //SS is configured to transmit MAC control element with matched UE Contention Resolution Identity f_NR_SS_RachProcedureConfig_Def(nr_Cell1); //@siclog "Step 2" siclog@ //SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordList. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 3-9" siclog@ //Steps 2-8 of generic test procedure in TS 38.508-1 Table 4.5.4.2-3 are performed on NR Cell 1. v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(nr_Cell1); f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Step 10" siclog@ //The SS sends a UEInformationRequest message to get connEstFailReportReq SRB.send(cas_NR_SRB_RrcPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, //cs_TimingInfo_Now cs_NR_UEInformationRequest_FAIL)); //@siclog "Step 11" siclog@ //Check: Does the UE send a UEInformationResponse message with connEstFailReport including the perRAInfoListra-ReportList-r16 on NR Cell1? v_CellIdentity := f_NR_CellInfo_GetCellIdentity(nr_Cell1); v_PLMN_Identity := f_NR_CellInfo_GetPLMN (nr_Cell1); v_ConnEstFailReport := cr_ConnEstFailReport_r16(cr_MeasResultFailedCell_r16(cr_CGI_Info_Logging_r16(v_PLMN_Identity, v_CellIdentity)), omit); v_UEInformationResponse := cr_UEInformationResponse_r16_CEF(v_ConnEstFailReport); v_UEInformationResponse.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.connEstFailReport_r16.perRAInfoList_r16 := {cr_PerRAInfo_r16_SSB(-,-,(1..200))} SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, v_UEInformationResponse)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test " & testcasename() & " Step 11"); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }// End of f_TC_8_1_6_1_4_8_NR5GC()

After Change:

module RRC_MDT_IntraNR_CEF_NR5GC { import from NR_ASP_SrbDefs all; // Added … … function f_TC_8_1_6_1_4_8_NR5GC() runs on NR5GC_PTC { /* Connection Establishment Failure / Logging and reporting / RACH failure report */ // added variables var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (present) UL_DCCH_Message v_UEInformationResponse; var NR_PhysicalParameters_Type v_NR_PhysicalParameters ; var template(value) DL_CCCH_Message v_RRCSetup; var template(value) NR_RachProcedureMsg4RrcMsg_Type v_RachProcedureMsg4RrcMsg; var octetstring v_EncodedMsg4; var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedure ; var template ConnEstFailReport_r16 v_ConnEstFailReport; var CellIdentity v_CellIdentity; var PLMN_Identity v_PLMN_Identity; var PerRAInfoList_r16 v_PerRAInfoList; var PerRAAttemptInfoList_r16 v_PerRAAttemptInfoList; var integer v_FieldCnt; var integer i; f_NR5GC_Init(NR_1); //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); //Preamble //The UE is in state 1N-A on Cell 1 as defined in TS 38.508-1 [4], subclause 4.4A f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); //@siclog "Step 1" siclog@ //SS is configured to transmit MAC control element with non-matched UE Contention Resolution Identity. v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); v_RRCSetup := f_NR_38508_RRCSetup(nr_Cell1); v_EncodedMsg4 := f_NR_RRC_DL_CCCH_Encvalue(v_RRCSetup); v_RachProcedureMsg4RrcMsg := cs_NR_RachProcedureMsg4WithCcchMsg(v_EncodedMsg4); //WA#WI=added RACH procedurelist to allow only one RRCSetupReq and subsequent PRACH there is no RAR v_NR_RachProcedure := { RachProcedureList := { cs_NR_RachProcedure_NoResponse, cs_NR_RachProcedure(v_NR_PhysicalParameters, cs_NR_ContentionResolutionCtrl_None), cs_NR_RachProcedure_NoResponse, cs_NR_RachProcedure_NoResponse } }; //commeted out v_NR_RachProcedure := cs_NR_RachProcedureConfig_ContentionResolution_Unmatched(v_NR_PhysicalParameters, v_RachProcedureMsg4RrcMsg); f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); //@siclog "Step 1A" siclog@ //The SS transmits a Paging message. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 1B" siclog@ //The UE transmits an RRCSetupRequest message. f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Step 1C" siclog@ //The SS waits for 2sec to ensure that the UE detects T300 expiry. f_Delay(2.0); //@siclog "Step 1D" siclog@ //SS is configured to transmit MAC control element with matched UE Contention Resolution Identity f_NR_SS_RachProcedureConfig_Def(nr_Cell1); //@siclog "Step 2" siclog@ //SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordList. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 3-9" siclog@ //Steps 2-8 of generic test procedure in TS 38.508-1 Table 4.5.4.2-3 are performed on NR Cell 1. v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(nr_Cell1); f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Step 10" siclog@ //The SS sends a UEInformationRequest message to get connEstFailReportReq SRB.send(cas_NR_SRB_RrcPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, //cs_TimingInfo_Now cs_NR_UEInformationRequest_FAIL)); //@siclog "Step 11" siclog@ //Check: Does the UE send a UEInformationResponse message with connEstFailReport including the perRAInfoListra-ReportList-r16 on NR Cell1? v_CellIdentity := f_NR_CellInfo_GetCellIdentity(nr_Cell1); v_PLMN_Identity := f_NR_CellInfo_GetPLMN (nr_Cell1); v_ConnEstFailReport := cr_ConnEstFailReport_r16(cr_MeasResultFailedCell_r16(cr_CGI_Info_Logging_r16(v_PLMN_Identity, v_CellIdentity)), omit); v_UEInformationResponse := cr_UEInformationResponse_r16_CEF(v_ConnEstFailReport); v_UEInformationResponse.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.connEstFailReport_r16.perRAInfoList_r16 := {cr_PerRAInfo_r16_SSB(?,-,(1..200))} //Accept any value and do the check later // added code change to check the UE information response list for contention detected. SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, v_UEInformationResponse)) -> value v_ReceivedAsp; v_PerRAInfoList := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.connEstFailReport_r16.perRAInfoList_r16; v_PerRAAttemptInfoList := v_PerRAInfoList[0].perRASSBInfoList_r16.perRAAttemptInfoList_r16; v_FieldCnt := lengthof(v_PerRAAttemptInfoList); for (i:=0; i<v_FieldCnt; i:=i+1){ if (match(v_PerRAAttemptInfoList[i],cr_PerRAAttemptInfo_r16)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Test " & testcasename() & " Step 11 contentionDetected set to true"); break; } else { if(i==v_FieldCnt-1){f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 11 contentionDetected not set to true");} } } f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }// End of f_TC_8_1_6_1_4_8_NR5GC()

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n78” using NEA1/NIA1

# Execution Log Files

## MediaTek MT6983

The MediaTek MT6983 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_8_1_6_1_4_8_NR5GC_MTK.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s220867: Supporting information for addition of NR5GC TC 8.1.6.1.4.8 in FR1. |
| --- | --- |