3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220953

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4682 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_SIP_BuildSipUri_RX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-07-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If-satement returns false when p_Port is ‘*’ and when p_URI doesn’t contain any port number, ttcn sets the expected portField to OMIT. This can cause an issue when the UE includes the port number under portField, while sending an INVITE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | if-statement is made based on contents of both p_URI and p_Port |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.4.3.4, 13.4.3.5, 13.4.3.6, 13.4.3.41 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_SIP_BuildSipUri_RX |
| --- | --- |
| Reason for change | If-satement returns false when p_Port is ‘*’ and when p_URI doesn’t contain any port number, ttcn sets the expected portField to OMIT. This can cause an issue when the UE includes port number under portField, while sending an INVITE. |
| Summary of change | if-statement is made based on contents of both p_URI and p_Port |
| TTCN module | IMS_SIP_Templates.ttcn |
| MCC160 Comment |  |

Before change:

function f_SIP_BuildSipUri_RX(charstring p_URI,

template integer p_Port := omit,

template SemicolonParam_List p_UrlParameters := *,

template AmpersandParam_List p_Headers := omit) return template (present) SipUrl

{

var template (present) SipUrl v_SipUrl := f_SIP_ParseURI(p_URI);

if (ischosen(v_SipUrl.components.sip) and ispresent(p_Port)) { /* @sic R5s130195 BASELINE MOVING 2013: change of SipUrl sic@

@sic R5s140020 change 11: p_Port may be * therefore shall not be checked to be present sic@

@sic R5s220593 change 2: ispresent(p_Port) sic@

!!!! CHECK: if p_Port is either omit or '*' and there are no cases p_Port is a a value or any value, then p_Port can be removed as the if-statement always is false !!!! */

v_SipUrl.components.sip.hostPort.portField := p_Port;

}

v_SipUrl.urlParameters := p_UrlParameters;

v_SipUrl.headers := p_Headers;  /* @sic R5s150382 change 4 sic@ */

return v_SipUrl;

}

After change:

function f_SIP_BuildSipUri_RX(charstring p_URI,

template integer p_Port := omit,

template SemicolonParam_List p_UrlParameters := *,

template AmpersandParam_List p_Headers := omit) return template (present) SipUrl

{

var template (present) SipUrl v_SipUrl := f_SIP_ParseURI(p_URI);

if (ischosen(v_SipUrl.components.sip) and ( not isvalue(v_SipUrl.components.sip.hostPort.portField) or isvalue(p_Port))) { /* @sic R5s130195 BASELINE MOVING 2013: change of SipUrl sic@

@sic R5s140020 change 11: p_Port may be * therefore shall not be checked to be present sic@

@sic R5s220593 change 2 sic@  */

v_SipUrl.components.sip.hostPort.portField := p_Port;

}

v_SipUrl.urlParameters := p_UrlParameters;

v_SipUrl.headers := p_Headers;  /* @sic R5s150382 change 4 sic@ */

return v_SipUrl;

}