3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220973

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2693 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR2 : Addition of test case 11.4.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add test case 11.4.2 to the 5G/NR5GC ATS for FR2 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 11.4.2 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

3	Branches executed	3

4	Execution Log Files	3

4.1	HiSilicon Balong 5000	3

5	References	3

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.4.2 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	11.4.2

ATS Version:	iwd-TTCN3-B2020-09_D22wk23

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

None

# Branches executed

The test case was executed on NR band n258 using NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_11_4_2_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_11_4_2_PIXIT.xml

# References

| [1] | R5s220974 : NR5GC FR2 : Supporting information for addition of test case 11.4.2 |
| --- | --- |