3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221115

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2764 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR RLC test case 7.1.2.3.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Since the UL grant assignment at step 14A is to occur 40ms following the grant assignment at step 13, it is observed that there it is sometimes not possible to configure the SS in time if there is any delay in the STATUS PDU at step 14 being delivered and processed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Preconfigure the grant assignment of step 14A in advance of receiving the RLC STATUS PDU at step 14, for increased test case stability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.9.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_2_3_9_NR_TestBody |
| --- | --- |
| Reason for change | Since the UL grant assignment at step 14A is to occur 40ms following the grant assignment at step 13, it is observed that there it is sometimes not possible to configure the SS in time if there is any delay in the STATUS PDU at step 14 being delivered and processed. |
| Summary of change | Preconfigure the grant assignment of step 14A in advance of receiving the RLC STATUS PDU at step 14, for increased test case stability. |
| TTCN module | RLC_TC_Common_NR |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_2_3_9_NR_TestBody(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { // AM RLC / Reassembling of AMD PDUs var integer v_RLC_Len1stSeg := 45; var integer v_RLC_Len2ndSeg := 44; //@sic R5s190605, R5s190539, R5-197007 sic@ var integer v_RLC_SDUsize := v_RLC_Len1stSeg + v_RLC_Len2ndSeg; // = 89 var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingDL1; var SubFrameTiming_Type v_TimingDL2; var SubFrameTiming_Type v_TimingDL3; var SubFrameTiming_Type v_TimingDL4; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_TimingUL1; var SubFrameTiming_Type v_TimingStatusRcvd;// Reception of a STATUS PDU which represents 2nd PDU in a time interval to be checked var integer v_Duration; var template (value) NR_RLC_Status_NackListSN18Bit_Type v_NR_NackSN18Bit_List; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; // generate 6 RLC SDUs = PDCP PDUs for transmission v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 6); // @siclog "Step 0" siclog@ …….. //@siclog "Step 13" siclog@ // After the expiry of t-StatusProhibit timer started at step 10, the SS assigns 1 UL grant (UL grant allocation type 3) of size 88 bits. (Note 2) v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(88, v_NR_UplinkBWP); //@sic R5s190890 sic@ f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation); //@siclog "Step 14" siclog@ // Check: Does the UE transmit a STATUS PDU with ACK_SN=4, thus acknowledging the reception of PDUs with SN=0 to SN=3, and no NACK_SN provided ? p_RLC_Rec.AM_RX_Next_Ack := 4; v_TimingStatusRcvd := f_NR_RxSTATUS_PDU_GetTimingInfo(p_RLC_Rec, p_NR_DRB_Id); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); //@siclog "Step 14A" siclog@ // SS waits for 40 ms to ensure UE RLC has all the required SDUs available in UL for loopback and then assigns 2 UL grants of 768 bits. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(768, v_NR_UplinkBWP); f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL1), v_NR_ResourceAllocation, 20, 2); //@siclog "Step 15" siclog@ // Check: Does the UE transmit RLC SDU#3? p_RLC_Rec.AM_RX_Next := 2; //@sic R5s190605, R5s190539 sic@ f_NR_RxAMD_SDU (p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_DataList[tsc_RLC_SDU3]); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); //@siclog "Step 15A" siclog@ …………………… }

After Change

function f_TC_7_1_2_3_9_NR_TestBody(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { // AM RLC / Reassembling of AMD PDUs var integer v_RLC_Len1stSeg := 45; var integer v_RLC_Len2ndSeg := 44; //@sic R5s190605, R5s190539, R5-197007 sic@ var integer v_RLC_SDUsize := v_RLC_Len1stSeg + v_RLC_Len2ndSeg; // = 89 var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingDL1; var SubFrameTiming_Type v_TimingDL2; var SubFrameTiming_Type v_TimingDL3; var SubFrameTiming_Type v_TimingDL4; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_TimingUL1; var SubFrameTiming_Type v_TimingStatusRcvd;// Reception of a STATUS PDU which represents 2nd PDU in a time interval to be checked var integer v_Duration; var template (value) NR_RLC_Status_NackListSN18Bit_Type v_NR_NackSN18Bit_List; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; // generate 6 RLC SDUs = PDCP PDUs for transmission v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 6); // @siclog "Step 0" siclog@ …….. //@siclog "Step 13" siclog@ // After the expiry of t-StatusProhibit timer started at step 10, the SS assigns 1 UL grant (UL grant allocation type 3) of size 88 bits. (Note 2) v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(88, v_NR_UplinkBWP); //@sic R5s190890 sic@ f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation); //@siclog "Step 14A" siclog@ // SS waits for 40 ms to ensure UE RLC has all the required SDUs available in UL for loopback and then assigns 2 UL grants of 768 bits. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(768, v_NR_UplinkBWP); f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL1), v_NR_ResourceAllocation, 20, 2); //@siclog "Step 14" siclog@ // Check: Does the UE transmit a STATUS PDU with ACK_SN=4, thus acknowledging the reception of PDUs with SN=0 to SN=3, and no NACK_SN provided ? p_RLC_Rec.AM_RX_Next_Ack := 4; v_TimingStatusRcvd := f_NR_RxSTATUS_PDU_GetTimingInfo(p_RLC_Rec, p_NR_DRB_Id); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); /* MOVED //@siclog "Step 14A" siclog@ // SS waits for 40 ms to ensure UE RLC has all the required SDUs available in UL for loopback and then assigns 2 UL grants of 768 bits. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(768, v_NR_UplinkBWP); f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL1), v_NR_ResourceAllocation, 20, 2); */ //@siclog "Step 15" siclog@ // Check: Does the UE transmit RLC SDU#3? p_RLC_Rec.AM_RX_Next := 2; //@sic R5s190605, R5s190539 sic@ f_NR_RxAMD_SDU (p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_DataList[tsc_RLC_SDU3]); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); //@siclog "Step 15A" siclog@ …………………… }