3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221141

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2783 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EN-DC CA test cases 8.2.4.2.1.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Security keys need to be sent again from EUTRA side to match the call to f_ENDC_GetKeysActivateSecurity() for p_SCell1 in fl_TC_8_2_4_2_1_x_ENDC_NR_TestBody() - It is required to set the C-RNTI of the new PCell before sending the SCGConfig to the UE via EUTRA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add 2nd call to f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys() Note : The coordination message send by the first call to this function at steps 2-3 is received inside f_NR_AddMod_PSCell_SCell_ENDC() and used to configure p_PSCell. - Add call to f_NR_CellInfo_SetRNTI() at step 5-6 before sending RRCConnectionReconfiguration message to reconfigure NR Cell 2 as PCell Note : This setting is already done later inside f_NR_RRC_PSCellChange_InterCell_Steps1To6(), but by this time SCGConfig will already be sent to the UE with incorrect value for “newUE-Identity” in “reconfigurationWithSync”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.2.1.1.ENDC, 8.2.4.2.1.3.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2 Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_8_2_4_2_1_x_ENDC_EUTRA_TestBody |
| --- | --- |
| Reason for change | Security keys need to be sent again from EUTRA side to match the call to f_ENDC_GetKeysActivateSecurity() for p_SCell1 in fl_TC_8_2_4_2_1_x_ENDC_NR_TestBody() |
| Summary of change | Add 2nd call to f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys() Note : The coordination message send by the first call to this function at steps 2-3 is received inside f_NR_AddMod_PSCell_SCell_ENDC() and used to configure p_PSCell. |
| TTCN module | ENDC\8_2_4\RRC_CarrierAggregation_ENDC_EUTRA |
| MCC160 Comment |  |

Before Change

function fl_TC_8_2_4_2_1_x_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { var EUTRA38_SecurityParamsExtd_Type v_Security_Params; // @siclog "Step 1" siclog@ //The SS changes Cell parameters according to the row "T0" in table 8.2.4.2.1.1.3.2-1. // Send trigger to NR to change power level of NR Cell1 f_EUTRA38_SetCellPower(eutra_Cell1, -85, -96); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("ChangePower")); //@sic R5s220809 sic@ //@siclog "Steps 2 - 3" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to configure NR Cell 1 as the NR PSCell and sCellToAddModList with an sCellIndex set to NR Cell 3 as the configured SCell. //Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? // Initiate SgNB Keys @sic R5s200647 sic@ v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR); f_EUTRA38_Security_Set(v_Security_Params); f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, MCG_SCG, v_Security_Params.SK_Counter); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 3"); if (not pc_IP_Ping) { //@sic R5s220809 R5-225406 sic@ f_EUTRA_CloseUE_TestLoopModeB(eutra_Cell1,'00'O); } //@siclog "Step 4" siclog@ //The SS changes NR Cell 2 and NR Cell 12 parameters according to the row "T1" in table 8.2.4.2.1.1.3.2-1. f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("ChangePower")); //@sic R5s220809 sic@ //@siclog "Steps 5 - 6" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to perform both NR Cell 1 and NR Cell 3 changed to the configured target NR Cell 2 and NR Cell 12. //Check: Does the UE transmit an RRCConnectionReconfigurationComplete message configuring the new PSCell and SCell and configure lower layers to consider the SCell to be in deactivated state? v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR, 1); f_EUTRA38_RRC_PSCellChange_InterCell (eutra_Cell1, v_Security_Params.SK_Counter); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6"); //@siclog "Steps 7 - 8" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to change the current PSCell (NR Cell 2) to NR Cell 12 equaling to one of the current UE SCell configuration. v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR, 1); f_EUTRA38_RRC_PSCellChange_InterCell (eutra_Cell1, v_Security_Params.SK_Counter); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 8"); if (not pc_IP_Ping) { //@sic R5s220809 R5-225406 sic@ f_EUTRA_OpenUE_TestLoopMode_Deactivate_TestMode (eutra_Cell1); } }

After Change

function fl_TC_8_2_4_2_1_x_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { var EUTRA38_SecurityParamsExtd_Type v_Security_Params; // @siclog "Step 1" siclog@ //The SS changes Cell parameters according to the row "T0" in table 8.2.4.2.1.1.3.2-1. // Send trigger to NR to change power level of NR Cell1 f_EUTRA38_SetCellPower(eutra_Cell1, -85, -96); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("ChangePower")); //@sic R5s220809 sic@ //@siclog "Steps 2 - 3" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to configure NR Cell 1 as the NR PSCell and sCellToAddModList with an sCellIndex set to NR Cell 3 as the configured SCell. //Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? // Initiate SgNB Keys @sic R5s200647 sic@ v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR); f_EUTRA38_Security_Set(v_Security_Params); f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, MCG_SCG, v_Security_Params.SK_Counter); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 3"); if (not pc_IP_Ping) { //@sic R5s220809 R5-225406 sic@ f_EUTRA_CloseUE_TestLoopModeB(eutra_Cell1,'00'O); } f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR); //@siclog "Step 4" siclog@ //The SS changes NR Cell 2 and NR Cell 12 parameters according to the row "T1" in table 8.2.4.2.1.1.3.2-1. f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("ChangePower")); //@sic R5s220809 sic@ //@siclog "Steps 5 - 6" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to perform both NR Cell 1 and NR Cell 3 changed to the configured target NR Cell 2 and NR Cell 12. //Check: Does the UE transmit an RRCConnectionReconfigurationComplete message configuring the new PSCell and SCell and configure lower layers to consider the SCell to be in deactivated state? v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR, 1); f_EUTRA38_RRC_PSCellChange_InterCell (eutra_Cell1, v_Security_Params.SK_Counter); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6"); //@siclog "Steps 7 - 8" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to change the current PSCell (NR Cell 2) to NR Cell 12 equaling to one of the current UE SCell configuration. v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR, 1); f_EUTRA38_RRC_PSCellChange_InterCell (eutra_Cell1, v_Security_Params.SK_Counter); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 8"); if (not pc_IP_Ping) { //@sic R5s220809 R5-225406 sic@ f_EUTRA_OpenUE_TestLoopMode_Deactivate_TestMode (eutra_Cell1); } }

## Change 2

| Function name | fl_TC_8_2_4_2_1_x_ENDC_NR_TestBody |
| --- | --- |
| Reason for change | It is required to set the C-RNTI of the new PCell before sending the SCGConfig to the UE via EUTRA. |
| Summary of change | Add call to f_NR_CellInfo_SetRNTI() at step 5-6 before sending RRCConnectionReconfiguration message to reconfigure NR Cell 2 as PCell Note : This setting is already done later inside f_NR_RRC_PSCellChange_InterCell_Steps1To6(), but by this time SCGConfig will already be sent to the UE with incorrect value for “newUE-Identity” in “reconfigurationWithSync”. |
| TTCN module | RRC_CarrierAggregation_ENDC_NR |
| MCC160 Comment |  |

Before Change

function fl_TC_8_2_4_2_1_x_ENDC_NR_TestBody(NR_CellId_Type p_PSCell1, NR_CellId_Type p_PSCell2, NR_CellId_Type p_SCell1, NR_CellId_Type p_SCell2) runs on ENDC_NR_PTC { var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) CellGroupConfig v_SCGConfig := f_NR_GetSCGConfigDef(p_PSCell2); var template(value) CellGroupConfig v_SCGConfig_SCell := f_NR_GetSCGConfig_SCellAdd(p_SCell2, 2); var template (value) RadioBearerConfig v_RadioBearerConfig := cs_NR_RadioBearer(tsc_NR_DRB2, secondary, f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()); var template (value) NR_RadioBearerList_Type v_RadioBearerToReleaseList := {cs_NR_SS_DRB_Release(tsc_NR_DRB2)}; var template (value) NR_RadioBearerList_Type v_RadioBearerList := {f_NR_GetRadioBearerConfig_FullAM_DRB2(p_PSCell2)}; v_CellPowerList_AtT0 := {cs_NR_CellPower(p_PSCell1, -88, -91), cs_NR_CellPower(p_SCell1, -88, -91)}; v_CellPowerList_AtT1 := {cs_NR_CellPower(p_PSCell2, -88, -91), cs_NR_CellPower(p_SCell2, -88, -91)}; // @siclog "Step 1" siclog@ //The SS changes Cell parameters according to the row "T0" in table 8.2.4.2.1.1.3.2-1. // Send trigger to NR to change power level of NR Cell1 f_NR_WaitForEUTRATrigger_And_ChangePower(v_CellPowerList_AtT0); //@siclog "Steps 2 - 3" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to configure NR Cell 1 as the NR PSCell and sCellToAddModList with an sCellIndex set to NR Cell 3 as the configured SCell. //Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? f_NR_AddMod_PSCell_SCell_ENDC( p_PSCell1, p_SCell1 ); //Get security info from EUTRA,calculate latest keys and configure SS f_ENDC_GetKeysActivateSecurity(p_SCell1,EUTRA); //@sic R5s220809 sic@ //@siclog "Step 3A" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1A indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 3? //@sic R5-203666 sic@ f_NR_CheckDataPath_CA(p_PSCell1, p_SCell1, "3A", true,tsc_SCellIndex_2); //@siclog "Step 4" siclog@ //The SS changes NR Cell 2 and NR Cell 12 parameters according to the row "T1" in table 8.2.4.2.1.1.3.2-1. f_NR_WaitForEUTRATrigger_And_ChangePower(v_CellPowerList_AtT1); //@siclog "Steps 5 - 6" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to perform both NR Cell 1 and NR Cell 3 changed to the configured target NR Cell 2 and NR Cell 12. //Check: Does the UE transmit an RRCConnectionReconfigurationComplete message configuring the new PSCell and SCell and configure lower layers to consider the SCell to be in deactivated state? v_SCGConfig := f_NR_GetSCGConfigDef(p_PSCell2); v_SCGConfig.sCellToAddModList := v_SCGConfig_SCell.sCellToAddModList; v_SCGConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; v_SCGConfig.sCellToReleaseList[0] := tsc_SCellIndex_2; v_RadioBearerConfig := f_NR_RetrieveRadioBearerConfig_PSCellChange( true, -, secondary); //@sic R5s220809 sic@ f_NR_RRC_PSCellChangeCA_InterCell(p_PSCell1, p_SCell1, p_PSCell2, p_SCell2, false, v_RadioBearerConfig, v_SCGConfig, v_RadioBearerToReleaseList); //@siclog "Step 6A" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1A indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 12? //@sic R5-203666 sic@ f_NR_CheckDataPath_CA(p_PSCell2, p_SCell2, "6A", true, tsc_SCellIndex_2); //@siclog "Steps 7 - 8" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to change the current PSCell (NR Cell 2) to NR Cell 12 equaling to one of the current UE SCell configuration. //Check: Does the UE send an RRCConnectionReconfigurationComplete message and reconfigure the current NR SCell to PSCell? //@sic R5s220809 sic@ v_SCGConfig := f_NR_GetSCGConfigDef(p_SCell2); v_SCGConfig_SCell := f_NR_GetSCGConfigDef(p_PSCell2); v_SCGConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; v_SCGConfig.sCellToAddModList := v_SCGConfig_SCell.sCellToAddModList; v_SCGConfig.sCellToReleaseList[0] := tsc_SCellIndex_2; f_NR_ReconfigSCellToNormalCell(p_SCell2, v_RadioBearerList); f_NR_RRC_PSCellChangeCA_InterCell(p_PSCell2, p_SCell2, p_SCell2, p_PSCell2, false, v_RadioBearerConfig, v_SCGConfig, v_RadioBearerToReleaseList); f_NR_ReconfigurePCellToNormalCell( p_PSCell2, tsc_PCellIndex_1, -, {tsc_NR_DRB2} ); }

After Change

function fl_TC_8_2_4_2_1_x_ENDC_NR_TestBody(NR_CellId_Type p_PSCell1, NR_CellId_Type p_PSCell2, NR_CellId_Type p_SCell1, NR_CellId_Type p_SCell2) runs on ENDC_NR_PTC { var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) CellGroupConfig v_SCGConfig := f_NR_GetSCGConfigDef(p_PSCell2); var template(value) CellGroupConfig v_SCGConfig_SCell := f_NR_GetSCGConfig_SCellAdd(p_SCell2, 2); var template (value) RadioBearerConfig v_RadioBearerConfig := cs_NR_RadioBearer(tsc_NR_DRB2, secondary, f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()); var template (value) NR_RadioBearerList_Type v_RadioBearerToReleaseList := {cs_NR_SS_DRB_Release(tsc_NR_DRB2)}; var template (value) NR_RadioBearerList_Type v_RadioBearerList := {f_NR_GetRadioBearerConfig_FullAM_DRB2(p_PSCell2)}; v_CellPowerList_AtT0 := {cs_NR_CellPower(p_PSCell1, -88, -91), cs_NR_CellPower(p_SCell1, -88, -91)}; v_CellPowerList_AtT1 := {cs_NR_CellPower(p_PSCell2, -88, -91), cs_NR_CellPower(p_SCell2, -88, -91)}; // @siclog "Step 1" siclog@ //The SS changes Cell parameters according to the row "T0" in table 8.2.4.2.1.1.3.2-1. // Send trigger to NR to change power level of NR Cell1 f_NR_WaitForEUTRATrigger_And_ChangePower(v_CellPowerList_AtT0); //@siclog "Steps 2 - 3" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to configure NR Cell 1 as the NR PSCell and sCellToAddModList with an sCellIndex set to NR Cell 3 as the configured SCell. //Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? f_NR_AddMod_PSCell_SCell_ENDC( p_PSCell1, p_SCell1 ); //Get security info from EUTRA,calculate latest keys and configure SS f_ENDC_GetKeysActivateSecurity(p_SCell1,EUTRA); //@sic R5s220809 sic@ //@siclog "Step 3A" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1A indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 3? //@sic R5-203666 sic@ f_NR_CheckDataPath_CA(p_PSCell1, p_SCell1, "3A", true,tsc_SCellIndex_2); //@siclog "Step 4" siclog@ //The SS changes NR Cell 2 and NR Cell 12 parameters according to the row "T1" in table 8.2.4.2.1.1.3.2-1. f_NR_WaitForEUTRATrigger_And_ChangePower(v_CellPowerList_AtT1); //@siclog "Steps 5 - 6" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to perform both NR Cell 1 and NR Cell 3 changed to the configured target NR Cell 2 and NR Cell 12. //Check: Does the UE transmit an RRCConnectionReconfigurationComplete message configuring the new PSCell and SCell and configure lower layers to consider the SCell to be in deactivated state? f_NR_CellInfo_SetRNTI(p_PSCell2,tsc_C_RNTI_Value3); v_SCGConfig := f_NR_GetSCGConfigDef(p_PSCell2); v_SCGConfig.sCellToAddModList := v_SCGConfig_SCell.sCellToAddModList; v_SCGConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; v_SCGConfig.sCellToReleaseList[0] := tsc_SCellIndex_2; v_RadioBearerConfig := f_NR_RetrieveRadioBearerConfig_PSCellChange( true, -, secondary); //@sic R5s220809 sic@ f_NR_RRC_PSCellChangeCA_InterCell(p_PSCell1, p_SCell1, p_PSCell2, p_SCell2, false, v_RadioBearerConfig, v_SCGConfig, v_RadioBearerToReleaseList); //@siclog "Step 6A" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1A indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 12? //@sic R5-203666 sic@ f_NR_CheckDataPath_CA(p_PSCell2, p_SCell2, "6A", true, tsc_SCellIndex_2); //@siclog "Steps 7 - 8" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to change the current PSCell (NR Cell 2) to NR Cell 12 equaling to one of the current UE SCell configuration. //Check: Does the UE send an RRCConnectionReconfigurationComplete message and reconfigure the current NR SCell to PSCell? //@sic R5s220809 sic@ v_SCGConfig := f_NR_GetSCGConfigDef(p_SCell2); v_SCGConfig_SCell := f_NR_GetSCGConfigDef(p_PSCell2); v_SCGConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; v_SCGConfig.sCellToAddModList := v_SCGConfig_SCell.sCellToAddModList; v_SCGConfig.sCellToReleaseList[0] := tsc_SCellIndex_2; f_NR_ReconfigSCellToNormalCell(p_SCell2, v_RadioBearerList); f_NR_RRC_PSCellChangeCA_InterCell(p_PSCell2, p_SCell2, p_SCell2, p_PSCell2, false, v_RadioBearerConfig, v_SCGConfig, v_RadioBearerToReleaseList); f_NR_ReconfigurePCellToNormalCell( p_PSCell2, tsc_PCellIndex_1, -, {tsc_NR_DRB2} ); }