3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221162

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2793 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC SON-MDT test case 8.1.6.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of f_TC_8_1_6_2_4_TestBody(), the NR PTC is waiting for a SysInfo coordination message from EUTRA PTC before step 1, but there is no matching send in the EUTRA code. Since the system information has already been exchanged before the preamble, it can be passed into the test case body function instead of needing to exchange again. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Pass the system information received from EUTRA before preamble into test case body function |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.2.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_6_2_4_NR5GC |
| --- | --- |
| Reason for change | In the current TTCN implementation of f_TC_8_1_6_2_4_TestBody(), the NR PTC is waiting for a SysInfo coordination message from EUTRA PTC before step 1, but there is no matching send in the EUTRA code. Since the system information has already been exchanged before the preamble, it can be passed into the test case body function instead of needing to exchange again. |
| Summary of change | Pass the system information received from EUTRA into f_TC_8_1_6_2_4_TestBody() |
| TTCN module | RRC_MDT_InterRAT_NR5GC_NR |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_6_2_4_NR5GC() runs on NR5GC_PTC { // Inter-RAT MDT / Connection Establishment Failure / Logging and reporting / Reporting of E-UTRA Inter-RAT handover var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; f_NR5GC_IRAT_Init(NR_6); v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); //Set Table 8.1.6.2.4.3.3-1: SIB2 of NR Cell 1 f_NR_CellInfo_SetSIB2_ThreshServingLowP(nr_Cell1, 10); //Table 8.1.6.2.4.3.3-2: SIB5 of NR Cell 1 f_NR_CellInfo_SetSIB5_CarrierFreq(nr_Cell1, 0, v_CoOrd_SysInfo.Eutra[0].Arfcn); f_NR5GC_CellInfo_SetSIB5_CellReselectionPriority(nr_Cell1, 0, 5); f_NR5GC_CellInfo_SetSIB5_ThreshXHigh(nr_Cell1, 0, 10); f_NR5GC_CellInfo_SetSIB5_ThreshXLow(nr_Cell1, 0, 20); f_NR5GC_CellInfo_SetSIB5_TReselectionEUTRA(nr_Cell1, 7); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -78, -78); //Maximum power level for FR2 FFS //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A); f_NR_SetCellPower (nr_Cell1, -78, -78); // Power levels on FR2 are FFS f_NR_TestBody_Set(true); f_TC_8_1_6_2_4_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Postamble")); }// End of f_TC_8_1_6_2_4_NR5GC()

After Change:

function f_TC_8_1_6_2_4_NR5GC() runs on NR5GC_PTC { // Inter-RAT MDT / Connection Establishment Failure / Logging and reporting / Reporting of E-UTRA Inter-RAT handover var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; f_NR5GC_IRAT_Init(NR_6); v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); //Set Table 8.1.6.2.4.3.3-1: SIB2 of NR Cell 1 f_NR_CellInfo_SetSIB2_ThreshServingLowP(nr_Cell1, 10); //Table 8.1.6.2.4.3.3-2: SIB5 of NR Cell 1 f_NR_CellInfo_SetSIB5_CarrierFreq(nr_Cell1, 0, v_CoOrd_SysInfo.Eutra[0].Arfcn); f_NR5GC_CellInfo_SetSIB5_CellReselectionPriority(nr_Cell1, 0, 5); f_NR5GC_CellInfo_SetSIB5_ThreshXHigh(nr_Cell1, 0, 10); f_NR5GC_CellInfo_SetSIB5_ThreshXLow(nr_Cell1, 0, 20); f_NR5GC_CellInfo_SetSIB5_TReselectionEUTRA(nr_Cell1, 7); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -78, -78); //Maximum power level for FR2 FFS //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A); f_NR_SetCellPower (nr_Cell1, -78, -78); // Power levels on FR2 are FFS f_NR_TestBody_Set(true); f_TC_8_1_6_2_4_TestBody(v_CoOrd_SysInfo); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Postamble")); }// End of f_TC_8_1_6_2_4_NR5GC()

## Change 2

| Function name | f_TC_8_1_6_2_4_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation of f_TC_8_1_6_2_4_TestBody(), the NR PTC is waiting for a SysInfo coordination message from EUTRA PTC before step 1, but there is no matching send in the EUTRA code. Since the system information has already been exchanged before the preamble, it can be passed into the test case body function instead of needing to exchange again. |
| Summary of change | Remove wait for coordination message, and used the new parameter passed from function f_TC_8_1_6_2_4_NR5GC() to calculate the values for variables v_ARFCN_ValueEUTRA and v_EUTRA_CellId |
| TTCN module | RRC_MDT_InterRAT_NR5GC_NR |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_6_2_4_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var CellIdentity v_CellIdentity_Cell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var PLMN_Identity v_PLMN_Identity_Cell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var ARFCN_ValueEUTRA v_ARFCN_ValueEUTRA; var EUTRA_PhysCellId v_EUTRA_CellId; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo (EUTRA); v_ARFCN_ValueEUTRA := v_CoOrd_SysInfo.Eutra[0].Arfcn; v_EUTRA_CellId := bit2int (v_CoOrd_SysInfo.Eutra[0].CellId); //@siclog "Steps 1" siclog@ //The SS changes NR Cell 1 and E-UTRA Cell 1 parameter according to the row "T1" in Table 8.1.6.2.4.3.2-1/2. f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Power Level")); … … … }// End of f_TC_8_1_6_2_4__TestBody()

After Change:

function f_TC_8_1_6_2_4_TestBody(EUTRA_NR_CoOrd_SysInfo_Type p_CoOrd_SysInfo) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var CellIdentity v_CellIdentity_Cell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var PLMN_Identity v_PLMN_Identity_Cell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var ARFCN_ValueEUTRA v_ARFCN_ValueEUTRA; var EUTRA_PhysCellId v_EUTRA_CellId; //REMOVED var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; //REMOVED v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo (EUTRA); v_ARFCN_ValueEUTRA := p_CoOrd_SysInfo.Eutra[0].Arfcn; v_EUTRA_CellId := bit2int (p_CoOrd_SysInfo.Eutra[0].CellId); //@siclog "Steps 1" siclog@ //The SS changes NR Cell 1 and E-UTRA Cell 1 parameter according to the row "T1" in Table 8.1.6.2.4.3.2-1/2. f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Power Level")); … … … }// End of f_TC_8_1_6_2_4__TestBody()