3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221180

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0174 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 Sep'22 baseline upgrade for POS Test Suite |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to develop the Rel-17 test cases in TTCN-3, the current baseline shall be upgraded. RAN5 agreed to move baseline to September 2022 in Rel-17 to all RAN5 TTCN-3 test suites. This CR is implementing this decision on 37.571-4 Test Suite. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR lists all changes affecting the messages and IEs type definitions caused by the baseline moving to the September 2022 Rel-17. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-17 test cases cannot be implemented and verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | POS ATS |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

1	Overview	2

2	Changes required to ASN.1 messages	2

2.1	ASN.1 Baseline Moving – TS 37.355	2

2.2	ASN.1 Baseline Moving - OMA-TS-LPPe	6

# Overview

This document lists all the essential changes needed in the iwd-TTCN3-B2020-09_D22wk37 for baseline moving.

Contact: 	Olivier Genoud

Olivier.Genoud@etsi.org

# Changes required to ASN.1 messages

## ASN.1 Baseline Moving – TS 37.355

#### LPP_PDU_Definitions ASN.1 module

The tables below list the new IEs included in TS 37.355-h20 and their impact on the existing TTCN code.

#### 2.1.1.1	Non-critical extensions with extension marker (ellipsis)

| ANS.1 type affected | New fields | Impact on TTCN |
| --- | --- | --- |
| CommonIEsRequestLocationInformation | scheduledLocationTime-r17 targetIntegrityRisk-r17 | Set to “omit” in cs_LppRequestLocationInfo |
| CommonIEsProvideLocationInformation | integrityInfo-r17 | Set to “*” in cr_CommonIEsProvideLocationInformation |
| NR-AdditionalPath-r16 | nr-DL-PRS-RSRPP-r17 | No impact |
| NR-DL-PRS-AssistanceDataPerTRP-r16 | nr-DL-PRS-ExpectedAoD-or-AoA-r17 | Set to “omit” in cs_DlPrsAssistanceDataPerTRP |
| NR-DL-PRS-Resource-r16 | dl-PRS-ResourcePrioritySubset-r17 | Set to “omit” in cs_NR_DL_PRS_Resource |
| NR-DL-PRS-ProcessingCapability-r16 | dummy | No impact |
| PRS-ProcessingCapabilityPerBand-r16 | supportedDL-PRS-ProcessingSamples-RRC-CONNECTED-r17 prs-ProcessingWindowType1A-r17 prs-ProcessingWindowType1B-r17 prs-ProcessingWindowType2-r17 prs-ProcessingCapabilityOutsideMGinPPW-r17 dl-PRS-BufferType-RRC-Inactive-r17 durationOfPRS-Processing-RRC-Inactive-r17 maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive-r17 supportedLowerRxBeamSweepingFactor-FR2-r17 supportedDL-PRS-ProcessingSamples-RRC-Inactive-r17 | No impact |
| NR-PositionCalculationAssistance-r16 | nr-TRP-BeamAntennaInfo-r17 nr-DL-PRS-Expected-LOS-NLOS-Assistance-r17 nr-DL-PRS-TRP-TEG-Info-r17 | Set to “omit” in cs_NrPositionCalculationAssist |
| SRS-CapabilityPerBand-r16 | posSRS-RRC-Inactive-InInitialUL-BWP-r17 posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 olpc-SRS-PosRRC-Inactive-r17 spatialRelationsSRS-PosRRC-Inactive-r17 | No impact |
| OTDOA-ProvideCapabilities | scheduledLocationRequestSupported-r17 | No impact |
| GNSS-CommonAssistData | gnss-Integrity-ServiceParameters-r17 gnss-Integrity-ServiceAlert-r17 | Set to “omit” in cs_GNSS_CommonAssistData |
| GNSS-PeriodicAssistData-r15 | gnss-Integrity-PeriodicServiceAlert-r17 | No impact |
| BDS-ClockModel2-r16 | bdsTgdB2ap-r17 bdsTgdB2ap-r17 | No impact |
| GNSS-SSR-OrbitCorrections-r15 | orbit-IntegrityParameters-r17 | No impact |
| SSR-OrbitCorrectionSatelliteElement-r15 | ssr-IntegrityOrbitBounds-r17 | No impact |
| GNSS-SSR-ClockCorrections-r15 | clock-IntegrityParameters-r17 | No impact |
| SSR-ClockCorrectionSatelliteElement-r15 | ssr-IntegrityClockBounds-r17 | No impact |
| SSR-CodeBiasSignalElement-r15 | ssr-IntegrityCodeBiasBounds-r17 | No impact |
| SSR-PhaseBiasSignalElement-r16 | ssr-IntegrityPhaseBiasBounds-r17 | No impact |
| GNSS-SSR-STEC-Correction-r16 | stec-IntegrityParameters-r17 | No impact |
| STEC-SatElement-r16 | stec-IntegrityErrorBounds-r17 | No impact |
| GNSS-SSR-GriddedCorrection-r16 | ssr-GriddedCorrectionIntegrityParameters-r17 | No impact |
| TropospericDelayCorrection-r16 | tropoDelayIntegrityErrorBounds-r17 | No impact |
| GNSS-CommonAssistDataReq | gnss-Integrity-ServiceParametersReq-r17 gnss-Integrity-ServiceAlertReq-r17 | Set to “omit” in cs_GNSS_CommonAssistDataReq |
| GNSS-PeriodicAssistDataReq-r15 | gnss-Integrity-PeriodicServiceAlertReq-r17 | No impact |
| GNSS-SSR-OrbitCorrectionsReq-r15 | orbit-IntegrityReq-r17 | No impact |
| GNSS-SSR-ClockCorrectionsReq-r15 | clock-IntegrityParametersReq-r17 ssr-IntegrityClockBoundsReq-r17 | No impact |
| GNSS-SSR-CodeBiasReq-r15 | ssr-IntegrityCodeBiasBoundsReq-r17 | No impact |
| GNSS-SSR-PhaseBiasReq-r16 | ssr-IntegrityPhaseBiasBoundsReq-r17 | No impact |
| GNSS-SSR-STEC-CorrectionReq-r16 | stec-IntegrityReq-r17 | No impact |
| GNSS-SSR-GriddedCorrectionReq-r16 | griddedCorrectionIntegrityReq-r17 | No impact |
| GNSS-LocationInformation | ha-GNSS-Metrics-r17 | Set to “*” in cr_A_GNSS_ProvideLocationInformation_LI |
| GNSS-PositioningInstructions | ha-GNSS-MetricsReq-r17 | Set to “omit” in cs_AgnssRequestLocationInfo |
| A-GNSS-ProvideCapabilities | scheduledLocationRequestSupported-r17 | Set to “*” in cr_A_GNSS_ProvideCapabilities |
| GNSS-SupportElement | ha-gnss-MetricsSupport-r17 | No impact |
| GNSS-CommonAssistanceDataSupport | gnss-Integrity-ServiceParametersSupport-r17 gnss-Integrity-ServiceAlertSupport-r17 | No impact |
| GNSS-SSR-OrbitCorrectionsSupport-r15 | orbit-IntegritySup-r17 | No impact |
| GNSS-SSR-ClockCorrectionsSupport-r15 | clock-IntegrityParameterSupport-r17 ssr-IntegrityClockBoundsSupport-r17 | No impact |
| GNSS-SSR-CodeBiasSupport-r15 | ssr-IntegrityCodeBiasBoundsSup-r17 | No impact |
| GNSS-SSR-PhaseBiasSupport-r16 | ssr-IntegrityPhaseBiasBoundsSup-r17 | No impact |
| GNSS-SSR-STEC-CorrectionSupport-r16 | stec-IntegritySup-r17 | No impact |
| GNSS-SSR-GriddedCorrectionSupport-r16 | griddedCorrectionIntegritySup-r17 | No impact |
| ECID-ProvideCapabilities | scheduledLocationRequestSupported-r17 | No impact |
| TBS-ProvideCapabilities-r13 | scheduledLocationRequestSupported-r17 | No impact |
| Sensor-ProvideCapabilities-r13 | scheduledLocationRequestSupported-r17 | No impact |
| WLAN-ProvideCapabilities-r13 | scheduledLocationRequestSupported-r17 | No impact |
| BT-ProvideCapabilities-r13 | scheduledLocationRequestSupported-r17 | No impact |
| NR-UL-ProvideCapabilities-r16 | nr-UE-TEG-Capability-r17 | No impact |
| NR-ECID-ProvideCapabilities-r16 | ten-ms-unit-ResponseTime-r17 scheduledLocationRequestSupported-r17 | No impact |
| NR-DL-TDOA-ProvideAssistanceData-r16 | nr-On-Demand-DL-PRS-Configurations-r17 nr-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17 assistanceDataValidityArea-r17 | Set to “omit” in cs_DlTdoaProvideAssistanceData |
| NR-DL-TDOA-RequestAssistanceData-r16 | nr-PosCalcAssistanceRequest-r17 nr-on-demand-DL-PRS-Request-r17 nr-DL-PRS-ExpectedAoD-or-AoA-Request-r17 pre-configured-AssistanceDataRequest-r17 | No impact |
| NR-DL-TDOA-ProvideLocationInformation-r16 | nr-DL-TDOA-SignalMeasurementInstances-r17 nr-DL-TDOA-LocationInformationInstances-r17 | Set to “*” in cr_NR_DL_TDOA_ProvideLocationInformation |
| NR-DL-TDOA-SignalMeasurementInformation-r16 | nr-UE-RxTEG-TimingErrorMargin-r17 | No impact |
| NR-DL-TDOA-MeasElement-r16 | nr-UE-Rx-TEG-ID-r17 nr-DL-PRS-FirstPathRSRP-Result-r17 nr-los-nlos-Indicator-r17 nr-AdditionalPathListExt-r17 nr-DL-TDOA-AdditionalMeasurementsExt-r17 | No impact |
| NR-DL-TDOA-AdditionalMeasurementElement-r16 | nr-UE-Rx-TEG-ID-r17 nr-DL-PRS-FirstPathRSRP-ResultDiff-r17 nr-los-nlos-IndicatorPerResource-r17 nr-AdditionalPathListExt-r17 | No impact |
| NR-DL-TDOA-LocationInformation-r16 | locationCoordinates-r17 locationSource-r17 | No impact |
| NR-DL-TDOA-RequestLocationInformation-r16 | nr-UE-RxTEG-Request-r17 nr-los-nlos-IndicatorRequest-r17 additionalPathsExt-r17 additionalPathsDL-PRS-RSRP-Request-r17 multiMeasInSameReport-r17 | Set to “omit” in cs_DlTDOARequestLocationInfoDef |
| NR-DL-TDOA-ReportConfig-r16 | measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 reducedDL-PRS-ProcessingSamples-r17 lowerRxBeamSweepingFactor-FR2-r17 | No impact |
| NR-DL-TDOA-ProvideCapabilities-r16 | ten-ms-unit-ResponseTime-r17 nr-PosCalcAssistanceSupport-r17 nr-los-nlos-AssistanceDataSupport-r17 nr-DL-PRS-ExpectedAoD-or-AoA-Sup-r17 nr-DL-TDOA-On-Demand-DL-PRS-Support-r17 nr-los-nlos-IndicatorSupport-r17 additionalPathsExtSupport-r17 scheduledLocationRequestSupported-r17 nr-dl-prs-AssistanceDataValidity-r17 multiMeasInSameMeasReport-r17 mg-ActivationRequest-r17 | No impact |
| NR-DL-TDOA-MeasurementCapability-r16 | nr-UE-TEG-Capability-r17 dl-tdoa-MeasCapabilityBandList-r17 | No impact |
| NR-DL-AoD-ProvideAssistanceData-r16 | nr-DL-PRS-BeamInfo-r17 nr-On-Demand-DL-PRS-Configurations-r17 nr-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17 assistanceDataValidityArea-r17 | Set to “omit” in cs_DlAoDProvideAssistanceData |
| NR-DL-AoD-RequestAssistanceData-r16 | nr-PosCalcAssistanceRequest-r17 nr-DL-PRS-ExpectedAoD-or-AoA-Request-r17 nr-DL-PRS-BeamInfoRequest-r17 nr-on-demand-DL-PRS-Request-r17 pre-configured-AssistanceDataRequest-r17 |  |
| NR-DL-AoD-ProvideLocationInformation-r16 | nr-DL-AoD-SignalMeasurementInstances-r17 nr-DL-AoD-LocationInformationInstances-r17 | Set to “*” in cr_NR_DL_AoD_ProvideLocationInformation |
| NR-DL-AoD-MeasElement-r16 | nr-DL-PRS-FirstPathRSRP-Result-r17 nr-los-nlos-Indicator-r17 nr-DL-AoD-AdditionalMeasurementsExt-r17 | No impact |
| NR-DL-AoD-LocationInformation-r16 | locationCoordinates-r17 locationSource-r17 | No impact |
| NR-DL-AoD-RequestLocationInformation-r16 | multiMeasInSameReport-r17 | Set to “omit” in cs_DlAoDRequestLocationInfoDef |
| NR-DL-AoD-ReportConfig-r16 | maxDL-PRS-RSRP-MeasurementsPerTRP-r17 maxDL-PRS-RSRPP-MeasurementsPerTRP-r17 nr-los-nlos-IndicatorRequest-r17 reducedDL-PRS-ProcessingSamples-r17 lowerRxBeamSweepingFactor-FR2-r17 | Set to “omit” in cs_DlAoDRequestLocationInfoDef |
| NR-DL-AoD-ProvideCapabilities-r16 | ten-ms-unit-ResponseTime-r17 nr-PosCalcAssistanceSupport-r17 nr-los-nlos-AssistanceDataSupport-r17 nr-DL-PRS-ExpectedAoD-or-AoA-Sup-r17 nr-DL-PRS-BeamInfoSup-r17 dl-PRS-ResourcePrioritySubset-Sup-r17 nr-DL-AoD-On-Demand-DL-PRS-Support-r17 nr-los-nlos-IndicatorSupport-r17 scheduledLocationRequestSupported-r17 nr-dl-prs-AssistanceDataValidity-r17 multiMeasInSameMeasReport-r17 mg-ActivationRequest-r17 | No impact |
| NR-DL-AoD-MeasurementCapability-r16 | maxDL-PRS-RSRP-MeasurementFR1-r17 maxDL-PRS-RSRP-MeasurementFR2-r17 | No impact |
| DL-AoD-MeasCapabilityPerBand-r16 | maxDL-PRS-FirstPathRSRP-MeasPerTRP-r17 dl-PRS-MeasRRC-Inactive-r17 | No impact |
| NR-Multi-RTT-ProvideAssistanceData-r16 | nr-On-Demand-DL-PRS-Configurations-r17 nr-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17 assistanceDataValidityArea-r17 | Set to “omit” in cs_MultiRTTProvideAssistanceData |
| NR-Multi-RTT-RequestAssistanceData-r16 | nr-on-demand-DL-PRS-Request-r17 nr-DL-PRS-ExpectedAoD-or-AoA-Request-r17 pre-configured-AssistanceDataRequest-r17 | No impact |
| NR-Multi-RTT-ProvideLocationInformation-r16 | nr-Multi-RTT-SignalMeasurementInstances-r17 | Set to “*” in cr_NR_Multi_RTT_ProvideLocationInformation |
| NR-Multi-RTT-SignalMeasurementInformation-r16 | nr-SRS-TxTEG-Set-r17 nr-UE-RxTEG-TimingErrorMargin-r17 nr-UE-TxTEG-TimingErrorMargin-r17 nr-UE-RxTxTEG-TimingErrorMargin-r17 | No impact |
| NR-Multi-RTT-MeasElement-r16 | nr-UE-RxTx-TEG-Info-r17 nr-DL-PRS-FirstPathRSRP-Result-r17 nr-los-nlos-Indicator-r17 nr-AdditionalPathListExt-r17 nr-Multi-RTT-AdditionalMeasurementsExt-r17 | No impact |
| NR-Multi-RTT-AdditionalMeasurementElement-r16 | nr-UE-RxTx-TEG-Info-r17 nr-DL-PRS-FirstPathRSRP-ResultDiff-r17 nr-los-nlos-IndicatorPerResource-r17 nr-AdditionalPathListExt-r17 | No impact |
| NR-Multi-RTT-RequestLocationInformation-r16 | nr-UE-RxTxTEG-Request-r17 measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17 measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 reducedDL-PRS-ProcessingSamples-r17 nr-los-nlos-IndicatorRequest-r17 additionalPathsExt-r17 additionalPathsDL-PRS-RSRP-Request-r17 multiMeasInSameReport-r17 lowerRxBeamSweepingFactor-FR2-r17 | Set to “omit” in cs_MultiRTTRequestLocationInfoDef |
| NR-Multi-RTT-ProvideCapabilities-r16 | ten-ms-unit-ResponseTime-r17 nr-DL-PRS-ExpectedAoD-or-AoA-Sup-r17 nr-Multi-RTT-On-Demand-DL-PRS-Support-r17 nr-UE-RxTx-TEG-ID-ReportingSupport-r17 nr-los-nlos-IndicatorSupport-r17 additionalPathsExtSupport-r17 scheduledLocationRequestSupported-r17 nr-dl-prs-AssistanceDataValidity-r17 multiMeasInSameMeasReport-r17 mg-ActivationRequest-r17 | No impact |
| NR-Multi-RTT-MeasurementCapability-r16 | nr-UE-TEG-Capability-r17 multi-RTT-MeasCapabilityBandList-r17 | No impact |

#### 2.1.1.2	New BIT STRING

| ANS.1 type affected | New values | Impact on TTCN |
| --- | --- | --- |
| NR-DL-TDOA-RequestLocationInformation-r16. nr-RequestedMeasurements-r16 | firstPathRsrpReq-r17 | No impact |
| NR-Multi-RTT-RequestLocationInformation-r16. nr-RequestedMeasurements-r16 | firstPathRsrpReq-r17 | No impact |

#### 2.1.1.3	New value in ENUMERATED type

| ANS.1 type affected | New values | Impact on TTCN |
| --- | --- | --- |
| ResponseTime.unit-r15 | ten-milli-seconds-v1700 | No impact |
| NR-DL-TDOA-LocationServerErrorCauses-r16. cause-r16 | on-demand-dl-prs-NotSupportedByServer-v1700 on-demand-dl-prs-SupportedButCurrentlyNotAvailableByServer-v1700 | No impact |
| NR-DL-AoD-LocationServerErrorCauses-r16. cause-r16 | on-demand-dl-prs-NotSupportedByServer-v1700 on-demand-dl-prs-SupportedButCurrentlyNotAvailableByServer-v1700 | No impact |
| NR-Multi-RTT-LocationServerErrorCauses-r16. cause-r16 | on-demand-dl-prs-NotSupportedByServer-v1700 on-demand-dl-prs-SupportedButCurrentlyNotAvailableByServer-v1700 | No impact |

#### LPP_Broadcast_Definitions ASN.1 module

No impact. No changes are included in TS 37.355-h20 compared with the current baseline.

## 2.2	ASN.1 Baseline Moving - OMA-TS-LPPe

No impact. ASN.1 remains as in OMA-TS-LPPe-V1_0-20200630-D.