3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221189

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0876 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS emergency call test case 10.14 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies, Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add test case 10.14 to the IMS/NR5GC ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS NR5GC call control service test case 10.14 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.14.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

3	Branches executed	10

4	Execution Log Files	10

4.1	Qualcomm SM 8450+SDX65	10

5	References	10

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 10.14 which is part of the IMS NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

Verification Test Summary

Test Case: 	10.14

ATS Version:	iwd-TTCN3-B2020-09_D12wk37

System Simulator used:	Keysight S8704A Protocol Conformance Toolset, R&S® 5G Protocol Conformance Test platform

UE used:	Qualcomm Snapdragon X70 5G Modem-RF System

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_10_14_NR5GC_IMS1 |
| --- | --- |
| Reason for change | For creating the NOTIFY message at step 15, the function f_IMS_PTC_ImsInfo_GetContactUrlUE()is being called with parameter p_ContactUriType set to “dialog” but since there is no ongoing (non-emergency) dialog at this step, this should use type “registration”. |
| Summary of change | Changed the parameter to function f_IMS_PTC_ImsInfo_GetContactUrlUE() to ‘registration’ |
| TTCN module | IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_10_14_NR5GC_IMS1() runs on IMS_PTC { /* In parallel emergency and non-emergency registrations / 5GS */ var template (value) Reginfo_Type v_XmlMessage; var SipUrl v_ContactUrlUE; var IMS_DATA_REQ v_IMS_REGISTER_REQ; var REGISTER_Request v_RegisterRequest; var boolean v_WaitForPublish := true; f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_NULL); f_IMS_CC_StartSignalling(IPCAN_InitialRegistration); v_IMS_REGISTER_REQ := f_IMS_Register(); f_IMS_Register_SubscribeNotify(v_IMS_REGISTER_REQ, v_WaitForPublish); f_IMS_CC_ReleaseConnection(IPCAN_InitialRegistration); f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); //@siclog "Steps 15" siclog@ v_RegisterRequest := v_IMS_REGISTER_REQ.Request.Register; v_XmlMessage := fl_TC_10_14_XmlMessageForNotify_Step15 (v_RegisterRequest); v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); fl_TC_10_14_Steps15_Notify (v_ContactUrlUE, v_XmlMessage); f_IMS_IMS_SendCoOrdMsg(OtherPDN); }

After Change:

function f_TC_10_14_NR5GC_IMS1() runs on IMS_PTC { /* In parallel emergency and non-emergency registrations / 5GS */ var template (value) Reginfo_Type v_XmlMessage; var SipUrl v_ContactUrlUE; var IMS_DATA_REQ v_IMS_REGISTER_REQ; var REGISTER_Request v_RegisterRequest; var boolean v_WaitForPublish := true; f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_NULL); f_IMS_CC_StartSignalling(IPCAN_InitialRegistration); v_IMS_REGISTER_REQ := f_IMS_Register(); f_IMS_Register_SubscribeNotify(v_IMS_REGISTER_REQ, v_WaitForPublish); f_IMS_CC_ReleaseConnection(IPCAN_InitialRegistration); f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); //@siclog "Steps 15" siclog@ v_RegisterRequest := v_IMS_REGISTER_REQ.Request.Register; v_XmlMessage := fl_TC_10_14_XmlMessageForNotify_Step15 (v_RegisterRequest); v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(registration); fl_TC_10_14_Steps15_Notify (v_ContactUrlUE, v_XmlMessage); f_IMS_IMS_SendCoOrdMsg(OtherPDN); }

## Change 2

| Function name | f_TC_10_14_NR5GC_IMS2 |
| --- | --- |
| Reason for change | At step 18, the prose states “Wait 10 seconds” but current TTCN implementation waits for 5s. |
| Summary of change | Changed the delay in the TTCN to 10s to align with the Prose |
| TTCN module | IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_10_14_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var SipUrl v_ContactUrlUE; f_IMS_PTC_Init(PDN_2); // Test Case Body f_IMS_TestBody_Set(true); f_IMS_IMS_WaitForTrigger(OtherPDN); //@siclog "Steps 1 - 14" siclog@ f_IMS_CC_StartCall(IPCAN_EmergencyCall_NormalService); f_IMS_EmergencyRegistration(); f_IMS_EmergencyCallSetup_NR5GC_A_6(); f_IMS_IMS_SendCoOrdMsg(OtherPDN); // Send sync message to other IMS PTC (for steps 16/17) f_IMS_IMS_WaitForTrigger(OtherPDN); // Wait for sync message from other IMS PTC v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); //@siclog "Steps 17" siclog@ f_Delay(5.0); //@siclog "Steps 18" siclog@ // Prose CR to void this is needed f_Delay(10.0); //@siclog "Steps 19 - 20" siclog@ f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20"); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService); }

After Change:

function f_TC_10_14_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var SipUrl v_ContactUrlUE; f_IMS_PTC_Init(PDN_2); // Test Case Body f_IMS_TestBody_Set(true); f_IMS_IMS_WaitForTrigger(OtherPDN); //@siclog "Steps 1 - 14" siclog@ f_IMS_CC_StartCall(IPCAN_EmergencyCall_NormalService); f_IMS_EmergencyRegistration(); f_IMS_EmergencyCallSetup_NR5GC_A_6(); f_IMS_IMS_SendCoOrdMsg(OtherPDN); // Send sync message to other IMS PTC (for steps 16/17) f_IMS_IMS_WaitForTrigger(OtherPDN); // Wait for sync message from other IMS PTC v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); //@siclog "Steps 17" siclog@ f_Delay(10.0); //@siclog "Steps 18" siclog@ // Prose CR to void this is needed f_Delay(10.0); //@siclog "Steps 19 - 20" siclog@ f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20"); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService); }

# Branches executed

The test case was executed on NR band n41 using NR ciphering nea1 and integrity algorithm nia1

# Execution Log Files

## Qualcomm Snapdragon X70 5G Modem-RF System

The Qualcomm Snapdragon X70 5G Modem-RF System UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_10_14_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_10_14_PIXIT.xml

The Qualcomm Snapdragon X70 5G Modem-RF System UE passed this test case R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

R&S/ tc_10.14_QC.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

# References

| [1] | R5s221191 : Supporting information for addition of IMS 5GS emergency call test case 10.14 |
| --- | --- |