3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221211

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2822 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC CAG Testcase 6.5.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently Step 42 configures the UE "CAG information list” with "indication that the MS is only allowed to access 5GS via CAG cells". This can have a persisting effect on some UEs since there is no mechanism in place for resetting this flag. On subsequent test runs, UE will not camp on a non-CAG cell due to the CAG-only indication. Following the added steps in Change 1, postamble will now be on NR Cell 2 in RRC_IDLE state |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Add new power level T5 to facilitate new steps 2) Added Steps 52 to 71a1 to make UE complete registration on CAG Cell NR Cell 2 3) Updated the Postamble to be completed on NR Cell 2 Note: An associated prose CR on 38.523-1 will be raised at RAN5#97 Update postamble to happen on NR Cell 2 in RRC_IDLE state |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will not be back to the original state before the start of the next testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR TS38-523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	8

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function f_TC_6_5_2_3_TestBody |
| --- | --- |
| Reason for change | Currently Step 42 configures the UE "CAG information list” with "indication that the MS is only allowed to access 5GS via CAG cells". This can have a persisting effect on some UEs since there is no mechanism in place for resetting this flag. On subsequent test runs, UE will not camp on a non-CAG cell due to the CAG-only indication. |
| Summary of change | - Add new power level T5 to facilitate new steps - Added Steps 52 to 71a1 to make UE complete registration on CAG Cell NR Cell 2 Note: An associated prose CR on 38.523-1 will be raised at RAN5#97 |
| TTCN module | CAG_Selection_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_6_5_2_3_TestBody() runs on NR5GC_PTC { var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell2)); var NAS_PlmnId v_PlmnID_11 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT3; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT4; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; timer t_Watchdog := 60.0; v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell4, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), //FR2 FFS cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR1)}; //FR2 FFS v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), //FR2 FFS cs_NR_CellPower(nr_Cell4, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR1)}; //FR2 FFS v_CellPowerList_AtT3 := {cs_NR_CellPower(nr_Cell4, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), //FR2 FFS cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR1)}; //FR2 FFS v_CellPowerList_AtT4 := {cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), //FR2 FFS cs_NR_CellPower(nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR1)}; //FR2 FFS //@siclog "Step 1" siclog@ //Power on the UE f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2" siclog@ //Check: Does the UE send a RRCSetupRequest on NR Cell 4 within the next 60 s? t_Watchdog.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell4, cr_38508_RRCSetupRequest))){ t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); } } //@siclog "Step 3" siclog@ //SS adjusts cell levels according to row T1 of table 6.5.2.3.3.2-1 for FR1 and table 6.5.2.3.3.2-2 for FR2 f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 4-16" siclog@ //Steps 1 to 13 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2 are performed on NR Cell 1. //@siclog "Step 17" siclog@ //The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message containing the CAG information list with the CAG ID of NR Cell 2 //@siclog "Step 18-23a1" siclog@ //Steps 15 to 20a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2 are performed on NR Cell 1. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); fl_NR5GC_RRC_IdleSteps4_20_withCAGInfoList(nr_Cell1, v_ReceivedMsg, -, -, cs_CAGInfoList_oneEntry(cs_CAGInfo_oneID(v_PlmnID_2,'0'B, int2oct(1,4)))); //@siclog "Step 24" siclog@ //The UE is switched off by executing generic procedure in Table 4.9.6.1-1 in TS 38.508-1 f_NR_UE_DeRegisterOnSwitchOff (nr_Cell1, STATE_IDLE_1A); //@siclog "Step 25" siclog@ //SS adjusts cell levels according to row T2 of table 6.5.2.3.3.2-1 for FR1 and table 6.5.2.3.3.2-2 for FR2 f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@siclog "Step 26" siclog@ //Power on the UE. f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 27" siclog@ //Check: Does the UE send a RRCSetupRequest on NR Cell 4 within the next 60 s? t_Watchdog.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell4, cr_38508_RRCSetupRequest))){ t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 27"); } [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 27"); } } //@siclog "Step 28" siclog@ //SS adjusts cell levels according to row T3 of table 6.5.2.3.3.2-1 for FR1 and table 6.5.2.3.3.2-2 for FR2 f_NR_SetCellPowerList(v_CellPowerList_AtT3); //@siclog "Step 29-47a1" siclog@ //Steps 1 to 20a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2 are performed on NR Cell 2. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell2, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); fl_NR5GC_RRC_IdleSteps4_20_withCAGInfoList(nr_Cell2, v_ReceivedMsg, -, -, cs_CAGInfoList_oneEntry(cs_CAGInfo_oneID(v_PlmnID_11,'1'B, int2oct(1,4)))); //@siclog "Step 48" siclog@ //The UE is switched off by executing generic procedure in Table 4.9.6.1-1 in TS 38.508-1 f_NR_UE_DeRegisterOnSwitchOff (nr_Cell2, STATE_IDLE_1A); //@siclog "Step 49" siclog@ //SS adjusts cell levels according to row T4 of table 6.5.2.3.3.2-1 for FR1 and table 6.5.2.3.3.2-2 for FR2 f_NR_SetCellPowerList(v_CellPowerList_AtT4); //@siclog "Step 50" siclog@ //Power on the UE. f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 51" siclog@ //Check: Does the UE send a RRCSetupRequest on NR Cell 11 within the next 60 s? t_Watchdog.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest))){ t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 51"); } [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 51"); } } }//End of f_TC_6_5_2_3_TestBody

After Change:

function fl_TC_6_5_2_3_TestBody() runs on NR5GC_PTC { var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell2)); var NAS_PlmnId v_PlmnID_11 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT3; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT4; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT5; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; timer t_Watchdog := 60.0; v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell4, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), //FR2 FFS cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR1)}; //FR2 FFS v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), //FR2 FFS cs_NR_CellPower(nr_Cell4, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR1)}; //FR2 FFS v_CellPowerList_AtT3 := {cs_NR_CellPower(nr_Cell4, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), //FR2 FFS cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR1)}; //FR2 FFS v_CellPowerList_AtT4 := {cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), //FR2 FFS cs_NR_CellPower(nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR1)}; //FR2 FFS v_CellPowerList_AtT5 := {cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR1), //FR2 FFS cs_NR_CellPower(nr_Cell11, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; //FR2 FFS //@siclog "Step 1" siclog@ //Power on the UE f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2" siclog@ //Check: Does the UE send a RRCSetupRequest on NR Cell 4 within the next 60 s? t_Watchdog.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell4, cr_38508_RRCSetupRequest))){ t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); } } //@siclog "Step 3" siclog@ //SS adjusts cell levels according to row T1 of table 6.5.2.3.3.2-1 for FR1 and table 6.5.2.3.3.2-2 for FR2 f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 4-16" siclog@ //Steps 1 to 13 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2 are performed on NR Cell 1. //@siclog "Step 17" siclog@ //The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message containing the CAG information list with the CAG ID of NR Cell 2 //@siclog "Step 18-23a1" siclog@ //Steps 15 to 20a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2 are performed on NR Cell 1. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); fl_NR5GC_RRC_IdleSteps4_20_withCAGInfoList(nr_Cell1, v_ReceivedMsg, -, -, cs_CAGInfoList_oneEntry(cs_CAGInfo_oneID(v_PlmnID_2,'0'B, int2oct(1,4)))); //@siclog "Step 24" siclog@ //The UE is switched off by executing generic procedure in Table 4.9.6.1-1 in TS 38.508-1 f_NR_UE_DeRegisterOnSwitchOff (nr_Cell1, STATE_IDLE_1A); //@siclog "Step 25" siclog@ //SS adjusts cell levels according to row T2 of table 6.5.2.3.3.2-1 for FR1 and table 6.5.2.3.3.2-2 for FR2 f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@siclog "Step 26" siclog@ //Power on the UE. f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 27" siclog@ //Check: Does the UE send a RRCSetupRequest on NR Cell 4 within the next 60 s? t_Watchdog.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell4, cr_38508_RRCSetupRequest))){ t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 27"); } [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 27"); } } //@siclog "Step 28" siclog@ //SS adjusts cell levels according to row T3 of table 6.5.2.3.3.2-1 for FR1 and table 6.5.2.3.3.2-2 for FR2 f_NR_SetCellPowerList(v_CellPowerList_AtT3); //@siclog "Step 29-47a1" siclog@ //Steps 1 to 20a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2 are performed on NR Cell 2. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell2, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); fl_NR5GC_RRC_IdleSteps4_20_withCAGInfoList(nr_Cell2, v_ReceivedMsg, -, -, cs_CAGInfoList_oneEntry(cs_CAGInfo_oneID(v_PlmnID_11,'1'B, int2oct(1,4)))); //@siclog "Step 48" siclog@ //The UE is switched off by executing generic procedure in Table 4.9.6.1-1 in TS 38.508-1 f_NR_UE_DeRegisterOnSwitchOff (nr_Cell2, STATE_IDLE_1A); //@siclog "Step 49" siclog@ //SS adjusts cell levels according to row T4 of table 6.5.2.3.3.2-1 for FR1 and table 6.5.2.3.3.2-2 for FR2 f_NR_SetCellPowerList(v_CellPowerList_AtT4); //@siclog "Step 50" siclog@ //Power on the UE. f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 51" siclog@ //Check: Does the UE send a RRCSetupRequest on NR Cell 11 within the next 60 s? t_Watchdog.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest))){ t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 51"); } [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 51"); } } //@siclog "Step 52" siclog@ //SS adjusts cell levels according to row T5 of table 6.5.2.3.3.2-1 for FR1 and table 6.5.2.3.3.2-2 for FR2 f_NR_SetCellPowerList(v_CellPowerList_AtT5); //@siclog "Step 53-71a1" siclog@ // Steps 1 to 20a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2 are performed on NR Cell 2. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell2, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); fl_NR5GC_RRC_IdleSteps4_20_withCAGInfoList(nr_Cell2, v_ReceivedMsg, -, -, cs_CAGInfoList_oneEntry(cs_CAGInfo_oneID(v_PlmnID_2,'0'B, int2oct(1,4)))); }//End of f_TC_6_5_2_3_TestBody

## Change 2

| Function name | function f_TC_6_5_2_3_NR5GC() |
| --- | --- |
| Reason for change | Following the added steps in Change 1, postamble will now be on NR Cell 2 in RRC_IDLE state |
| Summary of change | Update postamble to happen on NR Cell 2 in RRC_IDLE state |
| TTCN module | CAG_Selection_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_6_5_2_3_NR5GC() runs on NR5GC_PTC { //CAG / Limited Service / No Suitable cell var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), //FR2 FFS cs_NR_CellPower(nr_Cell4, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR1)}; //FR2 FFS f_NR5GC_Init(NR_2); //Cell2 and Cell4 are CAG Cells. default CAG-ID value used for Cell2, value 2 used for Cell4 f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell2); f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell4, 2); f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(nr_Cell4, true_);//@sic R5s221013 sic@ f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(nr_Cell2, true_);//@sic R5s221013 sic@ //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell2); //CAG Cell f_NR_CellConfig_Def(nr_Cell4); //CAG Cell f_NR_CellConfig_Def(nr_Cell11); //The UE is in Automatic PLMN selection mode. f_UT_AutomaticPLMN_Select (UT); //Make the pre-test condition satisfied v_ReceivedMsg := f_NR5GC_Preamble_Steps1_4 (nr_Cell1); //The UE is registered on NR Cell 1 using the procedure described in TS 38.508-1 [4] clause 4.5.2.2 except that the REGISTRATION ACCEPT message includes an empty CAG information list before it is in state Switched OFF (State 0N-B). fl_NR5GC_RRC_IdleSteps4_20_withCAGInfoList(nr_Cell1, v_ReceivedMsg, Initial_NoSecurity, -, cs_EmptyCAGInfoList); f_NR_UE_DeRegisterOnSwitchOff (nr_Cell1, STATE_IDLE_1A); //SS adjusts cell levels according to row T0 of table 6.5.2.3.3.2-1 for FR1 and table 6.5.2.3.3.2-2 for FR2 f_NR_SetCellPowerList(v_CellPowerList_AtT0); f_NR_TestBody_Set(true); fl_TC_6_5_2_3_TestBody(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell11,STATE_DEREGISTERED); }

After Change:

function f_TC_6_5_2_3_NR5GC() runs on NR5GC_PTC { //CAG / Limited Service / No Suitable cell var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), //FR2 FFS cs_NR_CellPower(nr_Cell4, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR1)}; //FR2 FFS f_NR5GC_Init(NR_2); //Cell2 and Cell4 are CAG Cells. default CAG-ID value used for Cell2, value 2 used for Cell4 f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell2); f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList(nr_Cell4, 2); f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(nr_Cell4, true_);//@sic R5s221013 sic@ f_NR_CellInfo_SetSIB1_CellReservedForOtherUse(nr_Cell2, true_);//@sic R5s221013 sic@ //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell2); //CAG Cell f_NR_CellConfig_Def(nr_Cell4); //CAG Cell f_NR_CellConfig_Def(nr_Cell11); //The UE is in Automatic PLMN selection mode. f_UT_AutomaticPLMN_Select (UT); //Make the pre-test condition satisfied v_ReceivedMsg := f_NR5GC_Preamble_Steps1_4 (nr_Cell1); //The UE is registered on NR Cell 1 using the procedure described in TS 38.508-1 [4] clause 4.5.2.2 except that the REGISTRATION ACCEPT message includes an empty CAG information list before it is in state Switched OFF (State 0N-B). fl_NR5GC_RRC_IdleSteps4_20_withCAGInfoList(nr_Cell1, v_ReceivedMsg, Initial_NoSecurity, -, cs_EmptyCAGInfoList); f_NR_UE_DeRegisterOnSwitchOff (nr_Cell1, STATE_IDLE_1A); //SS adjusts cell levels according to row T0 of table 6.5.2.3.3.2-1 for FR1 and table 6.5.2.3.3.2-2 for FR2 f_NR_SetCellPowerList(v_CellPowerList_AtT0); f_NR_TestBody_Set(true); fl_TC_6_5_2_3_TestBody(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell2,STATE_IDLE_1A); }