3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221251

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2846 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of 5GMM Rel-16 RACS test case 9.1.9.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add test case 9.1.9.6 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 9.1.9.6 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.9.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 R5-226409 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1 Change 1	3

2.2 Change 2	5

3	Branches executed	7

4	Execution Log Files	8

4.1	Samsung SM-G996B	8

5	References	8

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 9.1.9.6 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	9.1.9.6

ATS Version:	iwd-TTCN3-B2020-09_D22wk37

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Samsung SM-G996B

Verification Status:	PASS

# Corrections required

## 2.1 Change 1

| Function name | f_TC_9_1_9_6_NR5GC |
| --- | --- |
| Reason for change | SRB2 is not established during the test body so postamble signalling will use SRB1 |
| Summary of change | Pass SRB paramter to postamble function |
| TTCN module | RACS_NR5GC |
| MCC160 Comment |  |

Before Change

function f_TC_9_1_9_6_NR5GC() runs on NR5GC_PTC { //RACS / Change in radio capability / NW assigned URCID var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_MobileIdentity v_GUTIToSend; var NG_NAS_GutiParameters_Type v_GutiParams; var NAS_PlmnId v_PLMN; var TrackingAreaCode v_TAC; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_TrackingAreaIdList v_TAIList; f_NR5GC_Init_NAS(NR_1); // Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); v_GMM_MobilityInfo := f_NR5GC_Preamble_Steps1_13 (v_NGC_NASCellA); if (not match (v_GMM_MobilityInfo.GMMCap, cdr_NG_GMM_Cap_RACS)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); // Registration Accepted v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); f_NR5GC_RRC_Idle_Steps15_20(v_NGC_NASCellA, TESTMode_OFF, rrcConnectionRelease); f_NR_TestBody_Set(true); fl_TC_9_1_9_6_Body(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A); }

After Change

function f_TC_9_1_9_6_NR5GC() runs on NR5GC_PTC { //RACS / Change in radio capability / NW assigned URCID var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_MobileIdentity v_GUTIToSend; var NG_NAS_GutiParameters_Type v_GutiParams; var NAS_PlmnId v_PLMN; var TrackingAreaCode v_TAC; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_TrackingAreaIdList v_TAIList; f_NR5GC_Init_NAS(NR_1); // Create and configure cells f_NR_CellConfig_Def(v_NGC_NASCellA); v_GMM_MobilityInfo := f_NR5GC_Preamble_Steps1_13 (v_NGC_NASCellA); if (not match (v_GMM_MobilityInfo.GMMCap, cdr_NG_GMM_Cap_RACS)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); // Registration Accepted v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); f_NR5GC_RRC_Idle_Steps15_20(v_NGC_NASCellA, TESTMode_OFF, rrcConnectionRelease); f_NR_TestBody_Set(true); fl_TC_9_1_9_6_Body(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A, tsc_NR_RbId_SRB1); }

## 2.2 Change 2

| Function name | fl_TC_9_1_9_6_TestBody |
| --- | --- |
| Reason for change | Since the radio capability of the UE has changed, the UE will not include the NW-assigned UE radio capability ID in the REGISTRATION REQUEST at step 2 (also no UE radio capability ID was assigned during the preamble). |
| Summary of change | Remove the check of UE Radio Capability ID at step 2 Note : An associated prose CR on 38.523-1 will be raised at RAN5#97. |
| TTCN module | RACS_NR5GC |
| MCC160 Comment |  |

Before Change

function fl_TC_9_1_9_6_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_MobileIdentity v_GUTIToSend; var NG_NAS_GutiParameters_Type v_GutiParams; var NAS_PlmnId v_PLMN; var TrackingAreaCode v_TAC; var template (value) NG_TrackingAreaIdList v_TAIList; //@siclog "Step 1" siclog@ //Cause the UE to change the radio capability configuraiton. f_UT_ChangeRAC_Id(UT); //@siclog "Step 2" siclog@ //Check: Does the UE transmit REGISTRATION REQUEST message including 'NG-RAN-RCU'? bit and the UE radio capability ID#1 (assigned during preamble)? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cr_NG_Update ('1'B), -, -, -, -, -, cs_UERadioCapId('67'O, '01000000000010'O) )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); //@siclog "Step 3" siclog@ v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cs_UERadioCapId('67'O, '01000000000001'O) ); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Steps 4" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); } //End of fl_TC_9_1_9_6_Body}

After Change

function fl_TC_9_1_9_6_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_MobileIdentity v_GUTIToSend; var NG_NAS_GutiParameters_Type v_GutiParams; var NAS_PlmnId v_PLMN; var TrackingAreaCode v_TAC; var template (value) NG_TrackingAreaIdList v_TAIList; //@siclog "Step 1" siclog@ //Cause the UE to change the radio capability configuraiton. f_UT_ChangeRAC_Id(UT); //@siclog "Step 2" siclog@ //Check: Does the UE transmit REGISTRATION REQUEST message including 'NG-RAN-RCU'? bit and the UE radio capability ID#1 (assigned during preamble)? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cr_NG_Update ('1'B) )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); //@siclog "Step 3" siclog@ v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cs_UERadioCapId('67'O, '01000000000001'O) ); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Steps 4" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); } //End of fl_TC_9_1_9_6_Body

# Branches executed

The test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## Samsung SM-G996B

The Samsung SM-G996B UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_9_1_9_6_LOG.html

- PICS/PIXIT parameter file:

TC_9_1_9_6_PIXIT.xml

# References

| [1] | R5s221252 : NR5GC FR1 : Supporting information for addition of 5GMM Rel-16 RACS test case 9.1.9.6 |
| --- | --- |