3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230065

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2905 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.3.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the ASP definition for Symbol Timing: type union SymbolTimingInfo_Type { /* EUTRA, NBIOT: * REQ ASPs: TTCN shall set the SymbolTimingInfo to "FirstSymbol" for EUTRA, NBIOT addressing the whole subframe * IND ASPs: TTCN shall ignore the SymbolTimingInfo sent by the SS for EUTRA, NBIOT * NR: * REQ ASPs: Any:=true only if the symbol number is not relevant, * in which case the SS may choose the next available symbol of the given slot * IND ASPs: Any:=true only if there is no symbol information available for the particular kind of indication */ SymbolNumber_Type SymbolOffset, /* to address a particular symbol in a slot */ Null_Type FirstSymbol, /* to address the first symbol (REQ ASPs only) or for REQ ASPs in EUTRA and NBIOT */ Null_Type Any /* for IND ASPs in EUTRA and NBIOT or if symbol number is not relevant or not available */ }; In the TC, the Symbol is set as cs_SymbolTimingInfo_FirstSlot as a default assignment. However, when receiving UL data for PUSCH Aggregation, it is needed to change the Symbol: = cs_SymbolTimingInfo_Any as symbol number is not relevant here |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced a template cs_SymbolTimingInfo_Any to set the Symbol to “Any” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.9.NR5GC, 7.1.1.3.9.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

| Function name | f_TC_7_1_1_3_9_NR_TestBody () |
| --- | --- |
| Reason for change | According to the ASP definition for Symbol Timing: type union SymbolTimingInfo_Type { /* EUTRA, NBIOT: * REQ ASPs: TTCN shall set the SymbolTimingInfo to "FirstSymbol" for EUTRA, NBIOT addressing the whole subframe * IND ASPs: TTCN shall ignore the SymbolTimingInfo sent by the SS for EUTRA, NBIOT * NR: * REQ ASPs: Any:=true only if the symbol number is not relevant, * in which case the SS may choose the next available symbol of the given slot * IND ASPs: Any:=true only if there is no symbol information available for the particular kind of indication */ SymbolNumber_Type SymbolOffset, /* to address a particular symbol in a slot */ Null_Type FirstSymbol, /* to address the first symbol (REQ ASPs only) or for REQ ASPs in EUTRA and NBIOT */ Null_Type Any /* for IND ASPs in EUTRA and NBIOT or if symbol number is not relevant or not available */ }; In the TC, the Symbol is set as cs_SymbolTimingInfo_FirstSlot as a default assignment. However, when receiving UL data for PUSCH Aggregation, it is needed to change the Symbol: = cs_SymbolTimingInfo_Any as symbol number is not relevant here. |
| Summary of change | Introduced a template cs_SymbolTimingInfo_Any to set the Symbol to “Any” |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_3_9_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { <CODE SKIPPED> // @siclog "Step 4" siclog@ //Check: Does the UE transmit a MAC PDU including one RLC SDU, in HARQ process 1 and in slot n+4 and repeats in following pusch-AggregationFactor-1 slots with same resource allocation but different redundancy version (Note 3), if the slot can be used for uplink transmission (Note 4) //UL timing checked as per R5w210109 <CODE SKIPPED>

After Change:

function f_TC_7_1_1_3_9_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC {

<CODE SKIPPED> // @siclog "Step 4" siclog@ //Check: Does the UE transmit a MAC PDU including one RLC SDU, in HARQ process 1 and in slot n+4 and repeats in following pusch-AggregationFactor-1 slots with same resource allocation but different redundancy version (Note 3), if the slot can be used for uplink transmission (Note 4) //UL timing checked as per R5w210109 //WA# v_TimingUL.Symbol := valueof(cs_SymbolTimingInfo_Any); <CODE SKIPPED>

New Template

template (value) SymbolTimingInfo_Type cs_SymbolTimingInfo_Any := { Any := true };