3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230306

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3039 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.3.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2023-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation f_TC_11_3_10_NR5GC_IMS1, the function f_IMS_Registration (-, -, -, “120000”); is called. This needs to be changed to f_IMS_Registration (-, -, -, “120”); Also a coordination message is missing, which will cause the testacse to stall after premble. This needs to be added. In the current TTCN implementation of fl_TC_11_3_10_TestBody, a coordination message is sent to IMS PTC after Step 11-11A. This is not needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the function f_TC_11_3_10_NR5GC_IMS1 called f_IMS_Registration (-, -, -, “120”); Instead of calling f_IMS_Registration (-, -, -, “120000”); Also called f_IMS_IPCAN_SendCoOrdMsg(IPCAN); Please see below. Removed the coordination message after Step 11-11A in function fl_TC_11_3_10_TestBody |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.10.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	f_TC_11_3_10_NR5GC_IMS1	2

2.2	fl_TC_11_3_10_TestBody	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## f_TC_11_3_10_NR5GC_IMS1

| Function name | f_TC_11_3_10_NR5GC_IMS1 |
| --- | --- |
| Reason for change | In the current TTCN implementation of this function, the function f_IMS_Registration (-, -, -, “120000”); is called. This needs to bechanged to f_IMS_Registration (-, -, -, “120”); Also a coordination message is missing, which will cause the testacse to stall after premble. This needs to be added. |
| Summary of change | Called f_IMS_Registration (-, -, -, “120”); Instead of calling f_IMS_Registration (-, -, -, “120000”); Called f_IMS_IPCAN_SendCoOrdMsg(IPCAN); Please see below. |
| TTCN module | UAC_NR5GC_IMS.ttcn |
| MCC160 Comment |  |

Before Change:

<SKIPPED CODE>> f_IMS_PTC_Init(PDN_1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(-, -, -, "120000"); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); //@sic R5s230133 sic@ //@siclog "Step 10" siclog@ //Check: Does the UE performs SIP re-registration message in a time span between half of the expiration time and the full expiration time? t_RegExpireMax.start; <SKIPPED CODE>>

After Change:

<SKIPPED CODE>> f_IMS_PTC_Init(PDN_1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(-, -, -, "120"); //WA#11_3_10 120000 --> 120 f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); //WA#11_3_10 f_IMS_IPCAN_WaitForTrigger(IPCAN); //@sic R5s230133 sic@ //@siclog "Step 10" siclog@ //Check: Does the UE performs SIP re-registration message in a time span between half of the expiration time and the full expiration time? t_RegExpireMax.start; <SKIPPED CODE>>

## fl_TC_11_3_10_TestBody

| Template name | fl_TC_11_3_10_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation, a coordination message is sent to IMS PTC after Step 11-11A. This is not needed. |
| Summary of change | Removed the coordination message after Step 11-11A |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

<SKIPPED CODE>> //@siclog "Steps 11 - 11A" siclog@ //The SS changes the SIB1 according to 38.508-1 [4] Table 4.6.1-28 and transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification <-- PDCCH (DCI 1_0): Short Message //Wait for 2.1* modification period to allow the new system information to take effect f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1, -, RRC_CONNECTED); //@sic R5s230133 sic@ f_NR_WaitModificationPeriods(nr_Cell1); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); //Postamble after receiving coordination from IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); <SKIPPED CODE>>

After Change:

<SKIPPED CODE>> //@siclog "Steps 11 - 11A" siclog@ //The SS changes the SIB1 according to 38.508-1 [4] Table 4.6.1-28 and transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification <-- PDCCH (DCI 1_0): Short Message //Wait for 2.1* modification period to allow the new system information to take effect f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1, -, RRC_CONNECTED); //@sic R5s230133 sic@ f_NR_WaitModificationPeriods(nr_Cell1); //WA#11_3_10 COMMENTED OUT f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); //Postamble after receiving coordination from IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); <SKIPPED CODE>>