3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230331

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3056 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC UAC test case 11.3.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At step 35Ab3, incorrect timer name is used |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the timer name from t_WaitTime to t_Wait |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_11_3_5_TestBody |
| --- | --- |
| Reason for change | At step 35Ab3, incorrect timer name is used |
| Summary of change | Corrected the timer name from t_WaitTime to t_Wait |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_11_3_5_TestBody() runs on NR5GC_PTC { ......... ......... //@siclog "Step 35A siclog@ t_WaitTime.start(26.0); alt { //@siclog "Step 35Aa1 siclog@ //Check: Does the UE transmit an RRCSetupREQ message with establishmentCause of mps-PriorityAccess? [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(mps_PriorityAccess, ?))) { //@siclog "Step 35Aa2 siclog@ t_WaitTime.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 35Aa1"); } //@siclog "Step 35Ab1 siclog@ [] t_WaitTime.timeout { //@siclog "Step 35Ab2" siclog@ f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri); t_Wait.start(10.0); alt { //@siclog "Step 35Ab3" siclog@ //Check: Does the UE transmit an RRCSetupREQ message with establishmentCause of mps-PriorityAccess? [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(mps_PriorityAccess, ?))) { t_WaitTime.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 35Ab3"); } [] t_WaitTime.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 35Ab3"); } } } } //@siclog "Step 36 - 38 siclog@ //Void @sic R5-223436 sic@ ......... ......... }

After Change

function fl_TC_11_3_5_TestBody() runs on NR5GC_PTC { ......... ......... //@siclog "Step 35A siclog@ t_WaitTime.start(26.0); alt { //@siclog "Step 35Aa1 siclog@ //Check: Does the UE transmit an RRCSetupREQ message with establishmentCause of mps-PriorityAccess? [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(mps_PriorityAccess, ?))) { //@siclog "Step 35Aa2 siclog@ t_WaitTime.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 35Aa1"); } //@siclog "Step 35Ab1 siclog@ [] t_WaitTime.timeout { //@siclog "Step 35Ab2" siclog@ f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri); t_Wait.start(10.0); alt { //@siclog "Step 35Ab3" siclog@ //Check: Does the UE transmit an RRCSetupREQ message with establishmentCause of mps-PriorityAccess? [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(mps_PriorityAccess, ?))) { t_Wait.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 35Ab3"); } [] t_Wait.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 35Ab3"); } } } } //@siclog "Step 36 - 38 siclog@ //Void @sic R5-223436 sic@ ......... ......... }