3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230520

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0913 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS 5GS Emergency Call test case 10.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current implementation of the test case follows TS 34.229-5 Annex A.4.1 while it should procede according to Annex A.4.1a and use default configuration EVS A2. Adapt functions and usage of input parameters to establish the audio call with EVS default configuration following Annex A.4.1a. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed to Annex A.4.1a related functions and made usage of input parameters to configure the call to use EVS default configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 34.229-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | A prose CR will be raised for this change. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	1

1	Overview	3

2	Corrections required	3

2.1	f_TC_10_4_NR5GC_IMS1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required

## f_TC_10_4_NR5GC_IMS1

| Function name | f_TC_10_4_NR5GC_IMS |
| --- | --- |
| Reason for change | Current implementation of the test case follows TS 34.229-5 Annex A.4.1 while it should procede according to Annex A.4.1a and use default configuration EVS A2. |
| Summary of change | Changed to Annex A.4.1a related functions and made usage of input parameters to configure the call to use EVS default configuration. |
| TTCN module | \IMS\NR5GC_10\IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_TC_10_4_NR5GC_IMS1() runs on IMS_PTC { // Non-UE detectable emergency call / IM CN sends a 1xx response / UE geographical location information available or not / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var SipUrl v_ContactUrlUE; var boolean v_GeolocationUsage := pc_IMS_Geolocation; // @sic R5-225444, R5-233448 sic@ f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); // Trigger SS to execute location related functions to allow UE to get its position if (v_GeolocationUsage) { f_IMS_IPCAN_UpdateUELocationInformation(IPCAN); } // Test Case Body f_IMS_TestBody_Set(true); // @siclog "Step 1 - 7" siclog@ f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); // @siclog "Step 8" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1_Step1(); // @sic R5-225444, R5-226095 sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 8"); // @siclog "Step 9" siclog@ 100 Trying f_IMS_MOCallSetup_Send100Trying(v_InviteRequestWithSdp); // @siclog "Steps 10 - 12 Void" siclog@ // @siclog "Steps 13 - 15" siclog@ 183 Session in Progress, PRACK OK f_IMS_MOCallSetup_NR5GC_A_4_X_Step3_5(v_InviteRequestWithSdp, -, -, -, ASSERTED_EMERGENCY, EVS_16000, EVS_AnyNG114, -, tsc_SDP_WithPreconditions); // @sic R5-225444, R5233448 sic@ // @siclog "Steps 15A - 15C" siclog@ to trigger steps 10-12 of 38.508-1 Table 4.9.15.2.2-1 f_IMS_CC_TriggerDedicatedBearerActivation(IPCAN_MO_SpeechCall); // @sic R5-233448 sic@ // @siclog "Step 16" siclog@ // Check: Does the UE send an UPDATE? f_IMS_MOCallSetup_NR5GC_A_4_X_Step6_7(v_InviteRequestWithSdp, -, v_GeolocationUsage, ASSERTED_EMERGENCY, EVS_16000, EVS_AnyNG114); // @sic R5s230324 sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 16"); // @siclog "Steps 17A - 18" siclog@ Ringing, PRACK, 200, OK, 200 OK, ACK f_IMS_MOCallSetup_NR5GC_A_4_1_Step8_12(v_InviteRequestWithSdp, -, ASSERTED_EMERGENCY); // @sic R5-225444, R5s230324 sic@ // @siclog "Steps 19 - 20" siclog@ BYE, 200 OK v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); // @sic R5s230324 sic@ f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_TestBody_Set(false); // @siclog "Steps 21 - 23" siclog@ f_IMS_CC_Postamble(IPCAN_MO_SpeechCall); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_TC_10_4_NR5GC_IMS1() runs on IMS_PTC { // Non-UE detectable emergency call / IM CN sends a 1xx response / UE geographical location information available or not / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var SipUrl v_ContactUrlUE; var boolean v_GeolocationUsage := pc_IMS_Geolocation; // @sic R5-225444, R5-233448 sic@ f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); // Trigger SS to execute location related functions to allow UE to get its position if (v_GeolocationUsage) { f_IMS_IPCAN_UpdateUELocationInformation(IPCAN); } // Test Case Body f_IMS_TestBody_Set(true); // @siclog "Step 1 - 7" siclog@ f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); // @siclog "Step 8" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1a_Step1(); // @sic R5-225444, R5-226095 sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 8"); // @siclog "Step 9" siclog@ 100 Trying f_IMS_MOCallSetup_Send100Trying(v_InviteRequestWithSdp); // @siclog "Steps 10 - 12 Void" siclog@ // @siclog "Steps 13 - 15" siclog@ 183 Session in Progress, PRACK OK f_IMS_MOCallSetup_NR5GC_A_4_X_Step3_5(v_InviteRequestWithSdp, -, -, -, ASSERTED_EMERGENCY, EVS_16000, EVS_A2, -, tsc_SDP_WithPreconditions); // @sic R5-225444, R5233448 sic@ // @siclog "Steps 15A - 15C" siclog@ to trigger steps 10-12 of 38.508-1 Table 4.9.15.2.2-1 f_IMS_CC_TriggerDedicatedBearerActivation(IPCAN_MO_SpeechCall); // @sic R5-233448 sic@ // @siclog "Step 16" siclog@ // Check: Does the UE send an UPDATE? f_IMS_MOCallSetup_NR5GC_A_4_X_Step6_7(v_InviteRequestWithSdp, -, v_GeolocationUsage, ASSERTED_EMERGENCY, EVS_16000, EVS_A2); // @sic R5s230324 sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 16"); // @siclog "Steps 17A - 18" siclog@ Ringing, PRACK, 200, OK, 200 OK, ACK f_IMS_MOCallSetup_NR5GC_A_4_1_Step8_12(v_InviteRequestWithSdp, -, ASSERTED_EMERGENCY); // @sic R5-225444, R5s230324 sic@ // @siclog "Steps 19 - 20" siclog@ BYE, 200 OK v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); // @sic R5s230324 sic@ f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_TestBody_Set(false); // @siclog "Steps 21 - 23" siclog@ f_IMS_CC_Postamble(IPCAN_MO_SpeechCall); } <<SKIPPED CODE>>