3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250413

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4933 | rev | 1 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE testcases 7.2.3.10 and 7.2.3.13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-09-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Although sr_ConfigIndex in CellInfo has been changed to 10 for FDD, or to 7 for TDD. this configuration has not been updated in the local SS configuration. This results SR Configindex 30 is still being using locally. This issue is observed in both testcases 7.2.3.10 and 7.2.3.13. |  |  |  |  |  |  |  |  |
| Summary of change: |  | change the location of the SR ConfigIndex so that it can be used by local SS configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.10 and 7.2.3.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## 2.1 Change 1

| Function name | Correction to function f_TC_7_2_3_10_EUTRA |
| --- | --- |
| Reason for change | Although sr_ConfigIndex in CellInfo has been changed to 10 for FDD, or to 7 for TDD. this configuration has not been updated in the local SS configuration. This results SR Configindex 30 is still being using locally. |
| Summary of change | change the location of the SR ConfigIndex so that it can be used by local SS configuration. |
| TTCN module | LTE\7_2\RLC_AM_Testcases.ttcn |
| MCC160 Comments |  |

Before Change:

| /* * @desc AM RLC / Receiver Status Triggers * REFERENCE TS 36.523-1 clause 7.2.3.10 * @status APPROVED (LTE) */ function f_TC_7_2_3_10_EUTRA() runs on EUTRA_PTC { // @sic R5s100031 sic@ var RLC_SS_State v_RLC_Rec; var template (value) PhysicalConfigDedicated v_PhysicalConfigDedicated; //@sic R5-195310 sic@ var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode; //@sic R5-206052 sic@ //EUTRA initialisation f_EUTRA_Init(c1); //RLC global initialisation f_InitRLC_Record(v_RLC_Rec, AM_Mode); // overwrite non default values to be used v_RLC_Rec.t_Reordering := 0.15; v_RLC_Rec.t_StatusProhibit := 0.3; v_RLC_Rec.t_PollRetransmit := 0.5; //RLC test case specific initialisation v_RLC_Rec.UL_PDCP_SDU_size := 98; // Cell configuration, with one AM DRB at the SS side f_EUTRA_CellConfig_Def (eutra_Cell1); // Preamble to enter UE in E-UTRA RRC_IDLE (state 2) f_EUTRA_Preamble (eutra_Cell1, STATE2A_TESTLOOP_ModeA); //set null AS ciphering algorithm f_EUTRA_AS_CipheringAlgorithm_Set(eea0); v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD (eutra_Cell1); if (v_EUTRA_FDD_TDD_Mode == FDD) { f_EUTRA_CellInfo_SetSR_ConfigIndex(eutra_Cell1, 10); //@sic R5-195310 sic@ } else { f_EUTRA_CellInfo_SetSR_ConfigIndex(eutra_Cell1, 7); //@sic R5-206052, R5s201415r1 sic@ } v_PhysicalConfigDedicated := f_PhysicalConfigDedicated_Dsr_TransMax(eutra_Cell1, n16); //@sic R5-195310 sic@ //entering UE test state 3 with test case specific RLC parameters, with BSR and PHR disabled fl_EUTRA_GenericRbEst_WithRLC_AM_Params (eutra_Cell1, {cs_DRB_ToAddMod_RLC_AM (tsc_DRB1, cds_TX_AM_RLC_t_PollRetransmit (ms500), cds_RX_AM_RLC_Common (ms150, ms300))}, v_PhysicalConfigDedicated //@sic R5-195310 sic@ ); // reconfigure DRB locally to set the required test mode AM mapped to TM f_EUTRA_SS_DRB1_ConfigAM_TM_Mode(eutra_Cell1); //closing the test loop (mode A), entering state 4 f_EUTRA_CloseUE_TestLoopModeA (eutra_Cell1, cs_UE_TestLoopModeA_LB_Setup_NoScaling); f_EUTRA_TestBody_Set(true, layer2); // @sic R5-100050 sic@ f_EUTRA_HarqErrorHandling_Set(raiseInconc); fl_EUTRA_7_2_3_10_TestBody (v_RLC_Rec); f_EUTRA_HarqErrorHandling_Set(ignore); f_EUTRA_TestBody_Set(false); // assigning the default grant again f_EUTRA_StartDefULGrantTransmission(eutra_Cell1); // leaving UE test state 4 //@sic R5-103847 Deactivate test loop and open test loop is removed sic@ // postamble f_EUTRA_Postamble (eutra_Cell1, E3_TESTMODE); } // f_TC_7_2_3_10_EUTRA |  |
| --- | --- |

After Change:

| /* * @desc AM RLC / Receiver Status Triggers * REFERENCE TS 36.523-1 clause 7.2.3.10 * @status APPROVED (LTE) */ function f_TC_7_2_3_10_EUTRA() runs on EUTRA_PTC { // @sic R5s100031 sic@ var RLC_SS_State v_RLC_Rec; var template (value) PhysicalConfigDedicated v_PhysicalConfigDedicated; //@sic R5-195310 sic@ var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode; //@sic R5-206052 sic@ //EUTRA initialisation f_EUTRA_Init(c1); //RLC global initialisation f_InitRLC_Record(v_RLC_Rec, AM_Mode); // overwrite non default values to be used v_RLC_Rec.t_Reordering := 0.15; v_RLC_Rec.t_StatusProhibit := 0.3; v_RLC_Rec.t_PollRetransmit := 0.5; //RLC test case specific initialisation v_RLC_Rec.UL_PDCP_SDU_size := 98; // Set SR configIndex v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD (eutra_Cell1); if (v_EUTRA_FDD_TDD_Mode == FDD) { f_EUTRA_CellInfo_SetSR_ConfigIndex(eutra_Cell1, 10); //@sic R5-195310 sic@ } else { f_EUTRA_CellInfo_SetSR_ConfigIndex(eutra_Cell1, 7); //@sic R5-206052, R5s201415r1 sic@ } // Cell configuration, with one AM DRB at the SS side f_EUTRA_CellConfig_Def (eutra_Cell1); // Preamble to enter UE in E-UTRA RRC_IDLE (state 2) f_EUTRA_Preamble (eutra_Cell1, STATE2A_TESTLOOP_ModeA); //set null AS ciphering algorithm f_EUTRA_AS_CipheringAlgorithm_Set(eea0); v_PhysicalConfigDedicated := f_PhysicalConfigDedicated_Dsr_TransMax(eutra_Cell1, n16); //@sic R5-195310 sic@ //entering UE test state 3 with test case specific RLC parameters, with BSR and PHR disabled fl_EUTRA_GenericRbEst_WithRLC_AM_Params (eutra_Cell1, {cs_DRB_ToAddMod_RLC_AM (tsc_DRB1, cds_TX_AM_RLC_t_PollRetransmit (ms500), cds_RX_AM_RLC_Common (ms150, ms300))}, v_PhysicalConfigDedicated //@sic R5-195310 sic@ ); // reconfigure DRB locally to set the required test mode AM mapped to TM f_EUTRA_SS_DRB1_ConfigAM_TM_Mode(eutra_Cell1); //closing the test loop (mode A), entering state 4 f_EUTRA_CloseUE_TestLoopModeA (eutra_Cell1, cs_UE_TestLoopModeA_LB_Setup_NoScaling); f_EUTRA_TestBody_Set(true, layer2); // @sic R5-100050 sic@ f_EUTRA_HarqErrorHandling_Set(raiseInconc); fl_EUTRA_7_2_3_10_TestBody (v_RLC_Rec); f_EUTRA_HarqErrorHandling_Set(ignore); f_EUTRA_TestBody_Set(false); // assigning the default grant again f_EUTRA_StartDefULGrantTransmission(eutra_Cell1); // leaving UE test state 4 //@sic R5-103847 Deactivate test loop and open test loop is removed sic@ // postamble f_EUTRA_Postamble (eutra_Cell1, E3_TESTMODE); } // f_TC_7_2_3_10_EUTRA |  |
| --- | --- |

## 2.2 Change 1

| Function name | Correction to function f_TC_7_2_3_13_EUTRA |
| --- | --- |
| Reason for change | Although sr_ConfigIndex in CellInfo has been changed to 10 for FDD, or to 7 for TDD. this configuration has not been updated in the local SS configuration. This results SR Configindex 30 is still being using locally. |
| Summary of change | change the location of the SR ConfigIndex so that it can be used by local SS configuration. |
| TTCN module | LTE\7_2\RLC_AM_Testcases.ttcn |
| MCC160 Comments |  |

Before Change:

| /* * @desc AM RLC / Reconfiguration of RLC parameters by upper layers * REFERENCE TS 36.523-1 clause 7.2.3.13 * @status APPROVED (LTE) */ function f_TC_7_2_3_13_EUTRA() runs on EUTRA_PTC { // @sic R5s100145 sic@ @sic R5s102240 sic@ @sic R5-103807 sic@ @sic R5s100224 sic@ @sic R5s100279 sic@ var RLC_SS_State v_RLC_Rec; var template (value) PhysicalConfigDedicated v_PhysicalConfigDedicated; //@sic R5-195310 sic@ var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode; //@sic R5-206052 sic@ //EUTRA initialisation f_EUTRA_Init(c1); //RLC global initialisation f_InitRLC_Record(v_RLC_Rec, AM_Mode); // test case specific values v_RLC_Rec.t_Reordering := 0.15; v_RLC_Rec.t_StatusProhibit := 0.3; v_RLC_Rec.t_PollRetransmit := 0.4; v_RLC_Rec.pollPDU := 125000; // pollPDU to Infinity v_RLC_Rec.pollByte := 12500000;// pollByte to Infinity //RLC test case specific initialisation v_RLC_Rec.UL_PDCP_SDU_size := 98; // Cell configuration, with one AM DRB at the SS side f_EUTRA_CellConfig_Def (eutra_Cell1); // Preamble to enter UE in E-UTRA RRC_IDLE (state 2) f_EUTRA_Preamble (eutra_Cell1, STATE2A_TESTLOOP_ModeA); //set null AS ciphering algorithm f_EUTRA_AS_CipheringAlgorithm_Set(eea0); v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD (eutra_Cell1); if (v_EUTRA_FDD_TDD_Mode == FDD) { f_EUTRA_CellInfo_SetSR_ConfigIndex(eutra_Cell1, 10); //@sic R5-195310 sic@ } else { f_EUTRA_CellInfo_SetSR_ConfigIndex(eutra_Cell1, 7); //@sic R5-206052, R5s201415r1 sic@ } v_PhysicalConfigDedicated := f_PhysicalConfigDedicated_Dsr_TransMax(eutra_Cell1, n16); //@sic R5-195310 sic@ //entering UE test state 3 with test case specific RLC parameters, with BSR and PHR disabled fl_EUTRA_GenericRbEst_WithRLC_AM_Params (eutra_Cell1, {cs_DRB_ToAddMod_RLC_AM (tsc_DRB1, cds_TX_AM_RLC_no_mRT (ms400, pInfinity, kBinfinity), cds_RX_AM_RLC_Common (ms150, ms300))}, v_PhysicalConfigDedicated //@sic R5-195310 sic@ ); // reconfigure DRB locally to set the required test mode AM mapped to TM f_EUTRA_SS_DRB1_ConfigAM_TM_Mode(eutra_Cell1); //closing the test loop (mode A), entering state 4 f_EUTRA_CloseUE_TestLoopModeA(eutra_Cell1, cs_UE_TestLoopModeA_LB_Setup_1DRB (v_RLC_Rec.UL_PDCP_SDU_size * 8, tsc_DRB1)); f_EUTRA_TestBody_Set(true, layer2); // @sic R5-100050 sic@ f_EUTRA_HarqErrorHandling_Set(raiseInconc); fl_EUTRA_7_2_3_13_TestBody (v_RLC_Rec); f_EUTRA_HarqErrorHandling_Set(ignore); f_EUTRA_TestBody_Set(false); f_EUTRA_StartDefULGrantTransmission(eutra_Cell1); // leaving UE test state 4 //@sic R5-103847 Deactivate test loop and open test loop is removed sic@ // postamble f_EUTRA_Postamble (eutra_Cell1, E3_TESTMODE); } // f_TC_7_2_3_13_EUTRA |  |
| --- | --- |

After Change:

| function f_TC_7_2_3_13_EUTRA() runs on EUTRA_PTC { // @sic R5s100145 sic@ @sic R5s102240 sic@ @sic R5-103807 sic@ @sic R5s100224 sic@ @sic R5s100279 sic@ var RLC_SS_State v_RLC_Rec; var template (value) PhysicalConfigDedicated v_PhysicalConfigDedicated; //@sic R5-195310 sic@ var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode; //@sic R5-206052 sic@ //EUTRA initialisation f_EUTRA_Init(c1); //RLC global initialisation f_InitRLC_Record(v_RLC_Rec, AM_Mode); // test case specific values v_RLC_Rec.t_Reordering := 0.15; v_RLC_Rec.t_StatusProhibit := 0.3; v_RLC_Rec.t_PollRetransmit := 0.4; v_RLC_Rec.pollPDU := 125000; // pollPDU to Infinity v_RLC_Rec.pollByte := 12500000;// pollByte to Infinity //RLC test case specific initialisation v_RLC_Rec.UL_PDCP_SDU_size := 98; // Set SR configIndex v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD (eutra_Cell1); if (v_EUTRA_FDD_TDD_Mode == FDD) { f_EUTRA_CellInfo_SetSR_ConfigIndex(eutra_Cell1, 10); //@sic R5-195310 sic@ } else { f_EUTRA_CellInfo_SetSR_ConfigIndex(eutra_Cell1, 7); //@sic R5-206052, R5s201415r1 sic@ } // Cell configuration, with one AM DRB at the SS side f_EUTRA_CellConfig_Def (eutra_Cell1); // Preamble to enter UE in E-UTRA RRC_IDLE (state 2) f_EUTRA_Preamble (eutra_Cell1, STATE2A_TESTLOOP_ModeA); //set null AS ciphering algorithm f_EUTRA_AS_CipheringAlgorithm_Set(eea0); v_PhysicalConfigDedicated := f_PhysicalConfigDedicated_Dsr_TransMax(eutra_Cell1, n16); //@sic R5-195310 sic@ //entering UE test state 3 with test case specific RLC parameters, with BSR and PHR disabled fl_EUTRA_GenericRbEst_WithRLC_AM_Params (eutra_Cell1, {cs_DRB_ToAddMod_RLC_AM (tsc_DRB1, cds_TX_AM_RLC_no_mRT (ms400, pInfinity, kBinfinity), cds_RX_AM_RLC_Common (ms150, ms300))}, v_PhysicalConfigDedicated //@sic R5-195310 sic@ ); // reconfigure DRB locally to set the required test mode AM mapped to TM f_EUTRA_SS_DRB1_ConfigAM_TM_Mode(eutra_Cell1); //closing the test loop (mode A), entering state 4 f_EUTRA_CloseUE_TestLoopModeA(eutra_Cell1, cs_UE_TestLoopModeA_LB_Setup_1DRB (v_RLC_Rec.UL_PDCP_SDU_size * 8, tsc_DRB1)); f_EUTRA_TestBody_Set(true, layer2); // @sic R5-100050 sic@ f_EUTRA_HarqErrorHandling_Set(raiseInconc); fl_EUTRA_7_2_3_13_TestBody (v_RLC_Rec); f_EUTRA_HarqErrorHandling_Set(ignore); f_EUTRA_TestBody_Set(false); f_EUTRA_StartDefULGrantTransmission(eutra_Cell1); // leaving UE test state 4 //@sic R5-103847 Deactivate test loop and open test loop is removed sic@ // postamble f_EUTRA_Postamble (eutra_Cell1, E3_TESTMODE); } // f_TC_7_2_3_13_EUTRA |  |
| --- | --- |